TUGAS VIII

Tugas : mensimulasikan karakteristik I-V dan frekuensi tinggi transistor MOSFET.

_____________

Title N-channel MOSFET grid generation

$ Form grid of structure and give doping distribution

$ ****************** define rectangular grid **************

mesh	rectangular	nx=25	ny=25	diag.flip	smooth=1

x.mesh	n=1	l=1	r=1

x.mesh	n=25	l=1	r=1

y.mesh	n=1	l=-0.02	r=1

y.mesh	n=3	l=0	r=1

y.mesh	n=14	l=0.7	r=1.05

y.mesh	n=25	l=2.5	r=1.2

$ ********** ******* regions ***************************

regions	num=1	y.max=0	 	oxide

regioon	num=2 	y.min=0 	silicon

$ ****************** electrodes ************************

$ #1-GATE #2-SUBSTRATE #3-SOURCE #4-DRAIN

electrode	num=1	x.min=0.520	length=1.460

electrode	num=2	substrate

electrode	num=3	source	length=0.417	y.min=0		y.max=0

electrode	num=4	drain	length=0.417	y.min=0		y.max=0

$ ****************** doping and fixed charge *************

doping	reg=2	gaussian		concentration=1.e16	characteristic=0.10

+	acceptor	peak=0.0	outfile=dopingm

doping	reg=2	gaussian		concentration=1.e20	characteristic=0.10

+	donor	peak=0.0	x.r=0.6			ratio=0.75

doping	reg=2	gaussian		concentration=1.e20	characteristic=0.10

+	donor	peak=0.0	x.l=2.0			ratio=0.75

doping 	reg=2 	uniform		concentration=1.e16	acceptor

$ ****************** regrid on doping *******************

plot.2d		grid	no.top	boundry	no.fill	pa

regrid		doping	log	reg=2	ratio=4	smooth.k=1	outfile=gmesh1	dopfile=dopingm

regrid		doping	log	reg=2	ratio=4	smooth.k=1	outfile=gmesh2	dopfile=dopingm

contact		num=1	n.polysilicon

symbolic	gummel	carriers=0

solve		initial

regrid		potential	 ratio=0.2 region=2	smooth.k=1	outfile=gmesh3	dopfile=dopingm

plot.2d		grid	no.top	boundry	no.fill	pa

plot.1d		log	abs	doping	x.s=0	y.s=0	x.e=0	y.e=1.5	points	pa

plot.1d		log	abs	doping	x.s=0	y.s=0	x.e=2.5	y.e=0	line.type=2	pa

plot.1d		log	abs	doping	x.s=1.2	y.s=0	x.e=1.2	y.e=1.5	line.type=2	pa

end

Title N-channel MOSFET Id/Vd characteristic

Mesh	inf=gmesh3

Contact	num=1	n.polysilicon

Models	print	temp=300	conmob	fldmob	bgn	fermi	incomplete

+	tfldmb2	OX.L=0.6	OX.R=2

interf	qf=3E10

symb	newton	carrier=1	selectrons

method	xnorm	autonr		itlimit=30	trap=.5	p.tol=1.e-4

+	c.tol=1.e-4

solve	initial

solve	previous	v4=0.005

solve	project	vstep=0.25	nstep=3	elect=1

solve	project	v1=1.0		outf=g111

solve	project	vstep=0.25	nstep=3	elect=1

solve	project	v1=2.0		outf=g211

solve	project	vstep=0.25	nstep=3	elect=1

solve	project	v1=3.0		outf=g311

solve	project	vstep=0.25	nstep=3	elect=1

solve	project	v1=4.0		outf=g411

load	inf=g111

log	outf=d111

solve	vstep=0.1	nstep=9	elect=4	previous

solve	previous		v4=1.0	vstep=0.1	nstep=40	elect=4

load	inf=g211

log	outf=d211

solve	vstep=0.1	nstep=9	elect=4	previous

solve	previous		v4=1.0	vstep=0.1	nstep=40	elect=4

load	inf=g311

log	outf=d311

solve	vstep=0.1	nstep=9	elect=4	previous

solve	previous		v4=1.0	vstep=0.1	nstep=40	elect=4

load	inf=g411

log	outf=d411

solve	vstep=0.1	nstep=9	elect=4	previous

solve	previous		v4=1.0	vstep=0.1	nstep=40	elect=4

end

Title	N-channel MOSFET I/d/Vd characterisation

Mesh	inf=gmesh3

Plot.1d	inf=d411	x.ax=v4	y.ax=i4	abs	x.label=VDS	y.label=IDS	pa

+	title=”Fitting of NMOSFET”		max=3.9e-4

+	subtitle=”output characteristic”

plot.1d	inf=d311	x.ax=v4	y.ax=i4	abs	unch	pa

plot.1d	inf=d211	x.ax=v4	y.ax=i4	abs	unch	pa

plot.1d	inf=d111	x.ax=v4	y.ax=i4	abs	unch	pa

fit	mos		fit.type=2	bulk=2	drain=4	gate=1

+ 	line1.type=4	line2.type=5

Titile N-channel MOSFET substrate Current

Mesh	inf=gmesh3

Plot.1d	inf=substr5	x.ax=v1	y.ax=i2	abs	x.label=VGS	y.label=Isub	log  points  pa

+	title=”Substrate current”	subtitle=”versus gate bias NMOSFET”

plot.1d	inf=substr4	x.ax=v1	y.ax=i2	abs	log	points	unch	pa

end

Title	N.channel	MOSFET substrate Current extraction

Mesh	inf=gmesh3

Contact	num=1	n.polysilicon

Models	print	temp=300	conmob	fldmob	bgn	fermi	incomplete

+	tfldmb2	OX.L=0.6	OX.R=2

impact

interf	qf=3E10

symb	carrires=0

method	iccg	damped	itlimit=30

solve	init

solve	v1=0.3	v4=4.0	local	outf=sb4

solve	v1=0.3	v4=5.0	local	outf=sb5

symb	newton	carriers=2

load	inf=sb4

solve	prev	v1=0.4	vstep=0.1	nstep=6	elect=1

solve	prev	vstep=0.25		nstep=8	elect=1

solve	prev	vstep=0.50		nstep=8	elect=1

load	inf=sb5

solve	prev	v1=0.4	vstep=0.1	nstep=6	elect=1

solve	prev	vstep=0.25		nstep=8	elect=1

solve	prev	vstep=0.50		nstep=8	elect=1

end

$ PN dioda AC simulation

mesh	inf=gmesh3

contact	num=1	aluminum

interf	qf=3e10

symb	newton	carr=2

method	xnorm	autonr

models	print	temp=300	fldmob	srh

+	auger	conmob	fldmob	tfldmb2	ox.l=0.0	ox.r=2.0

solve	init

log	acfile=ac.log1

solve	previous	v1=1.00	ac	freq=1e6 term=12

solve	previous	v1=-0.75	vstep=0.5	nstep=25	elect=1	ac	freq=1e6

+	term=12

end

$ PN dioda AC simulation

mesh	inf=gmesh3

plot.1d	inf=ac.log1	x.ax=v1	y.ax=e12	points	pa

end