IONOPSHERIC COMMUNICATIONS

ENHANCED PROFILE ANALYSIS & CIRCUIT

(ICEPAC)

PREDICTION PROGRAM

USER’S  MANUAL

&  HF ANTENNA MODEL PROGRAM

Provided in the

“ITS HF Propagation Package”

PREFACE

The Institute for Telecommunication Sciences and its predecessors in the U.S. Department of Commerce have been collecting ionospheric data and developing methods to use these data in the prediction of the expected performance of high-frequency (HF) sky-wave systems since the start of World War II.

Much of these data and the techniques for using the data are stored for use by computers.  Several "standard" output formats have also emerged to assist in the planning and operation of high-frequency systems using sky-waves.  This report describes the use of the latest developed method - The "Ionospheric Communications Enhanced Profile Analysis and Circuit Prediction Program" (ICEPAC).  The input and output characteristics in this report relate to ICEPAC version .10.  The version number was generated to historically document the ICEPAC program as it currently exists and to facilitate a means of identifying subsequent versions of the program.

For many years, numerous organizations have been employing the HF spectrum to communicate over long distances.  It was recognized in the late thirties that these communication systems were subject to marked variations in performance.  The effective operation of long-distance HF systems increased in proportion to the ability to predict variations in the ionosphere, since such an ability permitted the selection of optimum frequencies, antennas, and other circuit parameters.  A worldwide network of ionosoundes was established to measure ionospheric parameters.  Worldwide noise measurement records were taken, and observed variations in signal and amplitudes were recorded over various HF paths.  The results of this research established that most variations in HF system performance were directly related to changes in the ionosphere, which in turn are affected in a complex manner by solar activity, seasonal and diurnal variations, as well as latitude and longitude.  By 1948, a treatise of ionospheric radio propagation was published by the Central Radio Propagation Laboratory (CRPL) of the National Bureau of Standards.  This document (CRPL, l948) outlined the state of the art in HF propagation.  Manual techniques were given for analyzing HF circuits of short, intermediate, and long distances.  Because the manual methods were laborious and time consuming, various organizations developed computer programs to analyze HF circuit performance.  All these programs were based on manual methods for short or intermediate distances and used various numeric representations of the ionospheric data. The IONCAP program was the latest program developed and used in HF prpagation predictions.  The IONCAP prediction program had poor performance in the polar region and used some of the older profile structures.  The program described here is a direct descendant of the IONCAP  program.  Use of the Ionospheric Communications Enhanced Profile Analysis and Circuit Prediction Program (ICEPAC) is described in this report.

The Ionospheric Communications Enhanced Profile Analysis and Circuit Prediction Program (ICEPAC) is in modular form and coded in simple FORTRAN, following as much as possible the ANSI 77 standard.  The modular form allows any subsection to be replaced without affecting the rest of the program.  As much as possible, table look-up techniques are used to reduce computer run time, to facilitate the modular structure, and to assist in the detection of errors in any subsection.  The program is divided into seven largely independent sections:

1    input subroutines,

2    path geometry subroutines,

3    antenna subroutines,

4    ionospheric parameter subroutines,

5    maximum usable frequency subroutines,

6    system performance subroutines, and

7    output subroutines.

The input subroutines handle the various input options.  There are four inputs:  Line command disk file, a long-term data disk file, corrected geomagnetic disk file, and an antenna disk file.  The line command disk file contains the circuit parameters and control run options.  The long-term data disk file contains numeric coefficients for ionospheric parameters and for atmospheric noise as well as tables of parameters needed for circuit performance.  The corrected geomagnetic coordinate file provides geomagnetic latitude and longitude for the polar model. The antenna disk file contains optional antenna patterns which can either be generated by the ICEPAC program or obtained from some other source.

The path geometry subroutines determine the circuit geometry, select areas to sample the ionosphere, and evaluate the magnetic field at these sample areas.

The antenna subroutines process antenna data command lines, calculate antenna gains, and output antenna patterns.  The program has the simple subroutine from ITSA-1 (Lucas and Haydon, 1966) for the basic antenna models. These assume the antennas are associated with existing systems that have been properly designed.

The ionospheric parameter subroutines evaluate the ionospheric parameters needed by the program.  Previous programs assumed an implicit two parabola ionosphere.  An explicit electron density profile is used in this program.  Observation indicates that absorption equations using the secant law require modification when frequencies do not traverse the entire absorbing region, i.e., with reflection heights lower than 90 km.  An empirical modification to the secant law is included in this program.

The maximum usable frequency (MUF) subroutine is a direct determination of the junction frequency based on an electron density profile derived from monthly median parameters of the ionosphere rather than an iterative search.  A corrected form of Martyn's theorem (Martyn, 1969) is used.  The E, Fl, and F2 layer MUFs are considered.  There is also a separate sporadic-E MUF.

The system performance subroutines evaluate all the usual circuit performance parameters.  There are two basic subroutines:  one for shorter distances and one for long distances (greater than 10,000 km). The models for the shorter distances and the long distance models have previously been incorporated in the IONCAP computer program and are continued in the ICEPAC program.  The short-distance models correspond to the manual method given by Haydon et al. (1969).  A manual method somewhat like the long-distance models is given in NBS Report 462, (CRPL, 1948).  The short-distance model evaluates all possible ray paths for the circuit, including high and low angle modes; E, Fl, and F2 modes; above the MUF modes; and sporadic-E modes.  Losses include regular D-E absorption (CCIR-252 loss), deviative losses, and sporadic-E losses.  The CCIR-262 loss is basically for F2 modes.  For E-layer modes, an adjustment of the absorption is required, and for frequencies which have low reflection heights (less than 90 km) a further correction to the frequency dependence is added.  The noise at the receiver site is evaluated and combined with signal statistics to estimate the signal-to-noise statistics.

An extension of the single-hop model to long paths would lead to the expectation that failure of propagation at any of the reflection areas would cause propagation to fail altogether.  Empirically, however,it has been found that propagation does not fail until the ionosphere either fails to launch a sky-wave or does not permit sky-wave reception; i.e., these are control areas about 2,000 km from each end of the path.  The long-distance model evaluates a sky-wave launch capability at the transmitter and a sky-wave intercept capability at the receiver, using an antenna-gain-minus-ionosphere-loss function at each end of the path.  Losses are the same as for the short paths at each end of the path, with a loss per kilometer function used to fill in the path.  Noise and signal statistics are the same for the short-distance or the long-distance paths.

The output subroutines generate all the output options as line printer images which can be printed or saved on disk.  The available output options and the corresponding input required to generate the output is described in this report.

Much of the work completed is an incorporation of the combined efforts of various laboratories, both government and private,  domestic and foreign.  Although this program is coded so that revision of any sub-part is relatively easy, it is difficult to join so many diverse sub-models while maintaining consistency and continuity of the entire program.  The whole in this case is much more than a sum of the parts.

The use of the program with a description of input and output options is described in this report.  The underlying assumptions and the mathematical-physical models are described in a companion report.

TABLE OF CONTENTS

Page

# LIST OF FIGURES                                                            ix

LIST OF TABLES                                                            xii

LIST OF COMMAND LINES                                                    xiii

ANTENNA COMMAND LINES                                                     xiv

LIST OF SYMBOLS                                                            xv

## Ionospheric Communications Enhanced Profile Analysis and

## Circuit (ICEPAC) Prediction Program User's Manual

1.   INTRODUCTION                                                           1

2.   PRIMARY PROGRAM RELATED REQUIREMENTS                                   2

2.1  Computer Implementation Requirements                             2

2.2  Files Used by ICEPAC                                             3

2.3  Program Operation Review                                         4

3.   INPUT DATA REQUIREMENTS                                                5

3.1  General Data Requirements                                        5

3.2  Input Language Definition                                        6

3.3  Description of the User-Defined Input Data                       7

3.3.1  Program Control, Execution, and

Termination Command Lines                                7

3.3.2  Diurnal, Month, and Solar Activity Command Lines         9

3.3.3  System Configuration Command Lines                      10

3.3.4  User-Defined Data Base and System

Override Command Lines                                  15

3.3.5  Input, Output, and Comment Command Lines                19

4.   OUTPUT OPTIONS                                                        23

4.1  Ionospheric Parameters Output Options, METHOD = 1 or 2          23

4.2  MUF Output Options, METHOD = 3 through 12                       25

4.3  Systems Performance Output Options, METHOD = 16 through 24      26

4.4  Antenna Output Options, METHOD = 13, 14, or 15                  29

TABLE OF CONTENTS (cont.)

# Page

# 5    APPLICATIONS                                                          29

5.1  Ionospheric Parameters Applications                             30

5.2  MUF Applications                                                30

5.3  System Performance Applications                                 30

5.3.1   Selecting an Optimum Frequency                         31

5.3.2   Selecting a Frequency Complement for a Single Circuit

in the Absence of Other Circuit Interference           31

5.3.3   Standard Frequency Complement                          32

5.3.4   Two-Frequency Complements                              33

5.3.5   Three-Frequency Complements                            35

5.3.6   Four-Frequency Complements                             37

5.3.7   Time Sharing on Circuits Separated Geographically      40

5.3.8   Time Sharing in the Same Geographic Area               40

5.3.9   Frequency Sharing                                      40

5.3.10  Broadcast Coverage                                     40

5.3.11  Optimum Times for Communication                        41

5.3.12  Selection of Relay Locations                           41

5.3.13  Determination of Lowest Effective Transmitter Power    41

5.4   Antenna Selection or Design                                    42

6.   PROGRAM DATA FILES                                                    42

6.1  BCD Program and Data Base Tape                                  42

6.2  Long-Term Prediction File                                       43

6.3  Antenna Patterns Stored on File                                 44

7.   SPECIFIC INPUT EXAMPLES                                               45

7.1  Ionospheric Parameter Example                                   45

7.2  Antenna Pattern Example                                         45

7.3  MUF Example                                                     46

7.4  System Performance Example                                      46

7.5  User-Selected Output Example                                    47

7.6  LUF Example                                                     48

7.7  Outgraph Example                                                49

7.8  External Antenna File Example                                   49

8.    ACKNOWLEDGMENTS                                                      51

9.    REFERENCES                                                          154

LIST OF FIGURES

Page

Figure  1.   Input data lines for all output options.                       52

Figure  2.   Ionospheric parameters output.                    (METHOD=l)   54

Figure  3.   Ionogram output.                                  (METHOD=2)   55

Figure  4.   MUF complete output table.                        (METHOD=7)   57

Figure  5.   MUF-FOT graph.                                    (METHOD=8)   59

Figure  6.   MUF-FOT-HPF graph.                                (METHOD=9)   60

Figure  7.   MUF-FOT-ANG graph.                                (METHOD=10)  61

Figure  8.   MUF-FOT-Es MUF graph.                             (METHOD=11)  62

Figure  9.   Transmitter antenna pattern.                      (METHOD=15)  63

Figure 10.   Receiver-antenna pattern.                         (METHOD=15)  65

Figure 11.   System performance.                               (METHOD=16)  67

Figure 12.   Condensed system performance, reliabilities.      (METHOD=17)  68

Figure 13.   Condensed system performance, service probability.(METHOD=18)  69

Figure 14.   Propagation path geometry.                        (METHOD=19)  70

Figure 15.   Complete system performance.                      (METHOD=20)  71

Figure 16.   Forced long-path model.                           (METHOD=21)  72

Figure 17.   Forced short-path model.                          (METHOD=22)  73

Figure 18.   User-selected system performance.                 (METHOD=23)  74

Figure 19.   Reliability table output.                         (METHOD=24)  75

Figure 20.   All modes output (user-defined freq.).            (METHOD=25)  76

Figure 21.   All modes output (MUF).                           (METHOD=25)  77

Figure 22.   LUF-FOT graph.                                    (METHOD=26)  78

Figure 23.   LUF-FOT graph.                                    (METHOD=27)  79

Figure 24.   LUF-MUF-FOT graph                                 (METHOD=28)  80

Figure 25.   LUF-MUF graph                                     (METHOD=29)  81

LIST OF FIGURES (cont.)

Page

Figure 26.   MUF complete output table (long-path example).     (METHOD=7)  82

Figure 27.   User-selected system performance (long-path example)

(METHOD=23)  83

Figure 28.   Antenna pattern input lines.                                   84

Figure 29.   Horizontal rhombic structure (1).                              86

Figure 30.   Horizontal rhombic pattern.                                    87

Figure 31.   Vertical monopole structure (2).                               89

Figure 32.   Vertical monopole pattern.                                     90

Figure 33.   Horizontal dipole structure (3).                               92

Figure 34.   Horizontal dipole pattern.                                     93

Figure 35.   Horizontal Yagi structure (4).                                 95

Figure 36.   Horizontal Yagi pattern                                        96

Figure 37.   Vertical log periodic structure (5)                            98

Figure 38.   Vertical log periodic pattern.                                 99

Figure 39.   Curtain structure (6).                                        101

Figure 40.   Curtain pattern.                                              102

Figure 41.   Sloping vee structure (7).                                    104

Figure 42.   Sloping vee pattern.                                          105

Figure 43.   Inverted L structure (8).                                     107

Figure 44.   Inverted L pattern.                                           108

Figure 45.   Sloping rhombic structure (9).                                110

Figure 46.   Sloping rhombic pattern.                                      111

Figure 47.   Interlaced rhombic structure (11).                            113

Figure 48.   Interlaced rhombic pattern.                                   114

Figure 49.   Constant gain pictorial pattern (12).                         116

LIST OF FIGURES (cont.)

Page

Figure 50.   Constant gain pattern.                                        117

Figure 51.   Constant gain receiver antenna with antenna efficiency.       119

Figure 52    Relationship between Effective Q index and effective Kp value 132

LIST OF TABLES

Page

Table  1.   ICEPAC Files                                                  121

Table  2.   Valid Name Identifiers                                        122

Table  3.   Available Output Methods                                      123

Table  4.   Required SNR's for Radiotelephone Service                     124

Table  5.   Required SNR's for Radio-teletype Service                     125

Table  6.   Typical values of Ground Electrical Characteristics           126

Table  7.   Output Combinations for the OUTGRAPH Command                  127

Table  8.   Header Line Options for the TOPLINES Command                  128

Table  9.   System Performance Output Line Options for

the BOTLINES Command                                          129

Table 10.   System Performance Output Line Options as

Preset by METHOD Number                                       130

Table 11.   Input Data Required for Output Methods                        131

Table 12.   Effective Q and Kp relationships                              132

LIST OF COMMAND LINES

Page

Command Line  1.   METHOD command                                         133

Command Line  2.   MONTH command                                          133

Command Line  3.   SUNSPOT command                                        133

Command Line  4.   CIRCUIT command                                        133

Command Line  5.   SYSTEM command                                         134

Command Line  6.   TIME command                                           134

Command Line  7.   FREQUENCY command                                      135

Command Line  8.   LABEL command                                          135

Command Line  9.   INTEGRATE command                                      136

Command Line 10.   EXECUTE command                                        136

Command Line 11.   EFVAR command                                          136

Command Line 12.   ESVAR command                                          137

Command Line 13.   EDP command                                            137

Command Line 13a.  Electron density commands, true height                 137

Command Line 13b.  Electron density commands, square of plasma frequency  138

Command Line 14.   ANTOUT command                                         138

Command Line 15.   COMMENT command                                        138

Command Line 16.   QUIT command                                           138

Command Line 17.   OUTGRAPH command                                       139

Command Line 18.   FPROB command                                          139

Command Line 19.   TOPLINES command                                       140

Command Line 20.   BOTLINES command                                       141

ANTENNA COMMAND LINES

Page

Command Line 21.   Terminated rhombic command                             142

Command Line 22.   Vertical monopole command                              143

Command Line 23.   Horizontal dipole command                              144

Command Line 24.   Horizontal Yagi command                                145

Command Line 25.   Vertical log periodic command                          146

Command Line 26.   Curtain command                                        147

Command Line 27.   Sloping vee command                                    148

Command Line 28.   Inverted L command                                     149

Command Line 29.   Sloping rhombic command                                150

Command Line 30.   Interlaced rhombic command                             151

Command Line 31.   Constant gain command                                  152

Command Line 32.   Antenna pattern read from file                         153

LIST OF SYMBOLS

The meanings of the most commonly used symbols are as follows (except as otherwise defined within the text for local usage) (where feasible the symbol definition is the same as in the list in Davies, 1969):

GREEK LETTERS

		Coefficient used in exponential tail of electron density profile, Section 3.2.

_		Euler constant 0.57721.

		Angle of elevation or take-off measured from earth's surface to ray.

		Permittivity.

o,r,		Permittivity of free space.

'h, o, 	Errors used in correction of Martyn's theorem; Section 4.4.

		Angle measured from true ray path to earth's normal.

(Positive for up going ray, negative for down going ray.)

j		A sampled  in true ray path model, Section 5.0.

		Wavelength in a medium.

µ		Refractive index (real part of n).

µo		Permeability of free space.

µ'		Group refractive index.

µ'(h, f)	Corresponds to a particular true height h and operating frequency f, Section 4.2.

		Electron collision frequency.

		Average  in a region, used in loss equations, Section 6.1.

		_f2 -ffh

		Pi, 3.14159...

		Reflection coefficient.

		Standard deviation of various distributions.

		Time constant.

t		Angle between virtual ray and earth's normal at true reflection height.

		Angle between virtual ray and earth's normal at virtual reflection height.  (Usually at one-half path distance.)

j		Angle between virtual ray and earth's normal at a virtual height corresponding to a sampled ground distance, Section  5.0.

		Absorption index.

		Sun's zenith angle.

m		Maximum sun's zenith angle at which median predicted Fl layer exists, Section 2.4.2.

		One-half of the angle subtended by a radio path at the center of the earth, i.e., one-half path ground distance divided by the radius of the earth.

		Angular frequency.

H		Angular gyro-frequency.

ROMAN LETTERS

A(fv)		Absorption factor corresponding to fv, Section 6.1.

A		Averaged absorption factor, Section 6.1.

AD		Deviative loss absorption factor, Section 6.2.

AE		E mode absorption factor for corrected loss, Section 6.2.

AT(fv)	Sum of absorption factors, Section 6.2.

a		Radius of the earth.

B(fv)		Absorption factor for  AD, includes averaged collision frequency () profile, Section 6.2.

CE		Coefficient for B(fv) in E region, Section 6.3.

CF		Coefficient for B(fv) in F2 region.

C1		Coefficient for B(fv) in Fl region, Section 6.3.

C2		Coefficient for B(fv) in F2 region when Fl layer is present, Section 6.3.

c		Velocity of waves in free space.

D		Ground distance of a radio path.

Dj		Ground distance to a sampled point of a radio path, Section  5.0.

E		RMS field strength referred to one microvolt per meter.

e		Charge on the electron.

F		Coefficient used in exponential tail of electron density profile, Section 3.2.

fbEs		Sporadic E blanketing frequency.

fc		Critical or penetration frequency.

fH		Gyro-frequency.

fN		Plasma frequency.

fob		Frequency of oblique radio path.

fv		Vertical sounding frequency.

fpE		Penetration frequency of E layer.

fmEs		Equivalent oblique frequency corresponding to foEs.

foE		Critical frequency of the ordinary component of the E layer.

foEs		Highest frequency of the ordinary component of the sporadic E layer.

foFl		Critical frequency of the ordinary component of the Fl layer.

foF2		Critical frequency of the ordinary component of the F2 layer.

fu, fv		Plasma frequencies used for filling in the valley in an electron density profile, Section 3.4.

Gt		Transmitting antenna power gain relative to an isotropic antenna in  free space.

Gr		Receiving antenna power gain relative to an isotropic antenna in free space.

h		True or real height.

h'		Virtual height, equivalent height, group height.

hp		Phase height.

h'Es		The minimum virtual height of the sporadic E layer.

hmE,		Height of maximum electron density of E layers.

hmFl,hmF2	Height of maximum electron density of Fl or F2 layers.

h'E		The minimum virtual height of the E layer.

h'F		The minimum virtual height of the F layer (Fl or F2).

h'F2		The minimum virtual height of the F2 layer.

hpF2		The virtual height of the F2 layer at a vertical sounding frequency  fv = 0.834 foF2.

h'v		Virtual height from vertical sounding as used in corrected Martyn's  theorem, Section 4.3.

h'ob		Corrected virtual height for an oblique radio path, Section 4.3.

ho		Minimum height of electron density profile, Section 4.2.

hr		True height upper limit of integration for a virtual height, Section 4.2.

hj		A sampled true height integration for a virtual height, Section 4.2.

h2		True height of F2 layer at foFl, Section 3.5.

hu		True height of F2 layer at a frequency of fu = Xu foE, used in valley  of electron density profile, Section 3.4.

hv		True height of top of E layer at fv = Xu foE, Section 3.4.

(h'-h)N	Normalized height difference factor used in non-deviative loss, Section 6.2.

I		Absorption index.

k		Correction factor in oblique transmission curve.

Lbf		Free space basic transmission loss.

Lb		Basic transmission loss, i.e., system loss, Ls, where actual antennas are replaced by isotropic, loss-free antennas.

L(fv)		Ionospheric loss for an oblique radio path at frequ, fv, Section 6.1.

L(fob)		Ionospheric loss for an oblique radio path at freq, fob, Section 6.1.

Lc		Loss correction factor for E modes, Section 6.2.

Li		Losses caused by ionospheric absorption.

Lo		Sporadic-E obscuration loss for a mode passing through the layer, Section 6.3.

LR		Sporadic-E reflection loss for a mode reflection for the layer.

Ls		Signal power available at the receiving antenna terminals relative to the available power at the transmitting antenna terminals, in decibels.  This excludes any transmitting or receiving antenna transmission line losses.

M(D)		M factor of a ground distance D, the ratio of fob to fc.

M(3000)F2	M factor for F2 layer at a distance of 3000 km.

m		Exponent of nonlinear transformation used in integration, Section 42.

N		Electron density.

n		Complex refractive index, (µ-i).

o		Ordinary wave.

P		Phase path; power; probability.

P'		Virtual path; equivalent path; group path.

R		Sunspot number; retardation.

SSN		Sunspot number.

S		Quantity relating hpF2 and M(3000)F2 (usually taken as 1490.) Section 2.4.2.

S1		Slope of a linear Fl layer, at a frequency of foFl, Section 3.5.

S2		Slope of a parabolic F2 layer, at a frequency of foFl, Section 3.5.

t		Time.

Xu		Frequency ratio at F2 layer for valley fill, Section 3.4. que radio path at frequency, f-, Section 6.1.

Xv		Frequency ratio at E layer for valley fill, Section 3.4

Xj		An abscissa for Gaussian integration, Section 4.2.

wj		Weight for Gaussian integration, Section 4.2.

ym		Half thickness of a parabola.

ymE		Half thickness of a parabola for E layer.

ymFl, ymF2	Half thickness of a parabola for F1 or F2 layer.

## Ionospheric Communications Enhanced Profile Analysis and

## Circuit Prediction Program User's Manual

This report describes the operation and use of the Ionospheric Communications Enhanced Profile Analysis and Circuit Prediction Program (ICEPAC).  The computer program is an integrated system of subroutines designed to predict high-frequency (HF) sky-wave system performance and analyze ionospheric parameters.  These computer-aided predictions may be used in the planning and operation of high-frequency communication systems using sky-waves.

This report contains instructions for the use of ICEPAC.  A description of the input data requirements, including data definition, organization, and instructions for setup of the various analysis tasks, is presented.  Procedures and formats are given for preparing the input data and executing the program.  The various outputs are presented and described with an interpretation of the analysis results.

Key Words:  communications; computer model; high frequency; ionosphere; LUF; MUF; sky-wave; user's manual.

1.   INTRODUCTION

In the initial planning or in the modification of many communication systems, there may be an appreciable delay between the circuit planning and the actual circuit construction or modification.  This is of particular importance for high frequency circuits which have marked time and geographic variations in optimum frequency, required power, and system performance.  Predictions of ionospheric characteristics and techniques for using these characteristics are, however, available and may be used to anticipate the performance of HF communication circuits and thereby provide the lead time for necessary equipment selection, frequency selection, and frequency and time-sharing arrangements.

High-frequency radio communication depends upon the ability of the ionosphere to return the radio signals back to earth.  Prediction of ionization levels in the various regions of the ionosphere is, therefore, essential to any prediction of HF sky-wave circuit performance.  The maximum frequency returned from the ionosphere usually establishes the upper limit of the useful HF range.  The degree of ionization in the various regions is useful in estimating probable modes, and the transmission loss for these modes is combined with the antenna performance and transmitter power available to estimate the expected HF sky-wave signal available anywhere at any time.

The expected sky-wave signal may be compared with the expected radio noise environment to predict the likelihood that the circuit will operate satisfactorily.  This likelihood may be used to select optimum frequencies, proper antennas, required transmitter power, optimum time of operation, and broadcast coverage as a function of time and/or frequency.

This user's manual is subdivided into several sections.  Section 2 describes the primary program-related requirements, including a program operation review, computer implementation requirements, and data files used by ICEPAC.  The input language definition, input data description, command line image formats, and input requirements (including data organization, maximum system size, restrictions, and default values) are presented in Section 3.  The various ICEPAC task options and control parameters are also discussed in Section 3.  Section 4 describes the output options and possible program termination error messages. Several applications for system performance predictions are presented in Section 6 along with a sample test case.  The geometry of the internal antenna package, ITSA-1, and typical antenna patterns generated are presented in Figures 29 through 51. Section 5 also contains a summary of the tabular and graphical outputs available with the ICEPAC version IC.10 computer program. Section 6 briefly describes the data base file used with the program.  Section 7 contains input examples to assist the user in constructing ICEPAC input.

2.   PRIMARY PROGRAM RELATED REQUIREMENTS

This section briefly describes the primary program related requirements of the ICEPAC program.  Included are computer implementation requirements, data files used by ICEPAC, and a program operation review.

2.1  Computer Implementation Requirements

The ICEPAC program was developed on the IBM PS2 Model 80 computer using FORTRAN 77. The computer code is as close to being ANSI 77 Standard FORTRAN as was possible to ease adaptation to other computers.  The computer resources required to run this program are 512K bytes of memory with a hard disk or floppy disk.  A math co-processor is not required for Version .10.  The program resides on disk as a single run module and does not require overlays.

2.2  Files Used By ICEPAC

The primary input is from a disk file with records resembling card images.  The file name is requested interactively from the user.  The program execution is then controlled by the data on the disk file and operator intervention is not allowed until the complete file has executed.  Each Command Line format consists of an alpha-numeric name of up to 10 characters, followed by up to 14 data fields of 5 columns each.  Each floating point variable may have a decimal point inserted, while the integer variables must be right justified in the field.  The name identifiers mentioned above constitute an input language which is the primary communications link between the user and the actual program analyses.  It provides a means of controlling the extent of an analyses as well as the type and amount of output desired.

The secondary input is the long term ionospheric coefficients which are stored on disk files in monthly segments.  Each segment represents one month for low and high sunspot number and all hours of the day.  When the month and sunspot number are read from the primary input file, the coefficients for the particular month are read and interpolated for the sunspot number.  The resulting coefficient data base is used for further ionospheric parameter calculation relative to time and geographic location.  A complete description of these data base files can be found in Section 6.  These data are not user-defined input.

The magnetic coordinate file is used in the polar model evaluation for converting from geomagnetic coordinates to corrected geomagnetic coordinates.  The file is not specified by the user.

The output from the program is written to a disk file in an 80 column format.  When the output format requires more than 80 columns, the output is either folded in half or out put as two separate pages.  The output from MUF-FOT calculations for the full ionosphere has been completely reformatted to conform to the column limitations.

The antenna output file is created if specified by the user.  This file contains antenna patterns created by the ICEPAC antenna package (ITSA-1). These antenna patterns can be saved as a permanent file and used as input to the ICEPAC program at a later date or possibly used as input to some other analysis program.  This is discussed in Section 3.4.

The antenna input file is a permanent file used to read antenna patterns that have previously been created by ICEPAC or by some other antenna package or routine.  If specified by the user, the antenna patterns are read from this file instead of being computed by the ICEPAC program (see ANTENNA control line subsection). Use of this option to input antenna patterns not generated by ICEPAC generally requires the user to create a software interface to ICEPAC.

2.3  Program Operation Review

The first step in using the ICEPAC program is to assemble the appropriate data for the system configuration to be analyzed.  These data are then prepared on command lines which are read as input directly to the computer program or perhaps stored on a disk file for reading as input to the program at a later date.

Each command line contains a name identifier that is used as a label to identify the specific type of data on that particular command line.  A listing of all valid name identifiers is given in Table 2, and a complete description of each name identifier and the associated input data is given in Section 3.3.

The computer program checks the name identifier of each command on the input file with the internal table of valid name identifiers.  If an invalid name identifier is located, the program responds with a command line error and terminates execution.  Errors in the name identifier are most often caused by a spelling error in the identifier name.

At this time, there is little or no diagnostic check of the data contained on any command line.  Only the name identifier is examined.  Thus, program execution will continue if there are no name identifier errors even though there may possibly be errors in the actual input data.  It is, therefore, essential for the user to examine each command line image to insure that it is typed correctly and that it contains the necessary data before attempting to execute the ICEPAC program.  Once the configuration is complete, the analysis is performed.

The ICEPAC program performs four basic analysis tasks which are discussed in detail in the ICEPAC theoretical report and summarized below:

(1)	Ionospheric Parameters.  The ionosphere is predicted using parameters which describe four ionospheric regions:  E, Fl, F2, and Es.  For each sample area, the location, time of day, and all ionospheric parameters are derived.  These may be used to find an electron density profile, which may be integrated to construct a predicted ionogram.  These options are specified by methods 1 and 2  which are briefly described in Table 3.

(2)	Antenna Patterns.  The user may precalculate the antenna gain pattern needed for the system performance predictions.  These options are specified by methods 13, 14, and 15, which are briefly described in Table 3.  If the pattern is precalculated, then the antenna gain is computed for all frequencies (1-30 MHz) and elevation angles.  If the pattern is not precalculated, then the gain value is determined for a particular frequency and elevation angle as needed.

(3)	Maximum usable frequency (MUF,.  The maximum frequency at which a sky-wave mode exists can be predicted.  The 10% (FOT), 50% (MUF), and 90% (HPF) levels are calculated for each of the four ionospheric regions predicted.  These numbers are a description of the state of the ionosphere between two locations on the earth and not a statement on the actual performance of any operational communications circuit. These options are specified by methods 3 to 12, which are briefly described in Table 3.

(4)	Systems Performance.  A comprehensive prediction of radio system performance parameters (up to 22) is provided.  Emphasis is upon the statistical performance over a period of a month.  A search to find the lowest usable high frequency (LUF) is provided.  These options are specified by methods 16 to 29, which are briefly described in Table 3.

3.   INPUT DATA REQUIREMENTS

This section describes the general data requirements and the input language definition.  It also includes a complete description of all user-defined input data and the corresponding command line input formats.  The minimum input requirements and default values for each analysis and prediction method are presented, as are the data organization and input restrictions.

3.1  General Data Requirements

The data requirements of the ICEPAC program consist of a fixed data base and user-defined input data.  Much of the basic data required to use the ICEPAC program is stored on the ionospheric long-term data base files.  These data base files includes geographic and time variations of the ionosphere and of atmospheric noise levels, the relationship between the ionospheric characteristics and the propagation path geometry and signal attenuation, and the theoretical performance of common antenna systems.  The data base is fixed to the user and is not part of the user-defined input data.  A complete description of the ionospheric long-term data base is contained in Section 6.  	The user-defined input data consist of line commands and perhaps an optional antenna file and the input depends primarily on the complexity of the analysis or system performance prediction requested (see Section 3.4).  As an example, to determine the upper useful frequency limits, only the geographic location of the circuit terminals and the time are required.  For more complex predictions such as the expected reliability of communications, additional user-defined input data are required.  This data would include frequency of operation, the type of antenna used, the transmitter power, the type of service required, and the man-made noise environment, among others.  A complete description of all user-defined data is presented in Section 3.3, and the required user-defined input data for each analysis or prediction method are summarized in Section 3.4.  The input required for each run option is given in Table 13.  The details of each command line format are given in Section 3.5.

3.2  Input Language Definition

The name identifiers mentioned in Section 2.3 constitute an input language which is the primary communication link between the user and the actual program analysis.  The language provides the user with the means of describing complex Physical configurations in somewhat familiar terms.  It also provides a means of controlling the type and the extent of the analysis to be performed as well as the type and amount of output desired.  Desirable characteristics of the input language include (1) format simplicity of the input statement and (2) quasi-order independence of the input data for a particular system configuration.  Thus, the input line commands can often be presented to the program in any order (with some restrictions) to describe the topology of a system configuration, the type of analysis to be performed, and the output desired.  Once these characteristics for a particular configuration have been defined, the program can be executed. Another desirable characteristic is the ability to alter certain characteristics of the configuration without having to redefine all other characteristics.  This allows the user to alter certain parameters in the system configuration while keeping other parameters fixed.  The benefit of this is that it allows the user to execute the program for several configurations that have common characteristics in the same run without redefining all characteristics of the system configuration.

3.3  Description of the User-defined Input Data

The user-defined input data are in the form of line commands and perhaps an optional antenna data file.  As mentioned in Section 2.3, each input line command contains a name identifier that is used as a label to identify the specific type of data on that particular line command.  A listing of all valid name identifiers is presented in Table 2, and a complete description of each name identifier and the associated data is given in the following subsections along with line command formats and examples.  Each input line command will henceforth be called a control line with the name identifier used as the name of the control line.  Formats for each line command are given in Section 3.5.  All commands start with a 10 column identifier.  All data fields are five columns long.

3.3.1   Program Control, Execution, and Termination Command Lines

The following command line defines the analysis task to be performed, indicate when the task will be performed for the current system configuration, and indicate how the program will reach a normal termination.

METHOD command line

The METHOD command defines the analysis task to be performed for a particular system configuration.  The parameter on the METHOD command line is (see Command Line 1.  References to the command line contain an example of each command along with a table of the contents of the line):

- METHOD is the parameter which controls the type of program analysis and predictions performed.  At the present time there are 30 task options available.  Many of these tasks differ only in the representation of the output and require the same (or nearly the same) computations.  The basic tasks consist of (1) tabulation of ionospheric parameters and ionograms from the ionospheric data base file, (2) MUF-FOT calculation and graphs, (3) computation and tabulation of antenna patterns, (4) system performance predictions, and (6) LUF calculation and graphs.  A brief description of each method available is given in Table 3, and a presentation of the output generated by each method is given in Section 4.

EXECUTE command line

The EXECUTE command causes the program to perform the indicated analysis task for the currently defined system configuration.  This requires the user to specify the program control command line and the system configuration command line prior to the EXECUTE command (see Command Line 10).  There is only one parameter on the EXECUTE command line described as follows:

- KRUN is the parameter that indicates the layers at the ionospheric parameters that are calculated.  If KRUN is less than or equal to zero, then the parameters of all layers (E, Fl, F2, and Es) are calculated; if KRUN = 1, then the parameters of the E layer, Fl layer, and F2 layers are calculated; if KRUN = 2, the parameters for the Es layer only are calculated; if KRUN is greater than or equal to three, then no ionospheric parameters are calculated.  This allows the user to skip the calculation of ionospheric parameters and use the last set previously calculated or a set of ionospheric parameters entered with the command lines.  The use of the KRUN option may require additional consideration beyond that presented here.

It is also often possible to cause program analysis to be performed by having two consecutive command lines with the same name identifier.  The command lines that allow this are:

METHOD

CIRCUIT

EXECUTE

Thus, if the user desires to have both a tabulation of two circuits, he may accomplish this by having two consecutive CIRCUIT commands on the input file.  The remainder of this system configuration is unchanged if this technique is used.

QUIT control command

The QUIT command causes termination of the ICEPAC program.  It must, therefore, be the last physical command line on the user-defined input file.  This command is essential to the program since it is used as the program termination indicator.  There are no data parameters on the QUIT command line (see Command line 16).

3.3.2  Diurnal, Month, and Solar Activity Control Lines

The following command lines define the time of day, the month, year, and sunspot number (solar activity) period of interest for which the analyses and predictions are performed.

TIME command line

The TIME command indicates the time of day for which the analysis and predictions are to be performed.  The hours can be either universal time or local mean time at the transmitter, depending on an input parameter.  There are four parameters on the TIME command line, described as follows (see Command line 6):

(a)	IHRO indicates the starting hour in universal time or in local mean time at the transmitter.

(b)	IHRE indicates the ending hour in universal time or in local mean time at the transmitter.

(c)	IHRS indicates the hourly increment.  The hourly increment is added to the starting hour to determine the next hour.  This incremental process continues until the ending hour is reached.

- ITIM indicates that the specified time is universal time or local mean time.  If ITIM is negative, then the specified times are local mean times (LMT) at the transmitter, otherwise the specified times are universal time (UT).

MONTH command line

The MONTH command indicates the year and months for which the analysis and prediction are to be performed.  The year is used for identification purposes only and has no effect on the program calculations.  June and December are typical of seasonal extremes for long-range planning.  The parameters on the MONTH command line are as follows (see Command line 2):

(a)	NYEAR indicates the year, but has no effect on the program calculations.

(b)	MONTH is a variable where 1 represents January, 2 represents February, etc.  The program analysis and predictions are performed for each of the months the user specifies.  The desired month can be specified in any order.  This, for example, allows the user to perform the analysis and predictions for December before January if he so desires.

SUNSPOT command line

The sunspot command line indicates the sunspot numbers of the solar activity period of interest and is the 12-month smoothed mean for each of the months specified.  The second is the "effective" geomagnetic activity index (QE).  This parameter is derived from the value of the effective Kp. (See table 12 and figure 52 for derivation of QE from Kp.  A sunspot number of 10 is typical for low solar activity and sunspot numbers between 110 and 130 are typical of high solar activity.  NOTE: monthly or daily solar activity indices are not used, only the expected 12-month running average is used. When QE is not available, the value 0.0 is used, thus depicting an undisturbed period.

3.3.3   System Configuration Command Lines

The following command lines identify and define the system configuration to be analyzed.  This includes a label to identify the system coordinates, the location of the transmitter and the receiver, and the actual system parameters necessary to define the system configuration.

LABEL command line

The LABEL command contains alphanumeric information used to describe the system location on both the input and output.  A recommended usage of the LABEL command is to specify a label for each transmitter-receiver configuration used, where the alphanumeric information describes the location of the coordinates given on the CIRCUIT command line.  The parameters on the LABEL command line are as follows (see Command Line 8):

(a)	ITRAN is an array of 20 alphanumeric characters used to describe the transmitter location.

(b)	IRCVR is an array of 20 alphanumeric characters used to describe the receiver location.

It should be noted that the information on the LABEL command line is used for identification purposes only.  Thus, the user is allowed a maximum of 40 alphanumeric characters to describe the transmitter-receiver location and need not be concerned with which characters are stored in ITRAN and which are stored in IRCVR.

CIRCUIT command line

The CIRCUIT command contains the geographic coordinates of the transmitter and receiver and a variable to indicate the user's choice between shorter or longer great circle paths from the transmitter to the receiver.  Normally the short circle path is desired by the user.  On the CIRCUIT command line, the symbol N denotes northern hemisphere, S denotes southern hemisphere, W denotes western hemisphere, and E denotes eastern hemisphere (the directions may be spelled out if desired; such as NORTH).  The parameters on the CIRCUIT command lines are as follows (see Command Line 3):

(a)	TLATD indicates the latitude at the transmitter in degrees.

(b)	ITLAT indicates the northern (N) or southern (S) hemisphere at the transmitter.

(c)	TLONGD indicates the longitude at the transmitter in degrees.

(d)	ITLONG indicates the eastern (E) or western (W) hemisphere at the transmitter.

(e)	RLATD indicates the latitude at the receiver in degrees.

(f)	IRLAT indicates the northern (N) or southern (S) hemisphere at the receiver.

(g)	RLONGD indicates the longitude at the receiver in degrees.

(h)	IRLONG indicates the eastern (E) or western (W) hemisphere at the receiver.

- NPSL indicates the user's choice between shorter or longer great circle paths from the transmitter to the receiver.  The longer great circle path is used if the user specifies NPSL = 1, otherwise the shorter great circle path is used.  Often desired is the shortest distance between transmitter and receiver.  This is accomplished by leaving the variable NPSL blank, therefore, selecting the shorter great circle path by default.

SYSTEM command line

The SYSTEM command line includes parameters necessary to define the system configuration.  This includes the transmitter power, man-made noise level, minimum take-off angle, required circuit reliabilities, required signal-to-noise ratio, the maximum difference in delayed signal power for multi-path, and the maximum difference in delay time for multi-path.  The parameters defined on the SYSTEM commands are described below (see Command Line 4):

(a)	PWR indicates the transmitter power in kilowatts.  Note that this is the power delivered to the transmitting antenna.  (The program internally converts this to decibels (db) above a kilowatt as needed for computations.)

(b)	XNOISE indicates the expected man-made noise level at the receiver in dBW (decibels below lW) in a 1 Hz bandwidth at 3 MHz.  XNOISE should be input as a positive value unless the user desires to designate the receiving location area as industrial, residential, rural, or remote un-populous by specifying:

XNOISE = -1. for industrial (the program then uses -125. dBW)

XNOISE = -2. for residential (the program then uses -136. dBW) XNOISE = -3. for rural (the program then uses -148. dBW)

XNOISE = -4. for remote un-populous (the program then uses -164. dBW).  In remote un-populous areas, cosmic noise will normally dominate over man-made noise.

(c)	AMIND indicates the minimum takeoff angle in degrees.  The value is normally very small unless antenna performance is expected to be so poor at low angles that these angles should not be used in the estimation of upper useful frequencies, or if the horizon is so obstructed that low takeoff and reception angles appear unlikely. The program uses a default value of 3o if this parameter is not set by the user.  A value of 0.001 will effectively give the full range of possible radiation angles and should be specified by the user if the 3o default is not required.

(d)	XLUFP indicates the required circuit reliability, which is an estimate of the percentage of days within the month that the signal quality will be acceptable, and should be specified for calculation of the LUF or time availability for service probability. (NOTE: XLUFP is expressed as a percentage.)  The program uses a default value of 90 if this parameter is not set by the user.

(e)	RSN indicates the required signal-to-noise and is the ratio of the hourly median signal power in the occupied bandwidth relative to the hourly median noise in a 1 Hz bandwidth, which is necessary to provide the type and quality of service required (expressed in decibels).  Table 4 and Table 6 show typical required S/N ratios for radiotelephone service and radio-teletype service.  NOTE:  The S/N values in this report and in the computer model are the ratio of signal in occupied bandwidth to noise in a 1 Hz bandwidth.

(f)	The user may choose the multi-path computations.  The multi-path probability is an estimate of the likelihood that two or more sky-wave modes will exist within the specified power tolerance and outside the time delay tolerance.

PMP indicates the maximum difference in delayed signal power in decibels between sky-wave modes to permit satisfactory system performance in the presence of multiple signals.  If PMP is blank or zero, multi-path is not considered.

- DMPX indicates the maximum difference in delay time in milliseconds between sky-wave propagation modes to permit satisfactory system performance in the presence of multiple signals.  The program uses a default value of .86 if this parameter is not set by the user.

FREQUENCY command line

The FREQUENCY complement command line contains up to 11 user-defined frequencies that are used in the calculation.  The FREQUENCY command is not necessary with the analysis and prediction options dealing only with ionospheric parameters, ionograms, MUFs, antennas, or LUFs.  system performance requires the FREQUENCY command.  The parameter on the FREQUENCY command is as described below (see Command Line 7):

FREL is the array of up to 11 user-defined frequencies in megahertz.  If the first frequency on the command line is left blank, then the FOT is inserted as the first frequency.  If the entire FREQUENCY command line is left blank, then the 11 frequencies used in the calculation are computed for each hour by the program.  The computed frequency range then begins at 2 MHz and ends at the HPF on a non-linear scale based on the E-MUF and F-MUF values.  It should be noted that, in this manner, a different frequency complement is generated for each hour.

ANTENNA command line

The ANTENNA command line defines whether the antenna is transmitting or receiving, specifies the type of the antenna, the ground conductivity, and dielectric constants in addition to the other parameters necessary to compute the necessary antenna patterns.  Typical values for selected ground types are given in Table 6 where typical good ground is low hills with unforested rich soil or flat wet coastal regions; fair ground is medium hills or forested heavy soil; and typical poor ground includes rocky steep hills, sandy dry coastal regions, and city industrial areas.  The ground constants are optional on each antenna command. If not specified by the user, they will be calculated with the land mass map from the ionospheric long-term data base file.  The ground constants will be set to poor ground as a default if they are not specified by the user and the land mass map is not available.  It is permissible for the user to specify up to three transmitter antennas and three receiver antennas to cover the frequency range considered for each system configuration.  It is also possible to indicate that the antenna is to be input from an external antenna file instead of being calculated by the program.  The input parameters required vary with antenna type.  The basic ANTENNA command line parameters are described below, and the geometric description of each available antenna type is given in Figures 29 through 51, and Command lines 21 through 32:

(a)	IAT indicates whether the antenna is transmitting or receiving, IAT = 1 for transmitting, IAT = 2 for receiving.

(b)	IANTR indicates the number which corresponds to the antenna type desired.  The user has the option to input the antenna pattern from the optional external antenna file as discussed in Section 3.4.

(c)	AETA indicates the antenna bearing in degrees from east of north.  Negative denotes the off azimuth angle from the circuit path.  Positive denotes the main beam direction.

(d)	ASIG indicates the ground conductivity (o) in mhos per meter.

- AEPS indicates the relative dielectric constant.

The following parameters describe various characteristics dependent on the antenna type specified.  The user should refer to Section 3.4 for the description of the following antenna parameters and the antenna characteristics required for each antenna type:

(f)	AND

(g)	ANL   The definition and description of these parameters

(h)	ANH    depend on the antenna type specified.

(i)	AEX

(j)	AFQB indicates the ending frequency when the user specifies more than one transmitting type or receiving antenna type for the frequency range considered.  The next antenna type used for the transmitter or receiver would then begin at 1 MHz greater than the ending frequency specified here.

- IAIN indicates the antenna number (up to 3) when the user specifies more than one transmitter or receiver antenna for the frequency range considered.  This parameter is also used as a control parameter to specify the location on the optional external antenna file if one is used.  This usage is discussed in Section 3.4.

3.3.4   User-defined Data Base and system Override Commands

The following command lines allow the user to specify an external data base consisting of (1) the geographical and geomagnetic parameters relating to a specified sample area, (2) the critical frequency, semi-thickness, and height of maximum ionization for the E, Fl, and F2 layers, (3) lower, median, and upper decile values of the critical frequency and the virtual height of reflection, and (4) the true height and electron density.  The ionospheric long-term data base file is not necessary when these ionospheric parameters are defined by the user. The user may also override internal program characteristics such as (1) the integration scheme used for the E and F2 layers and (2) the critical frequency multipliers to adjust the heights of the ionospheric layers.  These values can be obtained using METHOD equal to 1 in a previous run.  Use of the command lines in this section require in-depth information of ionospheric parameters and would be avoided by most users.  These command lines are used to alter the use of predefined ionospheric data.

EFVAR command line

The EFVAR command line allows the user to indicate the critical frequency, semi-thickness, and the height of maximum ionization for the E, Fl, and F2 layers for a specified sample area.  The user may specify the following parameters for each of the sample areas defined on the SAMPLE command lines (see Command Line 11):

(a)	I indicates the specific sample area number, 1 to 5.

(b)	FI(1,I) indicates the critical frequency for the E layer, foE, in megahertz for the specified sample area.

(c)	YI(1,I) indicates the semi-thickness for E layer, ymE, in kilometers for the specified sample area.

(d)	HI(1,I) indicates the height of maximum ionization for the E layer, hmE, in kilometers for the specified sample area.

(e)	FI(2,I) indicates the critical frequency for the Fl layer, foFl, in megahertz for the specified sample area.

(f)	YI(2,I) indicates the semi-thickness for the Fl layer, ymFl, in kilometers for the specified sample area.

(g)	HI(2,I) indicates the height of maximum ionization for the Fl layer, hmFl, in kilometers for the specified sample area.

(h)	FI(3,I) indicates the critical frequency for the F2 layer, foF2, in megahertz for the specified sample area.

(i)	YI(3,I) indicates the semi-thickness for the F2 layer, ymF2, in kilometers for the specified sample area.

- HI(3,I) indicates the height of maximum ionization for the F2 layer, hmF2, in kilometers for the specified sample area.

ESVAR command line

The ESVAR command line allows the user to indicate lower, median, and upper decile values for the Es layer and the virtual height of reflection for a specified sample area.  The user may specify the following parameters for each of the sample areas defined on the SAMPLE command lines (see Command line 12).

(a)	I indicates the specific sample area number, 1 to 5.

(b)	FS(1,I) indicates the lower decile critical frequency for the Es layer, foEs, in megahertz for the specified sample area (example lowest value = 2 MHz).

(c)	FS(2,I) indicates the median decile critical frequency for the Es layer, foEs, in megahertz for the specified sample area (example median value = 4 MHz).

(d)	FS(3,I) indicates the upper decile critical frequency for the Es layer, foEs, in megahertz for the specified sample area (example highest value = 6 MHz).

- HS(I) indicates the virtual height of reflection for the Es layer in kilometers for the specified sample area (example vertical height = 110 kilometers).

EDP command line

The EDP command line allows the user to read and use an external electron density profile for a specified sample area.  Current implementation restrictions prohibit defining more than one sample area if an external electron density profile is used.  The user may specify the following parameters for one sample area (see Command line 13).  A temporary parameter is used to switch to the internal electron density profile if desired.

(a)	JSAMP indicates the specific sample area number (=l).

(b)	ITEMP is a temporary parameter which causes the internal electron density profile to be used if the user specifies the word "OFF."  Thus, this parameter acts as a switch to "turn off" the external electron density profile and "turn on" the internal electron density profile.  If not "OFF," then the EDP command line is followed by eight lines which contain:

HTR is the array of 50 that indicates the true heights in kilometers (four lines, see Command line 13a).

FNSQ is the array of 50 that indicates plasma frequency squared in 'MHz)2 (four lines, see Command line 13b).

INTEGRATE command line

The INTEGRATE command line allows the user to specify model segment integration for the F2 layer instead of the slower Gaussian integration.  Thus, the user can request fast integration for the E layer and for the F2 layer when the Fl is not present.  The D-E layer is preset using calculations from the Gaussian integration.  A temporary parameter is used to allow the user to alternate between the model-segment and Gaussian integration.  The program performs the slow Gaussian integration as a default if no INTEGRATE command is present.

- INTEG is the temporary parameter used to specify the integration method. If the user specifies "-l" on the INTEGRATE command line, then Gaussian integration is performed.  If the user specifies a value greater than or equal to zero, then model-segment integration is performed.  If "OFF" is specified (variable ITEMP), the program returns to the fast integration (see Command line 9).

FPROB command line

The FPROB command line allows the user to adjust the heights of the E, Fl, F2, and Es ionospheric layers.  The predicted critical frequencies are multiplied by the parameters on the FPROB command to raise or lower each corresponding ionospheric layer.  If the user-defined parameter is greater than one, the critical frequencies used by the program will be larger than the predicted critical frequencies.  If the user-defined parameter is less than one, the critical frequencies used by the program will be smaller than the predicted critical frequencies.  The user may remove the Fl and Es layers entirely by specifying the value "0.0" for the critical frequency multiplier for either layer.  The user should not, however, specify the value "0.0" for the critical frequency multiplier for the E or F2 layers.  The following default values are used if (1) the critical frequency multipliers are not specified by the user or (2) the value specified for the E or F2 layer is negative or zero.  The program uses the default value of 1.0 for the critical frequency multipliers for the E, Fl, and F2 layers.  Thus, the actual predicted critical frequency values are used by the program and the corresponding ionospheric layer ionization is neither raised nor lowered.  The default value of the critical frequency multiplier for the Es layer is 0.7 to allow for median losses.  A temporary parameter is specified if the user desires to use the default values for the actual frequency multipliers rather than the values previously specified on the FPROB command.  If the user indicates "OFF" on a subsequent FPROB command line, the default critical frequency multiplier values are used.  The user may also redefine the critical frequency multipliers previously defined by providing a subsequent FPROB command line.  The following parameters are on the FPROB command line (see Command line 18):

(a)	PSC(1) indicates the critical frequency multiplier for the E layer.

(b)	PSC(2) indicates the critical frequency multiplier for the Fl layer.

(c)	PSC(3) indicates the critical frequency multiplier for the F2 layer.

- PSC(4) indicates the critical frequency multiplier for the Es layer.

3.3.5   Input, Output, and Comment Command Lines

The following command lines allow the user to place comments in the input file, use auxiliary input and output files, and identify additional input/output options available to the user.  These command lines are used to (1) create an external antenna pattern file and (2) allow the user to request the outputs of various program options without re-computing parameters.

COMMENT command line

The COMMENT command line permits the user to place comments anywhere in the user-defined input file.  This may benefit the user in specifying the system configuration in terms of descriptive text, and any number of comment lines may be used.  During program execution, the comment lines are listed along with the other command lines, but otherwise they have no effect on execution of the program (see Command line 15).

ANTOUT  command line

The ANTOUT command line allows the user to write antenna patterns generated by the ICEPAC program to a separate binary file if the program option to generate antenna patterns is used (methods 13, 14, 15).  This file could then be saved as a permanent file for use at a later date with the ICEPAC program, or possibly with some other analysis program.  This may be a desirable capability for the user who plans to use the same antennas for several sets of read data or to use the same antenna configuration for several periodic runs.  The user could generate and save the antenna patterns on a permanent file, then read the antennas from the permanent file when running each of his data sets.  It is also possible for the file to be used as a working file for direct input to some other analysis program.  The process of reading the antenna file is discussed in Section 6.

A temporary alphanumeric parameter on the ANTOUT command line is used to specify when antenna patterns are output to the antenna file (see Command line 14).  If the temporary parameter is left blank, then all antenna patterns generated following the ANTOUT command are written to the file.  If the temporary parameter is set to "OFF," then none of the antenna patterns generated following the ANTOUT command are written to the file.  It is often more efficient to generate the desired antenna patterns before performing other program analysis or predictions.  This process is demonstrated in Figure 12, where transmitter/receiver antenna pattern pairs are generated.

The user should be aware that the antenna patterns are generated and written on the antenna file only if methods 13, 14, or 15 are specified.  Thus, if the user has an ANTOUT command line but does not specify methods 13, 14, or 15, no antenna patterns are generated for output and consequently none are written to the antenna file.  Also, should the user specify methods 13, 14, or 16 but not include an ANTOUT command line, then the antenna patterns are generated and written to the primary (or auxiliary) output file along with any other program generated output, but the patterns are not written to the binary antenna file.  A method of using antenna patterns generated by the IONCAP program or by other means available to the user, is described in Section 6.3.

OUTGRAPH command line

The OUTGRAPH command line allows the user to specify the output of several methods without re-computing the variables.  This is of particular use if several line printer graphs or other output forms are needed for various combinations of variables.  A temporary alphanumeric parameter on the OUTGRAPH command line is used to indicate if additional output is requested for the current system configuration (see Command line 17).  If the word "OFF" is specified, then no additional output is desired.  If the user desires additional output, he simply indicates the method numbers (up to 12) of the output desired on the OUTGRAPH command.  As an example, if method 7 is run, the output from methods 8, 9, and 10 can be obtained without any additional computing as is demonstrated in Figure 13.

KTOUT is an array of up to 12 method numbers of the desired output.The user should be aware that the particular program option specified on the METHOD command dictates the output that can be requested on the OUTGRAPH command. There are two limitations as to the output available with certain program analysis options.  (1) The user may not request output on the OUTGRAPH command that requires computations which were not performed during the program analysis.  As an example, the user may not request output for methods which require system performance if only the MUF was computed.  (2) Several of the analysis options are performed within loops where the variables are not saved throughout the duration of the loop.  Such analysis options cannot be requested with the OUTGRAPH command since the loops are terminated before the output requested on the OUTGRAPH line is generated.  This implies that the user cannot always obtain the additional output by using the OUTGRAPH command.  It is, therefore, optimal to run the program analysis options (specified with the METHOD command) that are performed within the loops and specify any additional desired output with the OUTGRAPH command.

Should the user specify output for a method with the OUTGRAPH command which violates one of the above restrictions, the program ignores the particular request and attempts to process any additional user requests for output.  Thus, as an example, if the user should specify output for methods which require system performance and only the MUF was computed, the program would ignore the requests. Table 7 indicates the output available for methods specified on the OUTGRAPH line for each of the program analysis task options and also indicates the optimal combinations.  The user may request LUF output (methods 26-29) if system performance has been calculated by methods 16-25.  The LUF obtained would not be the computed LUF but would be the first frequency in the frequency complement that has a computed reliability greater than or equal to the required circuit reliability.  If none if the reliabilities are as large as the required reliability, the frequency with the largest reliability is chosen as the LUF.  A designator is printed to indicate the reliability.  The user would compute the actual LUF by specifying methods 26-29 rather than specifying this on the OUTGRAPH command line.

The OUTGRAPH command acts as a software implemented "sense switch."  Once the user specifies a desired output on an OUTGRAPH line, the user "enables" the software implemented "sense switch" and will continue to obtain the specified output options (when permissible) for every program analysis task he performs. This process continues until the user specifies either (1) an additional OUTGRAPH command or (2) an OUTGRAPH command with the word "OFF" specified, which disables the software implementation "sense switch."  Should the user obtain much more output than he believes he has requested when using an OUTGRAPH command and when processing several system configurations, it is probable that he needs an OUTGRAPH "OFF" command somewhere (usually following an EXECUTE command).  Should the user obtain less output than he has requested on an OUTGRAPH line, it is caused by an invalid user request that the program has ignored.  The OUTGRAPH command may be specified anywhere before the EXECUTE command to request additional output options; however, the additional output will be generated after the current task option has been completed.  If no OUTGRAPH command appears, the user receives only the output of the task option indicated on the METHOD command.  The output requested on the OUTGRAPH command line may be written to the auxiliary output file by specifying a negative method on the OUTGRAPH command line.

TOPLINES command line

The TOPLINES command line allows the user to select the output header lines he desires to print when running method 23.  The information output, for the most part, consists of the user-defined system configuration.  This command line, along with the BOTLINES command line, allows the user to indicate the specific output lines he desires and thus together they constitute a user-defined output option. The user can select the output desired rather than those defined by other method numbers.  There is one parameter array on the TOPLINES command line.  This option may be useful if the user does not desire the exact output generated by the task options implemented (see Command line 19).

LINTP is an array of up to 8 user-selected header lines.

Table 8 describes the header lines available to the user, along with the corresponding values of the parameter LINTP.  The user may redefine the header lines desired at any point in the input deck by specifying an additional TOPLINES command which overrides the previous specified lines.  The user may also cancel the header lines previously selected by indicating the word "OFF" as the value of a temporary parameter on the TOPLINES command.  The program has one default header line which consists of the program task option (METHOD), the program name and version number, as well as the page number.  If the user does not include a TOPLINES command or specifies TOPLINES "OFF," the default header line is the only one printed when method 23 is indicated.  The TOPLINES command has no effect on any program task option other than method 23.

BOTLINES command line

The BOTLINES command line allows the user to select the desired parameters to print when running method 23.  The information output consists of the current values of the variables the user indicates.  This command line, along with the TOPLINES command line, allows the user to indicate the specific output lines desired, and thus constitutes a user-defined output option.  The user may, therefore, request the desired output rather than those specified by other method numbers.  This may be useful if the user does not desire the exact output generated by the other task options.  There is one parameter array on the BOTLINES command (see Command line 20).

LINBD is an array of up to 14 user-selected variables.

Table 9 describes the variables available to the user along with the corresponding values of the parameter LINBD.  The user may redefine the variables desired at any point by specifying an additional BOTLINES command which overrides the previous specified variables.  The user also may cancel the variables previously selected by indicating the word "OFF" as the value of a temporary parameter on the BOTLINES command line.  The program outputs only the frequency complement as the default if the user does not include a BOTLINES command or specifies BOTLINES "OFF" when method 23 is indicated.

The BOTLINES command has no effect on any program task option other than method 23.  Table 10 describes the line numbers preset by using methods other than 23.

4.  OUTPUT OPTIONS

There are 30 output options that may be specified by the user (Table 3). These are divided into four subsets:

(1) ionospheric descriptions, METHOD = 1 or 2;

(2) antenna patterns, METHOD = 13, 14, or 15;

(3) MUF predictions, METHOD = 3 through 12; and

(4) LUF and system performance predictions, METHOD = 16 through 29.

The input required for each of the four run option subsets is summarized in Table 11.  All the output figures described in the next three subsections were generated using the command lines listed in Figure 2.  Each individual antenna type is described and an example given in the last subsection 4.4.  The first line of all the output options gives the method number, program version, and a page number, the start of which is specified by the user.

4.1  Ionospheric Parameters Output Options, METHOD = 1 or 2

There are two outputs, a list of ionospheric parameters (Figure 2) and an ionogram (Figure 3).  The data on Figure 2 are divided into a set of header lines and a body of output lines.

Line 2.	The month, day, year, and sunspot number are given on the     second line.

Line 3.	The first 40 characters on the third line are those specified by the user on the LABEL command line.

Line 4.	The fourth line is the location of the transmitter and of the receiver. The azimuths, transmitter to receiver, and receiver to transmitter, are given in degrees east of north.  The circuit distance is given in both nautical miles and kilometers.

Line 5.	The semi-thickness of the E layer, YE, the height of maximum ionization, HE, and the reflection height of the Es layer, HS, are given in kilometers.

The body of the figure consists of the result of sampling the ionosphere at one, three, or five areas along the path.  The output consists of 21 columns for each hour.  To display this output on an 80 column page, the output was folded at column 11 with the first 11 columns on line 1 and the last 10 columns on line 2 for each hour.  (There is only one sample area for this specific example.)  Columns 1 and 2 give the sample area location.  Columns 3 and 4 give the local time at the sample area and universal time (same for all sample areas).  Column 5 is the median E critical frequency, foE, in megahertz.  Columns 6  7  and 8 are the Fl-layer median critical frequency, foFl, in megahertz; semi-thickness, ymFl, in kilometers; and height of maximum ionization, hmFl, in kilometers.  Column 9 is one-half the gyro-frequency, fH.  Columns 10, 11, and 12 are F2 zero MUF, (foF2 - 1/2 fH), in megahertz; ymF2 in kilometers; and hmF2 in kilometers.  Columns 13, 14, and 15 are the Pole-ward, Center, and  Equator-ward  boundaries of the auroral trough in degrees north latitude. Column 16 is the F2 M(3000) factor. Column 17 is the local geomagnetic time, TCGM, in hours.  Column 18 is the ratio of hmF2 and ymF2.  Columns 19 is the zenith angle in degrees.  Column 20 is the flag that shows the location of the reflection point.  See the following table:

IFLAG = 0		Implies low lat/mid lat nighttime point

IFLAG = 1		Implies auroral zone nighttime point

IFLAG = 2		Is unused

IFLAG = 3		Is unused

IFLAG = 4		Implies low lat/mid lat sunrise/sunset point

IFLAG = 5		Implies auroral zone sunrise/sunset point

IFLAG = 6		Implies low lat/mid lat daytime point

IFLAG = 7		Implies auroral zone daytime point

IFLAG = 8		Is unused

IFLAG = 9		Implies polar cap point (all times)

Column 21 is the geomagnetic latitude.

The ionogram output option, method 2, is in Figure 3.  Up to three ionograms are constructed from the up to five sample areas along the circuit path. The ionogram plots are printed on two pages.  The first page contains the left half of the ionogram plot and the second page contains the right half of the ionogram plot.  Lines 2, 3  and 4 are the same as for Figure 2.  Line 5 gives the universal time, GMT, the local time at the sample area, the sample area location, the distances in kilometers from the transmitter at which the E and F parameters were taken, the type of integration used, and the manner in which the Fl layer was added to the electron density profile.  The graph is vertical sounding frequency (in megahertz) versus true and virtual heights (in kilometers) where true and virtual height are represented by "." and "x" respectively in the graph.  The Es layer is given by a line at the Es reflection height.  The character "U" is used until the 90 percent value of foEs is exceeded; "M" is used until the 50 percent value is exceeded; and "L" is used until the 10 percent value is exceeded.  The parameters for each layer are given in the upper left corner.  The table on the right gives the sounding frequencies and the true and virtual heights.

4.2  MUF Output Options, METHOD = 3 through 12

The maximum usable frequency (MUF) output options include all mode information as well as the distributions of the MUF for each layer.  First note that methods 3, 4  5  and 6 refer to the MUFs for the E(Fl) and F2 layers using the old nomogram method.  This assumes a virtual height of about 300 km.  The results of this model are not always valid in the 4000-10000 km range.  The other MUF output comes from a complete electron density profile.  All information possible from the MUF calculation are given in Figure 4.  Lines 2, 3, and 4 are as described for Figure 2. Line 5 is the minimum radiation angle used to determine the MUF.

The body of the figure is composed of four lines for each hour, consisting of MUF information for each layer.  Column 1 is universal time.  Column 2 is local time at the transmitter.  The next seven columns are for the E layer on line one, the F1 layer on line 2, the F2 layer on line 3, and the Es layer on the following line.  Column 3 is the 10 percent value of the MUF; i.e., the FOT, MHz.  The MUF during a month is expected to exceed the FOT 90 percent of the days, and to be less than the FOT 10 percent of the days.  Column 4 is the 50 percent value of the MUF in megahertz.  Column 5 is the 10 percent value of the MUF; i.e., the HPF in megahertz.  Column 6 is the radiation angle in degrees.  Column 7 is the virtual height of reflection in kilometers.  Column 8 is the true height of reflection in kilometers.  Column 9 is the equivalent vertical frequency in megahertz.   Note that, if the Fl layer is missing, the E values are used.  Figures 5, 6, 7, and 8 are diurnal plots and tables of some of these parameters that have proved useful to analysis in the past.

4.3  System Performance Output Options, METHOD = 16 through 24

The prediction of the performance of a communication system operating between two points on the earth's surface is the main output of the ICEPAC program.  There are two principle forms of output:  first, a table of up to 22 performance variables, and, second, a table or graph of the lowest usable high frequency (LUF). The program has been written in such a way that the user may specify the system performance lines desired.  The header identification lines can also be selected. Table 8 shows the eight toplines available; Table 9 shows the 22 system performance lines available; Table 10 shows the line types specified by using the METHOD command.  Table 7 shows the diurnal graphs that may be combined with the line selections.

Figures 9, 10, and 11 show a typical run with system performance output (method 16 with precalculation of antenna patterns).  Figures 9 and 10 are tables of antenna gain in decibels at frequencies from 2 to 30 MHz and radiation angles from 0 to 90 degrees.  These patterns and the input for each antenna type are described in detail in Section 4.4.

In METHOD 16, the header line information is largely input data supplied by the user.

Line 0:	This appears on each output page and includes the method, program version, and page number.

User selected header lines are:

Line 1:	Month, day, year, and sunspot number.

Line 2:	Label as supplied by user, and headings for next line.

Line 3:	Transmitter location, receiver location, the azimuth of the transmitter to the receiver in degrees east of north; the azimuth of the receiver to the transmitter in degrees east of north; path distance in nautical miles and kilometers.

Line 4:	Antenna subroutine used (defining the antenna module used in ICEPAC) and the minimum radiation angle in degrees.

Line 5:	Transmitter antenna line; up to three may be printed. Information varies with antenna type (see Section 3.4).  Typically, this is frequency range, antenna type, height of antenna above ground in meters (- is wavelengths), length of active part, an angle associated with the antenna structure in degrees, and the off main beam azimuth in degrees.  Note, lengths specified should be "electrical" lengths.

Line 5A:	Receiver antenna line.  Up to three may be printed.  The data are specified similar to the transmitter antenna line.

Line 6:	System line which has:  transmitter power in kilowatts, man-made noise level at 3 MHz in dBW, required reliability and required signal-to-noise ratio in decibels.

Line 7:	Multi-path line which has required power tolerance in decibels and required delay time tolerance in milliseconds.

The system performance lines are repeated each desired hour until the page is filled.  The process is repeated if additional pages are necessary.  The columns are time (universal) and up to 12 frequencies in megahertz; the first is the MUF and the remainder the frequency complement.  Figures 11 to 18 give the various system performance output defined by methods 16 to 23 as indicated in Table 3. All possible output lines for system performance are as follows and shown in Figure 15.

Line 0,FREQ:  Time and frequency line as associated with each column (default line always printed).The first four lines always refer to the most reliable mode (MRM).  The system performance parameter usually comes from the sum of all six modes.  If no decision can be made, the selection of MRM is based on number of hops and then on SNR.  All distribution refers to a diurnal period of an hour and a yearly period of a month.

Line 1, MODE:  For the short path model, number of hop and mode type.  E is E layer; Fl is Fl layer; F2 is F2 layer; ES is Es layer; N is a one hop Es with n Fl or F2 hops - (MRM).  For the long path model (Figure 16), the mode at the transmitter end and the mode at the receiver end are given.  The user can force the long path model by specifying method 21 regardless of the actual path length.

Line 2, ANGLE:  The radiation angle in degrees; for long paths (Figure 16), two lines are given:  the first for the transmitter end, and the second for the receiver end.  (MRM)

Line 3, DELAY:  Time delay in milliseconds.  (MRM)

Line 4, V HITE:  virtual height in kilometers.  (MRM)

Line 5, F DAYS:  The probability that the operating frequency will exceed the predicted MUF.

Line 6, LOSS:  Median system loss in decibels for the most reliable mode.

Line 7, DBU:  Median field strength expected at the receiver location in decibels above 1 microvolt per meter.

Line 8, S DBW:  Median signal power expected at the receiver input terminals in decibels above a watt.

Line 9, N DBW:  Median noise power expected at the receiver in decibels above 	a watt.

Line 10, SNR:  Median signal-to-noise-ratio in decibels.

Line 11, RPWRG:  Required combination of transmitter power and antenna gains 	needed to achieve the required reliability in decibels.

Line 12, REL:  Reliability.  The probability that the SNR exceeds the required SNR.  Note this applies to all days of the month and includes the effect of all mode types:  E, (Fl), F2, Es, and over-the-MUF modes.

Line 13, MPROB:  The probability of an additional mode within the multi-path tolerances (short paths only).

Line 14, S PRB:  Service probability.  The probability that the required reliability will be met.

Line 15, SIG LW:  Lower decile signal power (field strength and loss) increment in decibels.

Line 16, SIG UP:  Upper decile signal power (field strength and loss) increment in decibels.

A composite table of reliabilities only is given in Figure 19 which comes from the user-selecting method 24.  A similar table for any one variable can be selected by running method 23 and selecting the desired line.  However, a frequency line will be printed each hour.

If information for each mode is required, method 25 will produce the data in Figures 34 and 35.  For each hour, the ionospheric data, as in method 1, is printed.  For each frequency, the mode and signal parameters are printed for each mode up to six.  The last column is the selected most reliable mode.  The same information is provided for the MUF.

Methods 26, 27, 28, and 29 will calculate the LUF, the lowest frequency having a specified circuit reliability, usually 0.90.  Figure 36 is a table of LUF values with the MUF distribution and the median Es layer MUF.  Figures 37, 38, and 39 are available diurnal plots of MUF prediction along with the LUF.  Figures 40 and 41 provide MUF and system performance predictions for a long path (greater than 10000 km).

4.4  Antenna Output Options, METHOD = 13, 14, or 15

The detailed description of each antenna pattern and the required input definitions are given in this section.  The gain subroutines used in the ICEPAC are approximate models using the "one-term" theory and assume that the antenna parameters are within the design limits of each antenna and that the operating frequency is such that the antenna is close to resonance.  These antenna models are appropriate when used in a propagation model where other uncertainties overshadow the uncertainties in the gain due to the antenna mode limitations. However, it is not appropriate to use these models to design antennas or to evaluate their performance outside of their design limits or far from resonance. Also, they will not typically produce accurate results for full-wavelength or multiple-full-wavelength antennas--again due to the limitations of one-term theory.  The sub-module described here also assumes the main beams are pointed along the circuit path.  The input command lines in Figure 28 were used to generate the patterns of this section. Because of the 80 column limits on the outputs, the antenna patterns are divided and placed on two consecutive pages.  Frequencies 2 Mhz through 11 Mhz on the first page and frequencies 12 Mhz through 30 Mhz on the second page.  Figures 29 to 51 are pairs for each pattern, the first giving the structure of the antenna and the second giving a sample pattern.

5.   APPLICATIONS

The primary application of the ICEPAC program is to use the system performance options to select a frequency complement.  While intended mainly for program test and evaluation procedures, the other output options, if used with some interpretation, may provide the analyst with enough information to solve a particular problem.  Some possible applications will be discussed in the same order as the program complexity; i.e., as

(1)  ionospheric descriptions,

(2)  MUF predictions,

(3)  system performance predictions, and

(4)  antenna applications.

5.1  Ionospheric Parameters Applications

The ionospheric description output consists of a table of parameters (Figure 2) and a graph and table of ionograms (Figure 3).  The parameters are the output of the long-term world maps of the ionospheric parameters.  The accuracy of the maps were determined when they were generated.  The ICEPAC theoretical report has details and references to these maps.  The ionogram output includes the vertical sounding frequency and the virtual height, so that this ionogram may be compared to measured ionograms.  Note, however, a monthly median may not be characteristic of that for a given day.  The effect of the changes of critical frequencies from the maps may be studied using the FPROB command (Command 18) or by use of the EFVAR and ESVAR commands (Commands 11 and 12).

5.2  MUF Applications

In the absence of any other criteria for the planning of a system using HF sky-wave, the first, and simplest, criteria is an estimate of the frequency having efficient ionospheric reflections.  Normally, an estimate of the frequencies expected to have efficient ionospheric support 90 percent of the time, FOT, and those having efficient ionospheric support 50 percent of the time, MUF, are adequate estimates of upper frequency limits for system planning.  Figures 4 through 8 show the possible MUF outputs.  Note that Figure 4 (METHOD=7) gives all mode information and MUF distribution for each layer.  These MUF calculations are only a description of the state of the ionosphere and do not include any system parameters.  They should not be confused with the maximum operation frequency (MOF) for transmission between two points on an existing circuit.  A full system performance calculation should be made to estimate the MOF or to compare with observed MOFs.  If a full system performance computation is generated, the user should examine the frequency complement predictions rather than the MUF.

  - System Performance Applications

5.3.1   Selecting an Optimum Frequency

The complexities of propagation, the diversity of service requirements, and the fluctuation of spectrum congestion preclude any clear simple criteria for the selection of optimum frequencies.  An adequate signal-to-noise ratio at the receiver for the specified type and quality of service is often a useful criterion.  In general, within the HF spectrum, radio noise tends to decrease as frequency is increased.  During the daylight hours when HF power requirements are highest, the propagation loss tends to decrease as frequency is increased.  Since the noise normally decreases and signal normally increases with frequency, it is a general rule for HF sky-wave circuits that the higher the frequency the better the signal-to-noise ratio until frequency is increased to a point where reflection from the ionosphere becomes improbable.  A first approximation to the optimum frequency in the absence of interference may, therefore, be made by estimating the highest frequency having an efficient ionospheric reflection consistent with the circuit reliability required; i.e., the MUF calculations described in Section 6.2 above.

Since there is normally limited flexibility in the selection of frequencies and since the optimum frequency based on the probable upper useful frequency limit has mixed diurnal, seasonal, and other variations, it is desirable to establish the probable useful range of frequencies.  The FOT as shown is based on a 90 percent probability of efficient ionospheric support and may be used as an estimate of the probable upper frequency limit; a corresponding lower useful frequency limit may be estimated by considering the probability that the available signal-to-noise ratio will be adequate.  Since noise normally increases as frequency decreases and signal normally decreases as frequency decreases, there is usually a frequency below which the probability of an adequate signal-to-noise ratio is unacceptable.  This probability is often set at 90 percent and the corresponding frequency is known as the lowest useful frequency (LUF).  These limits; i.e., FOT and LUF, are shown in Figure 22 and graphically displayed in Figure 23.

5.3.2   Selecting a Frequency Complement for a Single Circuit in the Absence of Other Circuit Interference

The range of useful frequencies, such as shown in Figures 22 and 23, is basic to the selection of frequency complements and should be obtained for representative months over the time period the circuit under consideration will be required to operate.  For a semi-permanent operation, diurnal variation of the useful frequency range for seasonal extremes (e.g., June and December) and solar activity extremes (e.g., sunspot number 10 and 110) are normally adequate.

5.3.3   Standard Frequency Complement

Absolute continuity of any radio service, however desirable, is improbable even with an unlimited choice of operation frequencies, when high frequency sky-wave propagation must be relied upon.  Moreover, the return in improved continuity for an enlarged frequency complement beyond a certain size, depending upon the service, diminishes so rapidly that it can rarely be justified in the congested spectrum.  Frequency complements can, however, be based on a concept of maximum feasible continuity; i.e., the theoretical increase in circuit continuity may be negligible if additional frequencies are added, but a significant decrease is possible if fewer frequencies are available.  Since the required frequency complement depends upon the circuit parameters or usage, the following classification of circuits is introduced:

a.  Circuits requiring maximum feasible continuity.  These are the usually heavily loaded telegraph and telephone circuits, which must be available with good traffic capacity at all times.  Because of their loading, they employ relatively elaborate terminal equipment.  Telegraph circuits of this category are operated at machine speeds, while the telephone circuits generally employ several channels of a single-side-band system and are extended to line networks.  Such circuits are characterized by the use of large directional antenna systems, diversity reception in telegraphy. and high-powered transmitters.  Some other circuits, notably those immediately concerned with safety of life, may have an urgent need for continuous availability, although not necessarily carrying continuous traffic.  The standard frequency complement for these circuits provides at least one frequency between the LUF and FOT at all times, plus one or two additional frequencies to permit flexible operations in the event of interference and during ionospheric disturbances.  The maximum complement for these circuits should rarely exceed four.  If more than four frequencies are considered necessary, re-engineering of the circuit should be investigated.

- Circuits requiring moderate continuity.  Distinct from circuits requiring maximum continuity, there exists a larger group of circuits which, by nature of their operation, require only moderate continuity.  These circuits generally provide communication under circumstances where the needs are insufficiently critical to warrant the extension of wire, cable, or VHF facilities.  Many such circuits are operated to provide occasional service to remote installations.  There are also many circuits which may be designated nominally as continuous in operation, but on which the nature of the traffic is such as to allow occasional delays, reduction in transmitting speed, or rerouting.  Judicious scheduling of traffic contributes significantly to the satisfactory operation of these circuits.  Most circuits of an administrative nature belong in this category.  For frequency-complement considerations, circuits (except safety services mentioned earlier) should generally be constrained to this category if they employ manual telegraphy or telephony not extended to line networks, or are equipped with simpler transmitting and receiving installations than are capable of providing maximum feasible continuity.  Radio circuits of this kind have been operating on one or two frequencies in many parts of the world for many years, providing a quality of service consistent with particular needs.  The standard frequency complement for these circuits is two, one day frequency and one night frequency.

5.3.4   Two-Frequency Complements

Two-frequency complements are recommended with the clear understanding that the services receiving such complements are of the kind which do not have sufficient traffic to justify attempts to operate them on a twenty-four hour basis. These circuits must nevertheless be assured of as many hours of communication per day as possible within the two-frequency limitation.  Actually, in a very large number of cases during intermediate conditions of solar activity, two-frequency complements will give very nearly twenty-four hour service, except in the auroral regions where experience has shown that no complement of high frequencies can ever give a highly reliable service.

a.   General considerations.  There is available, at the present state of our knowledge, enough information on propagation, if the other factors such as terminal equipment, type of service, and actual operating conditions are known, to permit useful estimates of the total number of hours of satisfactory service, assuming no interference, to be expected from a given frequency over an average solar cycle.  It is, however, impossible to estimate for any particular circuit the frequency that would give the maximum number of hours of useful service over the solar cycle.  However, this frequency might not necessarily be one of the two frequencies assigned, since the cumulative number of hours of usefulness is a less urgent consideration than that of assuring operation during the normal peak traffic periods.

b.   The day frequency.  A simple method for selecting the day frequency consists of choosing.a frequency just below the lowest daytime FOT curve.  In the north temperate zone, this would be the FOT curve for June for the period of mini- mum solar activity.  Simply choosing one frequency, or some small range of frequencies on this basis, ignores the situation with regard to LUF, and could lead, for an actual circuit, to failure to provide communication during the middle of the day in months of maximum absorption at the period of maximum solar activity. An improved method is derived from the opinion that, if possible, the day frequency should not fail to give service during the midday hours even at the condition of maximum normal absorption which occurs at the maximum levels of solar activity.

This approach suggests immediately that a band of particularly useful frequencies exists between the frequency which is just above the maximum midday LUF and the highest frequency that will provide essentially skip-free service during the middle period of the daylight hours in the months of minimum daytime FOT.  The lowest frequency in such a band will provide the maximum hours of service, and such service may well exceed that obtained from a frequency selected only on the basis of minimum daytime FOTs.  Fortunately, a useful band of frequencies does exist in nearly all cases, and the day frequency should be chosen in the lower part of this band.

The possibility of skip on frequencies chosen in this way is unlikely when consideration is given not only to normal FOTs, but also to the well-established role of sporadic-E reflections.

c.  The night frequency.  The choice of the night frequency is complicated, depending not only on the type of service, terminal equipment, and a number of propagational considerations, but also on the day frequency selected.  As in the case of the selection of the best day frequency, it is not sufficient to select a frequency that will give the maximum number of hours of service throughout a complete solar cycle.  The solution is to make the best choice for those hours that the day frequency is not suitable.

At night ionospheric absorption is minimal, except in the high latitudes, and the LUFs are rather critically dependent on the type of service and equipment used, including the antennas and the required signal levels.  Too high a night frequency will be subject to skip too much of the time.  However, a very low and, therefore, virtually skip-free night frequency may make it impossible to deliver adequate signal-to-noise ratios to the receiving location for substantial periods of time.  The selection problem for this single night frequency is one of deciding what constitutes a suitable balance of these conflicting considerations.  For circuits of 4000 km or less in length, the difference in local time between the terminals can never be large in the regions of the world where the bulk of the HF circuits operate.  It is reasonable that traffic handling during the predawn hours be avoided, since, apart from the usual erratic behavior of the ionosphere during these hours. normal activities at both terminals are at their diurnal minimum during this period.  In view of this sort of consideration, it would seem incorrect to choose a night frequency so low that skip-free operation was safely assured during the predawn period of the months and solar activity conditions having the lowest FOTs at the expense of rendering communications impossible during other time periods because of high noise levels normally experienced at the lower frequencies.

It is recommended, therefore, that the night frequency be chosen as the highest frequency of which less than four hours of skip is indicated on the lowest of the FOT curves for the required months of operation.  This is usually during the winter months; e.g., December in the northern hemisphere and June in the southern hemisphere.

5.3.5   Three-Frequency Complement

Many radio circuits require a greater continuity of service than can generally be assured with two frequencies.  This is especially true of telephone circuits intended for extension to line networks.  The following suggestions are applicable to determining frequency complements for circuits less than 4000 km in length requiring maximum feasible continuity and employing telephony for extension to line networks, and also for speed of telegraphy for services not seriously troubled by multi-path effects.  Multi-path protection required for certain types of high-speed machine telegraphy and facsimile services is not necessarily provided by these three-frequency complement standards.

There is some justification for enlarging the complement to four frequencies in the case of telephony and manual telegraph services subject to the severe magnetic disturbances characteristic of the high latitude regions.  In other regions, however, it is possible by a suitable choice of three frequencies to maintain the signal-to-noise ratio high enough to provide an entirely adequate service.  This is possible because higher frequencies are generally usable in the lower latitudes as a consequence of the observed higher complement-limiting FOTs, daytime levels of which survive in many cases far into the night.  This situation is in marked contrast with the high latitudes where circuits are in all cases significantly more difficult to operate.

a.  The high frequency.  The highest frequency of a three-frequency complement is purely a day frequency.  Since a middle frequency is available, it is no longer necessary for the highest frequency of the complement to be constrained to give useful service at the minimum phases of the solar cycle.  This frequency should give many hours per day of useful service at all seasons during the maximum phases of the solar cycle.  The high frequency is the highest frequency which will be below the FOT at least four hours during all months during the period the circuit is to be operated.

b.  The low frequency.  The lowest frequency of a three-frequency complement is entirely a night frequency.  It must be selected on a basis related to the minimum FOT during any month the circuit is required to operate.  Select the frequency which indicates less than two hours of skip on the lowest of the FOT curves for the required months of circuit operation.

c.  The middle frequency.  The middle frequency is selected so as to maximize the number of hours during which at least one frequency is between the LUF and FOT during the required period of circuit operation.

d.  A special case for modifying the low frequency.  Certain special considerations may affect the choice of a low frequency for circuits between about 3600 and 4000 km in length.  For such circuits, the vertical angles of departure and arrival for single-reflection F2-layer transmission at night become very low.  With many antennas, very little energy can be transmitted or received at these low angles.  For these antennas, transmission, when possible, by two reflections from the F2 layer provides a superior service.  If highly-directional broadside arrays, which are relatively efficient at low angles, are used at sites permitting use of low angles of departure and arrival, the single-reflection transmission remains useful.  The possibility of site conditions which make impossible the use of low angles of arrival or departure should not be over- looked.  During the conditions of lowest MUFs, unobstructed sites, and efficient low-angle antenna systems, the low frequencies for paths 3000-5000 km will remain useful, but there will usually be a skip on the higher vertical-angle two-reflection mode.  With low-angle radiation or reception limited by inadequate antenna systems, terrain, etc., in the ways suggested, this skipping in the two- reflection mode may well interrupt the service.  In these instances, where it appears that reliance must be placed on two-reflection transmission during the   period of minimum MUFs, it is preferable to assign a frequency appropriate to two- reflection transmission.  This frequency will be automatically displayed in Figure 4 or on graphs displayed in Figures 5 to 8 if the minimum vertical angle is set at the lower limit of adequate antenna performance; e.g., 3 to 6 degrees.

5.3.6   Four-Frequency Complements

Standard three-frequency complements are confined to a number of services, operated over paths less than 4000 km in length. where maximum feasible continuity of operation is required, but when the effects of multi-path propagation are not serious.  Services which use high-speed digital transmission techniques are seriously affected by multi-path distortion, and a three-frequency complement may be insufficient.  Many such services can, however, be adequately satisfied by a three-frequency complement with respect to both continuity and multi-path protection.  This is particularly true for circuits in the 2000 to 4000 km range of lengths, where the probability of multi-path is low, and to a lesser extent for shorter circuits.  Whether or not the three-frequency complement for a particular circuit provides the requisite multi-path protection for high-speed service may be determined by a system performance prediction for the frequencies selected (e.g., METHOD=23, Figure 18 using a multi-path tolerance of two milliseconds and a power tolerance of 10 db) and noting the multi-path probability for frequencies having an acceptable reliability.

It was suggested in the discussion on the applicability of three-frequency complements that there is some justification, apart from multi-path considerations, for the assignment of a four-frequency complement to a circuit operating in high latitudes which has a critical need for maximum feasible continuity.  If one or both terminals of a circuit lie above 60 degrees north geomagnetic latitude, the circuit may be regarded as sufficiently high latitude to merit consideration for a four-frequency complement.

a.  The highest frequency.  The highest frequency of a four-frequency complement is purely a day and evening frequency.  Since three frequencies are available below it in a given complement, there is no longer any need that it give any important service at the minimum phases of the solar cycle, nor need it be the only frequency of the complement for service during maximum LUF periods.  The main purpose of this frequency is to permit the reception of high-speed digital information free of destructive multi-path distortion.  It is usually required in afternoon and evening periods when the second highest frequency--while giving a perfectly adequate signal-to-noise ratio--would be subject to multi-path distortion.  This frequency must, therefore, be chosen with the maximum FOT in mind; it should nevertheless be as low as possible to give as much service as possible.  In some of the complements, the highest frequency will give few cumulative hours of service over a solar cycle, though remaining indispensable to avoid multi-path.  In other complements, it will give considerable cumulative service and prove to be particularly useful in obtaining a good signal-to-noise ratio.

In general, the highest frequency is chosen to exceed 65 percent of the maximum FOT during the required period of circuit operation.  This procedure is intended to provide substantial protection against multi-path distortion on the highest frequency of the complement during hours and seasons of occurrence of maximum MUFs.  At shorter distances, because of the impracticability of providing complete multi-path protection, the factor provides as much protection as can reasonably be afforded by a four-frequency complement, while at the same time providing a highest frequency that will have significant usefulness at the maximum phases of the solar cycle.

It is necessary to invoke a LUF-determined lower limit or floor value below which the highest frequency of the complement is not selected regardless of the results of the above procedure.  This limit is intended to provide a minimum additional margin of signal-to-noise ratio over that provided by the second highest frequency for high-grade service during periods of maximum LUF.  The lower limit is set at 1.4 times the maximum LUF during the required period of circuit operation.

b.  The second-highest frequency.  The second-highest frequency of a four-frequency complement is the most important frequency of the complement in many ways.  It is certainly the frequency likely to receive the greatest cumulative use over a solar cycle.  It is low enough to provide reliable daytime operation at sunspot minimum during periods of minimum daytime MUF.  While the frequency may receive considerable daytime use at sunspot maximum, it will also be needed during the evening and night transition periods at sunspot maximum to permit continuation of high-speed digital operation.  Since there exists a frequency still higher in the complement and two lower, this frequency is chosen to remain just above the maximum LUF, as was the daytime frequency of the two-frequency complements.  On some circuits, it can be expected that this frequency will provide service far into the night at sunspot maximum during much of the spring, summer, and autumn seasons.  The principal use of this frequency is, nevertheless, as a day or evening frequency.

c.  The third-highest frequency.  The third-highest frequency of a four- frequency complement is probably the second most important frequency of the complement in terms of cumulative hours of use over a solar cycle.  While this frequency may receive some use in the early morning under certain conditions, its main use is as an evening or night frequency.  During sunspot maximum conditions, it will, in a large number of cases, cover the late night period even in winter; it will certainly be sufficiently low for summer night use, even down to sunspot minimum in many temperate regions.  In the high-noise regions, this frequency will nearly always be sufficiently low to cover the predawn period at the noisiest seasons, and for this reason it has not been necessary to give special consideration to high-noise-region floors for night frequencies in this report.

This frequency is the geometric mean of the second-highest and the lowest frequency of the complement.  This procedure results in frequency intervals which provide the maximum possible multi-path protection between these complement members.  It provides, at the same time, a good order of frequencies for intended major usage during evening and night periods.

- The lowest frequency.  The lowest frequency of a four-frequency complement is entirely a night frequency.  It must be relied upon at all times when the third-highest frequency is too high to carry the service.  It is selected just below the minimum FOT during the period the circuit is required to operate.

5.3.7   Time Sharing on Circuits Separated Geographically

Since there are marked diurnal and geographical variations in the useful frequency range, it is often possible to use this variation to develop time sharing plans when circuits are separated geographically.

Tabulations or graphs of useful frequency range (Figures 21 and 22) should be obtained for the time period or time periods of interest.  Whenever a frequency complement is such that it contains a frequency within the useful frequency range for one circuit, while outside the useful frequency range for the other circuit, this information may be used to develop time-sharing schedules.  The time period may be extended whenever other frequencies in the complement are useful on the second circuit.

5.3.8   Time Sharing in the Same Geographic Area

When long and short paths are involved in the same geographic area, the useful frequency ranges for each may differ sufficiently that sharing plans may be developed in a manner similar to that described above (see Section 5.3.7).

5.3.9   Frequency Sharing

The development of frequency-sharing plans requires the prediction of the available signal along the unwanted as well as the wanted radio path, taking particular account of the expected antenna performance for the unwanted radio path. The circuit reliability estimates (e.g., METHOD=23) can be used to develop frequency-sharing plans (i.e., share whenever the reliability is high for the wanted paths but low for the unwanted paths).

A quick determination of a frequency sharing opportunity may sometimes be made by MUF-FOT computations for the wanted paths (e.g., Figure 5) and the HPF (frequency having efficient ionospheric support only 10 percent of the days) computation for the unwanted paths (e.g., Figure 6).  Sharing should be possible if a frequency is below the FOT on the wanted path but above the HPF on the unwanted path.

5.3.10   Broadcast Coverage

Circuit reliability is probably the most valuable single output from the prediction program.  As various parameters are fixed, a computation of circuit reliability as a function of a remaining variable will often assist in decision making.  The question of broadcast coverage is a good example.  With time, frequency, antenna, transmitter location, transmitter power, etc. fixed, circuit reliability to sample points within a geographic area of interest will describe the coverage of the area in terms of the percentage of days within the month that satisfactory reception may be expected.  This process may be performed automatically by the computer to provide reliability tabulations over selected areas (e.g., one hemisphere).  Although this output is not available as a "standard output", tabulations of this type can be made using a special input command line processor and output variable formatter.

5.3.11   Optimum Times for Communication

For given transmitter location, receiver location, antenna types, etc., the diurnal variation in circuit reliability may be used to choose optimum communication time (e.g., METHOD=23).

5.3.12   Selection of Relay Locations

Careful consideration should be given to increasing the frequency complement, increasing power, antenna redesign, etc. before relay stations are used.  If there is no other solution, consider the use of relay stations.  Normally, if possible, these relay stations should be separated by at least 3000 km and preferably not more than 7000 km.  The relays should also assure the propagation path does not go to high latitudes; i.e., temperate or equivalent routes avoiding high noise regions are preferred.  In the final selection, it is a question of computing circuit reliability for the direct path and making a comparison with potential relay sites.

5.3.13   Determination of Lowest Effective Transmitter Power

Compute circuit reliability as a function of transmitter power with other variables fixed and plot reliability versus transmitter power.  The lowest effective power is the lowest power providing the required reliability.  An alternative is to use the required power plus antenna gain output, RPWRG, line 11, using METHOD=23.

5.4  Antenna Selection or Design

For the frequency or frequencies under consideration, make predictions covering the required time period of operation using a constant gain antenna with a typical gain; e.g., 12 db.  Determine the time or times the circuit reliability is the lowest.  Using these times, repeat the computation for the antennas under consideration to select an antenna.  Caution:  the calculated vertical angle for the most reliable mode provides some guidance in determining the antennas to be considered, but this angle alone should never be used as the sole criterion for antenna selection.  To select an antenna, repeat the computations for available antennas.  To design an antenna, repeat computations for variables in the antenna design; e.g., antenna height, rhombic leg length, etc.  Note, however, that these parameters must be part of a "well designed" antenna.  See the comments in Section 4.4 above.

- PROGRAM DATA FILES

6.1  Program and Data Base Files

The ICEPAC program and data base is provided on 360kb diskettes or on DC-600A data tape cartridges for UNIX based systems.  The programs are executable modules and the data bases are binary files.

6.2  Long-Term Data Base File

This section describes the long-term data file used with the ICEPAC program. Details of how each of the various parameters is used is given in the ICEPAC theoretical report.  The data files used here has evolved over many years.  The basic criteria used in retention or inclusion of any of the data sets are:  availability on a worldwide basis over all the time cycles (diurnal, yearly, and solar cycle) availability of distributions of the data, and consistency between the data sets.  There are 12 binary data files accessed from the hard disk, one for each month of the year.  The names are as follows:

BCOEFO1.DAT = January data base

BCOEFO2.DAT = February data base

.

.

BCOEF12.DAT = December data base

Logical Record 1 - F1 layer and Land mass data base

This record contains the following arrays:

ANEW(3),BNEW(3),ACHI(2),BCHI(2),FAKMAP(29,16),

ABMAP(2,3)

Logical Record 2 - Noise and Statistical data base

This record contains the following arrays:

F2D(16,6,6),DUD(5,12,5),FAM(14,12),SYS(9,16,6),

PERR(9,4,6)

Logical Record 3 - Noise Data

This record contains the following array:

p(29,16,6) ABP(2,6)

Logical Record 4 - FOF2 data

This record contains the following arrays:

IKIM(10,6),XF2 COF(13,76,2)

Logical Record 5 - Median sporadic E (FoEs) and height ratio data

This record contains the following arrays:

XESMCF(7,61,2), XPMAP(29,16,2)

Logical Record 6 - Lower and upper decile of sporadic FoEs

This record contains the following arrays:

XESLCF(5,55,2), XESUCF(5,55,2)

Logical Record 7 - M(3000)F2

This record contains the following array:

XFM3CF(9,49,2)

Logical Record 8 - Regular E layer (FoE)

This record contains the following array:

XERCOF(9,22,2)

6.3  Antenna Patterns Stored on File

It is possible, but not mandatory, to execute the ICEPAC program using antenna patterns that have been previously computed by (a) ICEPAC using the ANTOUT command line discussed in Section 3.3.6, (b) some other analysis program that has been modified to create and write antenna patterns in the format expected by ICEPAC or (c) to use field measurements and interpolation to create and write antenna patterns in the format expected by ICEPAC.  This use may require some in-depth knowledge of the working of ICEPAC.

The antenna pattern output file (defined by the ANTOUT command) is stored on disk as an external binary file.  The user should assign the local file name on the ANTOUT command line prior to the execution of the ICEPAC program.  The user would then specify antenna type 18 on the ANTENNA command line to indicate that the antenna pattern is to be input from the antenna file.  The location of the desired antenna pattern on the antenna file is specified using variable IAIN for antenna type 18.  (Note:  This variable indicates from one to three antennas to cover the frequency range specified for the transmitter and receiver antennas for all antenna types except antenna type 18.)

It is necessary to specify the location of the desired antenna pattern on the antenna file since several antenna patterns could be written on the same antenna file.  Locating the desired antenna pattern depends on whether the antenna pattern is beyond or before the current location on the antenna file.  If the desired antenna pattern is beyond the current position on the antenna file, the variable IAIN indicates the number of antenna patterns to skip forward from the present position on the antenna file before reading the desired pattern.  Thus, in this case, IAIN must be set to a positive number or zero by the user.  For example, IAIN = 0 indicates that no antenna patterns are to be skipped from the current position, which means the next antenna pattern on the file is read.  If IAIN = 3, then three antenna patterns are to be skipped from the current position before the desired antenna pattern is read.  If the desired antenna pattern is before the current location on the antenna file, the variable IAIN indicates that the antenna pattern should be rewound before searching for the desired pattern.  This is accomplished by setting IAIN negative.  The negative number causes the file to be rewound, and the pattern read is specified by the absolute value of IAIN.  For example, IAIN = -1 indicates that the antenna file is rewound before reading the desired antenna pattern and that the first pattern on the file is the antenna pattern read.  If IAIN = -3, then the antenna file is rewound before reading the desired antenna pattern and the third pattern on the file is the antenna pattern read.

7.   SPECIFIC INPUT EXAMPLES

This section is intended to demonstrate specific input options of the IONCAP program to reinforce information provided in earlier sections. Specific input requirements necessary to demonstrate particular IONCAP capabilities are discussed.  References to corresponding output for each example is also presented.

7.1  Ionospheric Parameter Example

The input required to generate the ionospheric parameters for the control points of a particular circuit consists of the following control:  METHOD, MONTH, SUNSPOT, TIME, LABEL, CIRCUIT, EXECUTE and QUIT.  The input necessary to generate the similar output represented in Figure 2 consists of the following:

METHOD        1

MONTH      l970    1

SUNSPOT    100.   0.

TIME          1   24    1   -1

LABEL     BOULDER, COLORADO TO ST. LOUIS, MO.

CIRCUIT   40.03N    105.3W    38.67N    9O.25W

EXECUTE

QUIT

It should be noted that the sporadic-E critical frequency has been multiplied by .7 to allow for median losses in this example by program default.  Figure 2, discussed earlier, is a presentation of the ionospheric parameters for this circuit without the sporadic-E critical frequency reduced.

7.2  Antenna Pattern Example

The input required to generate an antenna pattern for both a transmitter and receiver antenna consists of the following command lines:  METHOD, ANTENNA, EXECUTE, and QUIT.  The specific input necessary to generate the output represented in Figures 9 and 10 consists of the following:

METHOD       15

ANTENNA       1    2      .001   4.       -.5

ANTENNA       2    2      .001   4.      -.25

EXECUTE

QUIT

To generate the pattern of a transmitter antenna, only use method 13 and for a receiver antenna, only use method 14.

7.3  MUF Example

The input required to generate MUF predictions consists of the following command lines:  METHOD, MONTH, SUNSPOT, TIME, LABEL, CIRCUIT, EXECUTE, and QUIT. The specific input necessary to generate the output represented in Figure 4 consists of the  following:

METHOD        7

MONTH      1970    1

SUNSPOT    100.   0.

TIME          1   24    1   -1

LABEL     BOULDER, COLORADO TO St. LOUIS, MO.

CIRCUIT   40.03N    106.3W    38.67N    90.25W

EXECUTE

QUIT

7.4  System Performance Example

The input required to generate system performance predictions consists of the following command lines:  METHOD, MONTH, SUNSPOT, TIME, LABEL, CIRCUIT, ANTENNA, FREQUENCY, EXECUTE, and QUIT.  The input necessary to generate output similar to the output represented in Figure 11 consists of the following:

METHOD       16

MONTH      1970    1

SUNSPOT    100.   0.

TIME         12   24   12   -1

LABEL     BOULDER, COLORADO TO ST. LOUIS, MO.

CIRCUIT   40.03N    105.3W    38.67N    9O.25W

ANTENNA       1    2      .001   4.       -.5

ANTENNA       2    2      .001   4.      -.25

FREQUENCY   2.0  3.0  5.0  7.5 10.0 12.5 15.0 17.5 20.0 25.0 30.0

EXECUTE

QUIT

It should be noted that the above input does not cause the transmitter and receiver antenna patterns to be pre-computed as was the situation in Figure 11. (This is indicated by the absence of method 15 line to compute the gain pattern in this example.)  The above input causes the antenna gain values to be computed as needed for specific frequencies and elevation angles.  This indicates that the gain value for 7.6 MHz needed in Figure 11 was interpolated between 7 MHz and 8 MHz.  The gain value for 7.5 MHz in the above example is computed as the frequency 7.5 MHz is processed.  It should further be noted that this value would be re-computed for each hour specified in the above example.  Thus, the gain is computed at each frequency and for each hour as needed.

7.5  User-Selected Output Example

The input required to generate user-selected system performance predictions consists of the following command lines:  METHOD, TOPLINES, BOTLINES, MONTH, SUNSPOT, TIME, LABEL, CIRCUIT, ANTENNA, FREQUENCY, EXECUTE, and ED.  The input necessary to generate output similar to the output represented in Figure 18 consists of the following:

METHOD       23

TOPLINES      1    2    3    4    6    6    7

BOTLINES      1    2    4   10   11   12

MONTH      1970    1

SUNSPOT    100.   0.

TIME         12   24   12   -1

LABEL     BOULDER, COLORADO TO ST. LOUIS, MO.

CIRCUIT   40.03N    106.3W    38.67N    9O.25W

ANTENNA       1    2      .001   4.       -.5

ANTENNA       2    2      .001   4.      -.25

FREQUENCY 2.0  3.0  5.0  7.5  10.0  12.5  15.0  		17.5  20.0  25.0  30.0

EXECUTE

QUIT

The header lines specified on the TOPLINES are given in Table 8 and are listed below:

(1)			month, day, year, and sunspot number

(2)			alphanumeric information on the LABEL command line

(3)			transmitter and receiver information (coordinates)

(4)			minimum takeoff angle

(5)			transmitter antenna information

(6)			receiver antenna information

(7)			transmitter power, man-made noise, required reliability, and required SNR.

The output predictions specified on the BOTLINES command are given in Table 9 and listed below:

(1)			number of hops and mode type

(2)			radiation angle and transmitter

(4)			virtual height of most reliable mode

(10)			median SNR

(11)			required power gain for most reliable mode

(12)			reliability.

It should be noted that the above input does not cause the antenna pattern to be pre-computed as was the case in Figure 18.  (This is indicated by the absence of the method 15 command to compute the gain tables in this example.)  The above input would compute the gain value of each frequency for each hour as needed.  This has been discussed in Section 7.4.

7.6  LUF Example

The input required to generate LUF predictions consists of the following command lines:  METHOD, MONTH, SUNSPOT, TIME, LABEL, CIRCUIT, ANTENNA, EXECUTE, and ED.  The input necessary to generate output similar to the output represented in Figure 22 consists of the following:

METHOD       26

MONTH      1970    1

SUNSPOT    100.   0.

TIME         2    24    2   -1

LABEL     BOULDER, COLORADO TO ST. LOUIS, MO.

CIRCUIT   40.03N    105.3W    38.67N    90.25W

ANTENNA       1    2      .001   4.       -.5

ANTENNA       2    2      .001   4.      -.26

EXECUTE

QUIT

It should be noted that the above input does not cause the antenna pattern to be pre-computed as was the situation in Figure 18.  (This is indicated above by the absence of the method 16 command to compute the gain tables in this example.)  The above input would cause the antenna gain to be computed for the LUF rather than interpolate in the gain table as was the case in Figure 18.

7.7  Outgraph Example

The input required to generate the MUF predictions given in Figures 4 through 8 consists of the following command lines:  METHOD, OUTGRAPH, MONTH, SUNSPOT, TIME, LABEL, CIRCUIT, ExECUTE, and ED.  The specific input necessary to generate the output represented in Figures 4 through 8 consists of the following:

METHOD        7

OUTGRAPH      8    9   10   11

MONTH      1970    1

SUNSPOT    100.   0.

TIME          1   24    1   -1

LABEL     BOULDER, COLORADO TO ST. LOUIS, MO.

CIRCUIT   40.03N    106.3W    38.67N    90.25W

EXECUTE

QUIT

The method command defines the predictions to be generated and the output form generated by that method.  Therefore, the predictions generated consist of the MUF-FOT table for each ionospheric layer since method 7 was specified in this example.  The user may select additional output providing that the computations for the output have already been generated.  Therefore, in this case, the user can select the outputs of methods 8 through 11 consisting of the MUF-FOT graph, HPF-MUF-FOT graph, MUF-FOT-ANG graph, and MUF-FOT-Es graph, as is indicated in Table 7.

7.8  External Antenna File Example

The user may wish to save the antenna gain table of his antennas for use on a separate run.  This may be particularly significant if the same antennas are used for several communication paths, or the predictions are generated with the same antennas each month, or the user has a specific antenna pattern created ha another program, or possibly the actual measured antenna gain values.  To create the antenna file corresponding to the patterns given in Figures 9 and 10, the following input should be used:

METHOD       15

ANTOUT          ANTOUT.FIL

ANTENNA       1    2      .001   4.       -.5

ANTENNA       2    2      .001   4.      -.26

EXECUTE

QUIT

The user could then store the patterns generated on disk for reference at a later time.  See the discussion of the ANTOUT command in Sections 3.3.5 and 6.3.  Use of the antenna patterns previously generated and stored externally, can be done using the following input, which will also generate the system performance output represented in Figure 11.

METHOD       16

MONTH      1970    1

SUNSPOT    100.   0.

TIME         12   24   12   -1

LABEL     BOULDER, COLORADO TO ST. LOUIS, MO.

CIRCUIT   40.03N    106.3W    38.67N    9O.25W

ANTENNA       1   18  ANTOUT.FIL

ANTENNA       2   18  ANTOUT.FIL

FREQUENCY   2.0  3.0  5.0  7.6 10.0 12.5 15.0 17.5 20.0 26.0 30.0

EXECUTE

QUIT

The difference in this example and the example presented in Section 7.4 is that the antenna patterns are read from a file instead of being computed.  Section 6.3 explains this procedure in depth.

8.   ACKNOWLEDGMENTS

This report is the result of the efforts of many groups and individuals.  It is an update of the IONCAP users manual with the orientation toward the PC computer use.  The wording is sometimes a duplication of the older user manuals.

In regard to individuals, the following list may not include everyone, but hopefully the majority will be mentioned.  The author wishes to acknowledge the contributions of Mr. A.F. Barghausen, Dr. Mark T. Ma, Mr. D.H. Zacharisen, Dr. E.L. Crow, Mr. G.W. Haydon, Mr. D.L. Lucas, Mr. J.L. Lloyd, Mr. G.R. Hand, Dr. A.D. Spaulding, Dr. R.K. Rosich, Mr. L.R. Teeters, Mr. R.M. Davis Jr., and many others who have been instrumental in the evolution of the propagation programs and documentation.

Input File for ICEPAC Prediction Program   Version IC.10

1         2         3         4         5         6         7         8

12345678901234567890123456789012345678901234567890123456789012345678901234567890

COMMENT   **********************************************************************

COMMENT   SAMPLE INPUT TO RUN ICEPAC IC.10  - ALL METHODS

COMMENT   **********************************************************************

LABEL     BOULDER,COLORADO TO ST. LOUIS,MO.

CIRCUIT   40.03N    105.3W    38.67N    90.25W        0

SYSTEM     3000  150 .001   90 5500 1000   85

MONTH      1970    1

SUNSPOT    100.   0.

TIME          1   24    1   -1

FPROB     1.0  1.0  1.0  1.0

METHOD        1

EXECUTE

COMMENT   CHANGE CRITICAL FREQUENCY MULTIPLIER FOR ES BACK TO PROGRAM DEFAULT

FPROB     OFF

COMMENT   ES CRITICAL FREQUENCY NOW IS MULTIPLIED BY .7 TO ALLOW FOR MEDIAN LOSS

METHOD        2

TIME         12   12    1   -1

EXECUTE

COMMENT   **********************************************************************

COMMENT   METHODS 3 THROUGH 11 ARE MUF CALCULATIONS (METHOD 12 NOT IMPLEMENTED)

COMMENT   METHODS 3,4,5 AND 6 ARE MUF USING NOMOGRAM AND AREN"T PRESENTED HERE

TIME          1   24    1  -1

METHOD        7

COMMENT   OUTPUT METHODS 8 THROUGH 11 WITHOUT RECOMPUTATION USING "OUTGRAPH"

OUTGRAPH      8    9   10   11

EXECUTE

OUTGRAPH  OFF

COMMENT   **********************************************************************

COMMENT   METHODS 13 THROUGH 15 ARE ANTENNA PATTERN CALCULATIONS

COMMENT   METHODS 13 AND 14 ARE ANTENNAS ONE AT A TIME AND AREN"T PRESENTED HERE

COMMENT   ANTOUT        ANTFIL.BIN

METHOD       15

ANTENNA       1    2      .001 4.       -0.5

ANTENNA       2    2      .001 4.       -0.25

COMMENT   ANTENNA       1   18 ANTFIL.BIN

COMMENT   ANTENNA       2   18 ANTFIL.BIN

EXECUTE

COMMENT   ANTOUT    OFF

COMMENT   **********************************************************************

COMMENT   METHODS 16 THROUGH 23 ARE SYSTEM PERFORMANCE PREDICTIONS

FREQUENCY   2.0  3.0  5.0  7.5 10.0 12.5 15.0 17.5 20.0 25.0 30.0

TIME         12   24   12   -1

METHOD       16

EXECUTE

METHOD       17

EXECUTE

METHOD       18

EXECUTE

METHOD       19

EXECUTE

METHOD       20

EXECUTE

Figure 1.  Input data commands for all output options.

COMMENT   METHOD 21 FORCES THE PROGRAM TO EXERCISE THE "LONG" PATH MODEL

METHOD       21

EXECUTE

COMMENT   METHOD 22 FORCES THE PROGRAM TO EXERCISE THE "SHORT" PATH MODEL

METHOD       22

EXECUTE

COMMENT   METHOD 23 ALLOWS THE USER TO SELECT THE DESIRED OUTPUT BY SPECIFYING

COMMENT   PREDEFINED LINE NUMBERS ON THE "TOPLINES" AND "BOTLINES" COMMANDS

COMMENT   LINES ARE NUMBERED IN ORDER AS IN METHOD 20 (SEE TABLES 8 AND 9)

METHOD       23

TOPLINES      1    2    3    4    5    6    7

BOTLINES      1    2    4   10   11   12

EXECUTE

COMMENT   **********************************************************************

COMMENT   METHOD 24 IS THE MUF-RELIABILITY TABLE

COMMENT   **********************************************************************

METHOD       24

TIME          1   24    1   -1

EXECUTE

COMMENT   **********************************************************************

COMMENT   METHOD 25 IS THE ALL MODES TABLE

METHOD       25

COMMENT   NOTE THAT THE MUF ALL MODES TABLE IS ALSO PRINTED

FREQUENCY   3.0

TIME         12   12    1   -1

EXECUTE

COMMENT   **********************************************************************

COMMENT   METHODS 26 THROUGH 29 ARE LUF PREDICTIONS

METHOD       26

COMMENT   OUTPUT METHODS 27, 28 AND 29 WITHOUT RECOMPUTATION USING "OUTGRAPH"

OUTGRAPH     27   28   29

TIME          2   24    2  -1

EXECUTE

OUTGRAPH  OFF

COMMENT   **********************************************************************

COMMENT   INCLUDE A LONG PATH CIRCUIT EXAMPLE

LABEL     BOULDER,COLORADO TO AUCKLAND,N. Z.

CIRCUIT   40.03N    105.3W    36.92S    17475E

METHOD        7

TIME          6   18    6   -1

EXECUTE

FREQUENCY   2.0  3.0  5.0  7.5 10.0 12.5 15.0 17.5 20.0 25.0 30.0

METHOD       23

COMMENT   NOTE THAT THE PREVIOUSLY DEFINED "TOPLINES" AND "BOTLINES" STILL USED

EXECUTE

QUIT

Figure 1a.  Input data commands for all output options.

(Continued)

METHOD  1   ICEPAC IC.10   PAGE   1

JAN    1970          SSN = 100.

BOULDER,COLORADO TO ST. LOUIS,MO.         AZIMUTHS          N. MI.      KM

40.03 N  105.30 W - 38.67 N   90.25 W     91.84  281.42     702.6   1301.1

XMTR   2.0  TO  30.0  CONST. GAIN  H     .00 L     .00 A     .0  OFF AZ     .0

YE =  20.0     HE = 110.0     HS = 110.0

LAT   LONG    LMT    UT    E     F1    Y1     H1   FH/2 F2Z   Y2

H2     PB    CEN    EB M3000   TCGM   RAT    ZEN FLAG   MAGL

39.6N  97.7W   1.5   8.0   .58     .2  50.0  200.0   .7   4.4  68.7

317.4   72.7   71.4  70.0  2.97    1.4   4.6  153.4    0  50.1N

39.6N  97.7W   2.5   9.0   .58     .2  50.0  200.0   .7   4.5  70.6

322.5   73.2   71.6  70.0  2.94    2.4   4.6  143.2    0  50.1N

39.6N  97.7W   3.5  10.0   .61     .2  50.0  200.0   .7   4.1  73.5

328.5   73.7   71.9  70.0  2.89    3.4   4.5  131.9    0  50.1N

39.6N  97.7W   4.5  11.0   .74     .2  50.0  200.0   .7   3.5  75.8

326.1   74.0   72.1  70.3  2.87    4.4   4.3  120.3    0  50.1N

39.6N  97.7W   5.5  12.0  1.02     .2  50.0  200.0   .7   3.4  75.6

307.1   74.0   72.5  71.0  2.90    5.4   4.1  108.9    0  50.1N

39.6N  97.7W   6.5  13.0  1.44     .2  50.0  200.0   .7   4.2  76.8

289.1   74.0   72.9  71.7  3.00    6.3   3.8   97.9    6  50.1N

39.6N  97.7W   7.5  14.0  1.95     .2  50.0  200.0   .7   6.0  80.4

277.6   74.0   73.2  72.4  3.13    7.2   3.5   87.5    6  50.1N

39.6N  97.7W   8.5  15.0  2.48     .2  50.0  200.0   .7   8.0  84.5

268.0   74.1   73.6  73.1  3.23    8.2   3.2   78.1    6  50.1N

39.6N  97.7W   9.5  16.0  2.93     .2  50.0  200.0   .7   9.3  90.1

264.2   75.0   74.4  73.9  3.26    9.1   2.9   70.2    6  50.1N

39.6N  97.7W  10.5  17.0  3.26    4.5  50.0  200.0   .7  10.2  96.8

267.6   75.9   75.2  74.6  3.23   10.1   2.8   64.4    6  50.1N

39.6N  97.7W  11.5  18.0  3.44    4.6  50.0  200.0   .7  10.9 103.1

275.8   76.7   76.0  75.3  3.16   11.1   2.7   61.3    6  50.1N

39.6N  97.7W  12.5  19.0  3.46    4.6  50.0  200.0   .7  11.4  95.9

283.9   77.5   76.7  76.0  3.11   12.1   3.0   61.3    6  50.1N

39.6N  97.7W  13.5  20.0  3.28    4.4  50.0  200.0   .7  11.5  95.4

288.4   77.4   76.6  75.9  3.09   13.1   3.0   64.5    6  50.1N

39.6N  97.7W  14.5  21.0  2.93     .2  50.0  200.0   .7  11.2  92.4

289.9   77.2   76.5  75.7  3.10   14.1   3.1   70.3    6  50.1N

39.6N  97.7W  15.5  22.0  2.43     .2  50.0  200.0   .7  10.8  88.4

290.6   77.1   76.4  75.6  3.11   15.1   3.3   78.2    6  50.1N

39.6N  97.7W  16.5  23.0  1.88     .2  50.0  200.0   .7   9.9  84.3

291.4   76.9   76.2  75.4  3.13   16.1   3.5   87.6    6  50.1N

39.6N  97.7W  17.5    .0  1.38     .2  50.0  200.0   .7   8.7  80.3

292.8   76.1   75.2  74.3  3.13   16.9   3.6   98.0    6  50.1N

39.6N  97.7W  18.5   1.0   .99     .2  50.0  200.0   .7   7.2  76.7

295.1   75.2   74.3  73.3  3.12   17.8   3.9  109.1    0  50.1N

39.6N  97.7W  19.5   2.0   .74     .2  50.0  200.0   .7   6.0  73.7

299.3   74.4   73.4  72.3  3.10   18.6   4.1  120.5    0  50.1N

39.6N  97.7W  20.5   3.0   .61     .2  50.0  200.0   .7   5.1  71.6

305.3   73.4   72.2  71.1  3.06   19.6   4.3  132.0    0  50.1N

39.6N  97.7W  21.5   4.0   .55     .2  50.0  200.0   .7   4.3  70.2

311.4   72.8   71.6  70.4  3.01   20.7   4.4  143.3    0  50.1N

39.6N  97.7W  22.5   5.0   .54     .2  50.0  200.0   .7   3.8  69.1

315.2   72.5   71.4  70.2  2.98   22.1   4.6  153.6    0  50.1N

39.6N  97.7W  23.5   6.0   .56     .2  50.0  200.0   .7   3.8  68.3

316.0   72.2   71.1  70.1  2.97   23.3   4.6  160.7    0  50.1N

39.6N  97.7W    .5   7.0   .58     .2  50.0  200.0   .7   4.1  67.9

315.9   72.2   71.1  70.0  2.97     .4   4.7  160.6    0  50.1N

Figure 2     Ionospheric parameters output            (METHOD = 1)

METHOD  2   ICEPAC IC.10   PAGE   2

JAN    1970          SSN = 100.

BOULDER,COLORADO TO ST. LOUIS,MO.         AZIMUTHS          N. MI.      KM

40.03 N  105.30 W - 38.67 N   90.25 W     91.84  281.42     702.6   1301.1

XMTR   2.0  TO  30.0  CONST. GAIN  H     .00 L     .00 A     .0  OFF AZ     .0

GMT= 19.0  LMT= 12.5  LAT= 39.59 N  LONG=  97.70 W  DIST= 651.  651. KM

600+----+----+----+----+----+----+----+----+----+----+----+----+----+----+

-  E= 3.46  20.0 110.0

- F1= 4.58  50.0 200.0

- F2=11.38  95.9 283.9

- ES= 2.15  2.40  2.98 110.0

-

-

-

-

-

500+

-

-

-

-

-

-

-

-                       X

-

400+

-                                                        X

-

-

-                                                       X

-                      X

-                                                      X

-

-

-                                                    X

300+                           X

-                      X                        X

-                                X    X    X

-                     X                                  .

-                                                      ..

-                                                    .

-                     X                         .

-                                          .

-                    X           .    .

-                    X      .

200+                   X

-

-                   X   .

-                  X

-                  X   .

-                      .

-                XX   .

-                    .

-               X   ..

-             X    ..

100+UUUUUUUUUUUMLLL----+----+----+----+----+----+----+----+----+----+----+

2         4         6         8        10        12        14

Figure 3.    Ionogram output.                         (METHOD = 2)

GAUSSIAN   HP-F1 IS PARABOLIC    FVERT   HTRUE   HPRIM

----+----+----+----+----+----++

-  1     .01   70.00   70.00

-  2     .61   70.89   72.07

-  3    1.22   74.47   79.16

-  4    1.82   80.40   90.90

-  5    2.12   84.20   98.40

-  6    2.42   88.56  107.04

-  7    2.71   93.46  116.65

-  8    3.01   98.92  127.42

-  9    3.31  106.06  145.12

+ 10    3.42  109.07  151.54

- 11    3.63  116.13  172.00

- 12    3.73  119.84  180.56

- 13    3.83  124.32  194.80

- 14    3.93  128.94  206.05

- 15    4.03  134.12  221.26

- 16    4.13  139.57  234.50

- 17    4.23  145.81  253.57

- 18    4.33  152.70  276.09

- 19    4.43  160.78  304.17

+ 20    4.53  171.87  356.99

- 21    4.63  186.99  430.56

- 22    5.60  215.64  306.15

- 23    6.57  225.34  290.15

- 24    7.55  233.31  288.72

- 25    8.52  240.95  294.62

- 26    9.49  249.01  304.74

- 27   10.47  259.14  324.81

- 28   10.92  266.35  349.52

- 29   11.15  270.68  371.54

+ 30   11.26  275.43  403.26

-

-

-

-

-

-

-

-

-

+

-

-

-

-

-

-

-

-

-

----+----+----+----+----+----++

16        18        20

Figure 3a.    Ionogram output continued

METHOD  7   ICEPAC IC.10   PAGE   3

JAN    1970          SSN = 100.

BOULDER,COLORADO TO ST. LOUIS,MO.         AZIMUTHS          N. MI.      KM

40.03 N  105.30 W - 38.67 N   90.25 W     91.84  281.42     702.6   1301.1

MINIMUM ANGLE    .0  DEGREES

XMTR   2.0  TO  30.0  CONST. GAIN  H     .00 L     .00 A     .0  OFF AZ     .0

UT    LT            FOT   MUF   HPF ANGLE VIRTL  TRUE  FVERT

8.0   1.0  E-LAYER   2.0   2.3   2.6   9.9  150.  103.    .6

F1LAYER   2.0   2.3   2.6   9.9  150.  103.    .6

F2LAYER   6.8   8.2  10.4  25.9  369.  299.   4.2

ESLAYER   4.3   5.9  12.9   6.6  110.  110.   1.3

9.0   2.0  E-LAYER   2.0   2.3   2.6   9.9  150.  103.    .6

F1LAYER   2.0   2.3   2.6   9.9  150.  103.    .6

F2LAYER   6.6   8.1   9.5  26.8  382.  305.   4.3

ESLAYER   4.0   5.7  12.0   6.6  110.  110.   1.2

10.0   3.0  E-LAYER   2.1   2.4   2.7   9.9  150.  103.    .6

F1LAYER   2.1   2.4   2.7   9.9  150.  103.    .6

F2LAYER   6.1   7.5   8.8  26.6  380.  310.   3.9

ESLAYER   3.7   5.5  11.4   6.6  110.  110.   1.2

11.0   4.0  E-LAYER   2.5   2.9   3.3   9.8  149.  103.    .7

F1LAYER   2.5   2.9   3.3   9.8  149.  103.    .7

F2LAYER   5.2   6.4   7.5  26.7  382.  307.   3.4

ESLAYER   3.4   5.4  11.1   6.6  110.  110.   1.2

12.0   5.0  E-LAYER   3.5   4.0   4.5   9.7  148.  103.   1.0

F1LAYER   3.5   4.0   4.5   9.7  148.  103.   1.0

F2LAYER   5.1   6.3   7.4  25.3  359.  288.   3.2

ESLAYER   3.4   5.5  11.3   6.6  110.  110.   1.2

13.0   6.0  E-LAYER   4.9   5.7   6.4   9.7  147.  103.   1.4

F1LAYER   4.9   5.7   6.4   9.7  147.  103.   1.4

F2LAYER   7.0   8.2   9.2  24.2  342.  269.   4.0

ESLAYER   3.7   6.0  11.9   6.6  110.  110.   1.3

14.0   7.0  E-LAYER   6.7   7.7   8.7   9.7  147.  103.   1.9

F1LAYER   6.7   7.7   8.7   9.7  147.  103.   1.9

F2LAYER  10.3  12.2  13.6  22.8  321.  259.   5.6

ESLAYER   4.3   7.0  12.8   6.6  110.  110.   1.5

15.0   8.0  E-LAYER   8.5   9.8  11.0   9.7  147.  103.   2.4

F1LAYER   8.5   9.8  11.0   9.7  147.  103.   2.4

F2LAYER  13.9  16.4  18.4  22.0  310.  251.   7.4

ESLAYER   5.0   8.2  13.6   6.6  110.  110.   1.8

16.0   9.0  E-LAYER  10.1  11.6  13.0   9.7  147.  103.   2.8

F1LAYER  10.1  11.6  13.0   9.7  147.  103.   2.8

F2LAYER  16.6  19.5  21.8  21.2  299.  248.   8.6

ESLAYER   5.8   9.5  14.3   6.6  110.  110.   2.0

17.0  10.0  E-LAYER  11.2  12.9  14.6   9.4  144.  106.   3.1

F1LAYER   8.8  10.0  11.3  21.5  303.  154.   4.2

F2LAYER  17.6  20.4  23.3  22.5  318.  248.   9.3

ESLAYER   6.6  10.6  14.6   6.6  110.  110.   2.3

18.0  11.0  E-LAYER  11.9  13.6  15.3   9.5  145.  106.   3.3

F1LAYER   9.0  10.3  11.7  21.6  304.  152.   4.3

F2LAYER  18.2  21.2  24.2  23.4  331.  253.  10.0

ESLAYER   8.0  11.2  14.4   6.6  110.  110.   2.4

Figure 4.    MUF complete output table                (METHOD = 7)

19.0  12.0  E-LAYER  11.9  13.6  15.4   9.5  145.  106.   3.3

F1LAYER   9.0  10.3  11.6  21.6  304.  152.   4.3

F2LAYER  18.9  22.0  25.1  23.9  338.  260.  10.5

ESLAYER   8.5  11.1  13.8   6.6  110.  110.   2.4

20.0  13.0  E-LAYER  11.3  13.0  14.6   9.5  145.  106.   3.1

F1LAYER   8.7  10.0  11.3  21.6  304.  153.   4.2

F2LAYER  18.7  21.8  24.8  24.7  350.  264.  10.6

ESLAYER   8.0  10.5  13.0   6.6  110.  110.   2.3

21.0  14.0  E-LAYER  10.1  11.6  13.1   9.7  147.  103.   2.8

F1LAYER  10.1  11.6  13.1   9.7  147.  103.   2.8

F2LAYER  18.7  21.8  24.8  23.8  337.  268.  10.4

ESLAYER   6.7   9.4  12.1   6.6  110.  110.   2.0

22.0  15.0  E-LAYER   8.4   9.6  10.8   9.7  147.  103.   2.3

F1LAYER   8.4   9.6  10.8   9.7  147.  103.   2.3

F2LAYER  18.0  20.9  23.8  23.9  337.  269.  10.0

ESLAYER   5.0   8.2  11.4   6.6  110.  110.   1.8

23.0  16.0  E-LAYER   6.5   7.4   8.4   9.7  147.  103.   1.8

F1LAYER   6.5   7.4   8.4   9.7  147.  103.   1.8

F2LAYER  16.6  19.3  22.0  23.9  338.  270.   9.3

ESLAYER   4.3   7.1  11.1   6.6  110.  110.   1.5

.0  17.0  E-LAYER   4.7   5.4   6.1   9.7  147.  103.   1.3

F1LAYER   4.7   5.4   6.1   9.7  147.  103.   1.3

F2LAYER  14.3  16.7  19.0  24.5  348.  271.   8.1

ESLAYER   3.8   6.2  11.3   6.6  110.  110.   1.3

1.0  18.0  E-LAYER   3.4   3.9   4.4   9.7  148.  103.    .9

F1LAYER   3.4   3.9   4.4   9.7  148.  103.    .9

F2LAYER  11.0  13.9  17.8  24.5  348.  276.   6.8

ESLAYER   3.5   5.7  11.9   6.6  110.  110.   1.2

2.0  19.0  E-LAYER   2.5   2.9   3.3   9.8  149.  103.    .7

F1LAYER   2.5   2.9   3.3   9.8  149.  103.    .7

F2LAYER   9.2  11.6  14.9  24.5  347.  280.   5.7

ESLAYER   4.0   5.5  12.7   6.6  110.  110.   1.2

3.0  20.0  E-LAYER   2.1   2.4   2.7   9.9  150.  103.    .6

F1LAYER   2.1   2.4   2.7   9.9  150.  103.    .6

F2LAYER   7.6   9.6  12.3  25.3  360.  287.   4.8

ESLAYER   4.1   5.5  13.6   6.6  110.  110.   1.2

4.0  21.0  E-LAYER   1.9   2.1   2.4   9.9  150.  103.    .5

F1LAYER   1.9   2.1   2.4   9.9  150.  103.    .5

F2LAYER   6.3   8.0  10.2  26.1  371.  293.   4.1

ESLAYER   4.3   5.6  14.2   6.6  110.  110.   1.2

5.0  22.0  E-LAYER   1.8   2.1   2.4   9.9  150.  103.    .5

F1LAYER   1.8   2.1   2.4   9.9  150.  103.    .5

F2LAYER   5.8   7.0   8.9  26.1  372.  297.   3.6

ESLAYER   4.6   5.7  14.5   6.6  110.  110.   1.2

6.0  23.0  E-LAYER   1.9   2.2   2.5   9.9  150.  103.    .5

F1LAYER   1.9   2.2   2.5   9.9  150.  103.    .5

F2LAYER   5.8   7.0   8.8  26.0  371.  298.   3.6

ESLAYER   4.7   5.9  14.3   6.6  110.  110.   1.3

7.0  24.0  E-LAYER   2.0   2.2   2.5   9.9  150.  103.    .6

F1LAYER   2.0   2.2   2.5   9.9  150.  103.    .6

F2LAYER   6.3   7.6   9.6  26.0  371.  298.   3.9

ESLAYER   4.6   5.9  13.8   6.6  110.  110.   1.3

Figure 4a.    MUF complete output table continued

METHOD  8   ICEPAC IC.10   PAGE   4

JAN    1970          SSN = 100.

BOULDER,COLORADO TO ST. LOUIS,MO.         AZIMUTHS          N. MI.      KM

40.03 N  105.30 W - 38.67 N   90.25 W     91.84  281.42     702.6   1301.1

MINIMUM ANGLE    .0  DEGREES

XMTR   2.0  TO  30.0  CONST. GAIN  H     .00 L     .00 A     .0  OFF AZ     .0

MUF(....)       FOT(XXXX)

00  02  04  06  08  10  12  14  16  18  20  22  00

MHZ+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+MHZ

40-                                                   -40

-                                                   -

38-                                                   -38

-                                                   -

36-                                                   -36  GMT   MUF   FOT

-                                                   -

34-                                                   -34  1.0  13.9  11.0

-                                                   -    2.0  11.6   9.2

32-                                                   -32  3.0   9.6   7.6

-                                                   -    4.0   8.0   6.3

30-                                                   -30  5.0   7.0   5.8

-                                                   -    6.0   7.0   5.8

28-                                                   -28  7.0   7.6   6.3

-                                                   -    8.0   8.2   6.8

26-                                                   -26  9.0   8.1   6.6

-                                                   -   10.0   7.5   6.1

24-                                                   -24 11.0   6.4   5.2

-                                                   -   12.0   6.3   5.1

22-                                       . . .       -22 13.0   8.2   7.0

-                                     .       .     -   14.0  12.2  10.3

20-                                 . .               -20 15.0  16.4  13.9

-                                       X X X   .   -   16.0  19.5  16.6

18-                                   X X       X     -18 17.0  20.4  17.6

- .                               X             X . -   18.0  21.2  18.2

16-                               .                   -16 19.0  22.0  18.9

-                                                   -   20.0  21.8  18.7

14- X .                           X                 X -14 21.0  21.8  18.7

-                                                   -   22.0  20.9  18.0

12-     .                       .                     -12 23.0  19.3  16.6

-   X                                               -     .0  16.7  14.3

10-       .                     X                     -10

-     X                                             -

08-       X .     . . .       .                       -08

-           . .   X X .     X                       -

06-         X X     . .                         -06

-                       X X                         -

04-                                                   -04

-                                                   -

02-                                                   -02

-                                                   -

MHZ+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+MHZ

00  02  04  06  08  10  12  14  16  18  20  22  00

UNIVERSAL TIME

Figure 5.    MUF-FOT graph                            (METHOD = 8)

METHOD  9   ICEPAC IC.10   PAGE   5

JAN    1970          SSN = 100.

BOULDER,COLORADO TO ST. LOUIS,MO.         AZIMUTHS          N. MI.      KM

40.03 N  105.30 W - 38.67 N   90.25 W     91.84  281.42     702.6   1301.1

MINIMUM ANGLE    .0  DEGREES

XMTR   2.0  TO  30.0  CONST. GAIN  H     .00 L     .00 A     .0  OFF AZ     .0

MUF(....)       FOT(XXXX)       HPF(++++)

00  02  04  06  08  10  12  14  16  18  20  22  00

MHZ+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+MHZ

40-                                                   -40

-                                                   -

38-                                                   -38

-                                                   -

36-                                                   -36  GMT   MUF   FOT   HPF

-                                                   -

34-                                                   -34  1.0  13.9  11.0  17.8

-                                                   -    2.0  11.6   9.2  14.9

32-                                                   -32  3.0   9.6   7.6  12.3

-                                                   -    4.0   8.0   6.3  10.2

30-                                                   -30  5.0   7.0   5.8   8.9

-                                                   -    6.0   7.0   5.8   8.8

28-                                                   -28  7.0   7.6   6.3   9.6

-                                                   -    8.0   8.2   6.8  10.4

26-                                                   -26  9.0   8.1   6.6   9.5

-                                       + + +       -   10.0   7.5   6.1   8.8

24-                                     +       +     -24 11.0   6.4   5.2   7.5

-                                   +               -   12.0   6.3   5.1   7.4

22-                                 +     . . .   +   -22 13.0   8.2   7.0   9.2

-                                     .       .     -   14.0  12.2  10.3  13.6

20-                                 . .               -20 15.0  16.4  13.9  18.4

- +                                     X X X   . + -   16.0  19.5  16.6  21.8

18-   +                           +   X X       X     -18 17.0  20.4  17.6  23.3

- .                               X             X . -   18.0  21.2  18.2  24.2

16-                               .                   -16 19.0  22.0  18.9  25.1

-     +                                             -   20.0  21.8  18.7  24.8

14- X .                         + X                 X -14 21.0  21.8  18.7  24.8

-                                                   -   22.0  20.9  18.0  23.8

12-     . +                     .                     -12 23.0  19.3  16.6  22.0

-   X                                               -     .0  16.7  14.3  19.0

10-       . +     + + +         X                     -10

-     X     + +       +     +                       -

08-       X .     . . .   +   .                       -08

-           . .   X X .   + X                       -

06-         X X X X     X . .                         -06

-                       X X                         -

04-                                                   -04

-                                                   -

02-                                                   -02

-                                                   -

MHZ+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+MHZ

00  02  04  06  08  10  12  14  16  18  20  22  00

UNIVERSAL TIME

Figure 6.    MUF-FOT-HPF graph                        (METHOD = 9)

METHOD 10   ICEPAC IC.10   PAGE   6

JAN    1970          SSN = 100.

BOULDER,COLORADO TO ST. LOUIS,MO.         AZIMUTHS          N. MI.      KM

40.03 N  105.30 W - 38.67 N   90.25 W     91.84  281.42     702.6   1301.1

MINIMUM ANGLE    .0  DEGREES

XMTR   2.0  TO  30.0  CONST. GAIN  H     .00 L     .00 A     .0  OFF AZ     .0

MUF(....)       FOT(XXXX)       ANG(++++)

00  02  04  06  08  10  12  14  16  18  20  22  00

MHZ+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+MHZ

40-                                                   -40

-                                                   -

38-                                                   -38

-                                                   -

36-                                                   -36  GMT   MUF   FOT   ANG

-                                                   -

34-                                                   -34  1.0  13.9  11.0  24.5

-                                                   -    2.0  11.6   9.2  24.5

32-                                                   -32  3.0   9.6   7.6  25.3

-                                                   -    4.0   8.0   6.3  26.1

30-                                                   -30  5.0   7.0   5.8  26.1

-                                                   -    6.0   7.0   5.8  26.0

28-                                                   -28  7.0   7.6   6.3  26.0

-                   + + +                           -    8.0   8.2   6.8  25.9

26-         + + + + +                                 -26  9.0   8.1   6.6  26.8

- + +   +                 +               +       + -   10.0   7.5   6.1  26.6

24-     +                     +           +   + + +   -24 11.0   6.4   5.2  26.7

-                             +     + +             -   12.0   6.3   5.1  25.3

22-                               +       . . .       -22 13.0   8.2   7.0  24.2

-                                 +   .       .     -   14.0  12.2  10.3  22.8

20-                                 . .               -20 15.0  16.4  13.9  22.0

-                                       X X X   .   -   16.0  19.5  16.6  21.2

18-                                   X X       X     -18 17.0  20.4  17.6  22.5

- .                               X             X . -   18.0  21.2  18.2  23.4

16-                               .                   -16 19.0  22.0  18.9  23.9

-                                                   -   20.0  21.8  18.7  24.7

14- X .                           X                 X -14 21.0  21.8  18.7  23.8

-                                                   -   22.0  20.9  18.0  23.9

12-     .                       .                     -12 23.0  19.3  16.6  23.9

-   X                                               -     .0  16.7  14.3  24.5

10-       .                     X                     -10

-     X                                             -

08-       X .     . . .       .                       -08

-           . .   X X .     X                       -

06-         X X X X     X . .                         -06

-                       X X                         -

04-                                                   -04

-                                                   -

02-                                                   -02

-                                                   -

MHZ+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+MHZ

00  02  04  06  08  10  12  14  16  18  20  22  00

UNIVERSAL TIME

Figure 7.    MUF-FOT-ANG graph                        (METHOD = 10)

METHOD 11   ICEPAC IC.10   PAGE   7

JAN    1970          SSN = 100.

BOULDER,COLORADO TO ST. LOUIS,MO.         AZIMUTHS          N. MI.      KM

40.03 N  105.30 W - 38.67 N   90.25 W     91.84  281.42     702.6   1301.1

MINIMUM ANGLE    .0  DEGREES

XMTR   2.0  TO  30.0  CONST. GAIN  H     .00 L     .00 A     .0  OFF AZ     .0

MUF(....)       FOT(XXXX)     ESMUF(++++)

00  02  04  06  08  10  12  14  16  18  20  22  00

MHZ+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+MHZ

40-                                                   -40

-                                                   -

38-                                                   -38

-                                                   -

36-                                                   -36  GMT   MUF   FOT ESMUF

-                                                   -

34-                                                   -34  1.0  13.9  11.0   5.7

-                                                   -    2.0  11.6   9.2   5.5

32-                                                   -32  3.0   9.6   7.6   5.5

-                                                   -    4.0   8.0   6.3   5.6

30-                                                   -30  5.0   7.0   5.8   5.7

-                                                   -    6.0   7.0   5.8   5.9

28-                                                   -28  7.0   7.6   6.3   5.9

-                                                   -    8.0   8.2   6.8   5.9

26-                                                   -26  9.0   8.1   6.6   5.7

-                                                   -   10.0   7.5   6.1   5.5

24-                                                   -24 11.0   6.4   5.2   5.4

-                                                   -   12.0   6.3   5.1   5.5

22-                                       . . .       -22 13.0   8.2   7.0   6.0

-                                     .       .     -   14.0  12.2  10.3   7.0

20-                                 . .               -20 15.0  16.4  13.9   8.2

-                                       X X X   .   -   16.0  19.5  16.6   9.5

18-                                   X X       X     -18 17.0  20.4  17.6  10.6

- .                               X             X . -   18.0  21.2  18.2  11.2

16-                               .                   -16 19.0  22.0  18.9  11.1

-                                                   -   20.0  21.8  18.7  10.5

14- X .                           X                 X -14 21.0  21.8  18.7   9.4

-                                                   -   22.0  20.9  18.0   8.2

12-     .                       .                     -12 23.0  19.3  16.6   7.1

-   X                               + + +           -     .0  16.7  14.3   6.2

10-       .                     X   +       +         -10

-     X                                     +       -

08-       X .     . . .       .   +             +     -08

-           . .   X X .     X +                 +   -

06- + +     + + + + + + + . + +                     + -06

-     + +               + X                         -

04-                                                   -04

-                                                   -

02-                                                   -02

-                                                   -

MHZ+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+MHZ

00  02  04  06  08  10  12  14  16  18  20  22  00

UNIVERSAL TIME

Figure 8.    MUF-FOT-Es MUF graph.                    (METHOD = 11)

METHOD 15   ICEPAC IC.10   PAGE   8

ITSA 1 ANTENNA PACKAGE             ANTENNA PATTERN

FREQUENCY RANGE  ANTENNA TYPE      HEIGHT    LENGTH     ANGLE

2.0 TO  30.0   VER MONOPOLE        .000     -.500      .000

2     3     4     5     6     7     8     9    10    11

90 -50.0 -50.0 -50.0 -50.0 -50.0 -50.0 -50.0 -50.0 -50.0 -50.0

88 -36.5 -35.5 -35.0 -34.7 -34.5 -34.4 -34.3 -34.2 -34.2 -34.1

86 -30.5 -29.5 -29.0 -28.7 -28.5 -28.4 -28.3 -28.2 -28.2 -28.1

84 -27.0 -26.0 -25.5 -25.2 -25.0 -24.8 -24.8 -24.7 -24.6 -24.6

82 -24.6 -23.6 -23.0 -22.7 -22.5 -22.4 -22.3 -22.2 -22.1 -22.1

80 -22.7 -21.7 -21.1 -20.8 -20.6 -20.4 -20.3 -20.2 -20.2 -20.1

78 -21.2 -20.2 -19.6 -19.2 -19.0 -18.8 -18.7 -18.7 -18.6 -18.5

76 -20.0 -18.9 -18.3 -17.9 -17.7 -17.5 -17.4 -17.3 -17.2 -17.2

74 -18.9 -17.8 -17.1 -16.8 -16.5 -16.3 -16.2 -16.1 -16.1 -16.0

72 -18.0 -16.8 -16.1 -15.7 -15.5 -15.3 -15.2 -15.1 -15.0 -14.9

E 70 -17.2 -15.9 -15.2 -14.8 -14.5 -14.4 -14.2 -14.1 -14.0 -14.0

L 68 -16.4 -15.1 -14.4 -14.0 -13.7 -13.5 -13.3 -13.2 -13.1 -13.1

E 66 -15.7 -14.4 -13.6 -13.2 -12.9 -12.7 -12.5 -12.4 -12.3 -12.2

V 64 -15.0 -13.6 -12.9 -12.4 -12.1 -11.9 -11.7 -11.6 -11.5 -11.4

A 62 -14.3 -12.9 -12.2 -11.7 -11.4 -11.2 -11.0 -10.9 -10.8 -10.7

T 60 -13.6 -12.2 -11.5 -11.0 -10.7 -10.4 -10.3 -10.1 -10.0 -10.0

I 58 -12.8 -11.6 -10.8 -10.3 -10.0  -9.7  -9.6  -9.4  -9.3  -9.2

O 56 -12.1 -10.9 -10.1  -9.6  -9.3  -9.0  -8.9  -8.7  -8.6  -8.5

N 54 -11.3 -10.2  -9.4  -8.9  -8.6  -8.4  -8.2  -8.1  -8.0  -7.9

52 -10.6  -9.5  -8.8  -8.3  -8.0  -7.7  -7.5  -7.4  -7.3  -7.2

A 50  -9.8  -8.8  -8.1  -7.6  -7.3  -7.1  -6.9  -6.8  -6.6  -6.6

N 48  -9.0  -8.1  -7.4  -7.0  -6.7  -6.4  -6.3  -6.1  -6.0  -5.9

G 46  -8.2  -7.4  -6.8  -6.3  -6.0  -5.8  -5.6  -5.5  -5.4  -5.3

L 44  -7.4  -6.7  -6.1  -5.7  -5.4  -5.2  -5.1  -4.9  -4.8  -4.7

E 42  -6.7  -6.0  -5.5  -5.1  -4.9  -4.7  -4.5  -4.4  -4.3  -4.2

40  -6.0  -5.4  -4.9  -4.6  -4.3  -4.1  -4.0  -3.8  -3.7  -3.7

I 38  -5.3  -4.8  -4.4  -4.1  -3.8  -3.6  -3.5  -3.3  -3.2  -3.2

N 36  -4.7  -4.3  -3.9  -3.6  -3.3  -3.1  -3.0  -2.9  -2.8  -2.7

34  -4.1  -3.8  -3.4  -3.1  -2.9  -2.7  -2.6  -2.5  -2.4  -2.3

D 32  -3.6  -3.3  -3.0  -2.7  -2.5  -2.3  -2.2  -2.1  -2.0  -1.9

E 30  -3.1  -2.9  -2.6  -2.4  -2.2  -2.0  -1.9  -1.8  -1.7  -1.6

G 28  -2.7  -2.6  -2.3  -2.1  -1.9  -1.7  -1.6  -1.5  -1.4  -1.4

R 26  -2.4  -2.3  -2.1  -1.9  -1.7  -1.5  -1.4  -1.3  -1.2  -1.2

E 24  -2.1  -2.1  -1.9  -1.7  -1.5  -1.4  -1.3  -1.2  -1.1  -1.1

E 22  -2.0  -2.0  -1.8  -1.6  -1.5  -1.3  -1.2  -1.1  -1.1  -1.0

S 20  -1.9  -2.0  -1.8  -1.6  -1.5  -1.4  -1.3  -1.2  -1.1  -1.1

18  -1.9  -2.0  -1.9  -1.8  -1.6  -1.5  -1.4  -1.3  -1.3  -1.2

16  -2.1  -2.2  -2.1  -2.0  -1.9  -1.8  -1.7  -1.6  -1.5  -1.5

14  -2.4  -2.6  -2.5  -2.4  -2.3  -2.2  -2.1  -2.0  -2.0  -1.9

12  -2.9  -3.1  -3.1  -3.0  -2.9  -2.8  -2.7  -2.6  -2.6  -2.5

10  -3.6  -4.0  -3.9  -3.9  -3.8  -3.7  -3.6  -3.5  -3.4  -3.4

8  -4.7  -5.1  -5.2  -5.1  -5.0  -4.9  -4.8  -4.7  -4.7  -4.6

6  -6.4  -6.9  -6.9  -6.9  -6.8  -6.7  -6.6  -6.5  -6.5  -6.4

4  -9.1  -9.6  -9.7  -9.7  -9.6  -9.5  -9.4  -9.4  -9.3  -9.3

2 -14.3 -14.9 -15.0 -15.0 -14.9 -14.8 -14.8 -14.7 -14.7 -14.6

0 -50.0 -50.0 -50.0 -50.0 -50.0 -50.0 -50.0 -50.0 -50.0 -50.0

2     3     4     5     6     7     8     9    10    11

FREQUENCY IN MEGAHERTZ

ANTENNA EFFICIENCY

.0    .0    .0    .0    .0    .0    .0    .0    .0    .0

2     3     4     5     6     7     8     9    10    11

FREQUENCY IN MEGAHERTZ

Figure 9.    Transmitter antenna pattern.             (METHOD = 15)

METHOD 15   ICEPAC IC.10   PAGE   9

AZIMUTH     EX(1)     EX(2)     EX(3)     EX(4)  CONDUCT.  DIELECT.

.000     .000      .000      .000      .000     .001     4.000

12    13    14    16    18    20    22    24    26    28    30

-50.0 -50.0 -50.0 -50.0 -50.0 -50.0 -50.0 -50.0 -50.0 -50.0 -50.0 90

-34.1 -34.1 -34.1 -34.0 -34.0 -34.0 -34.0 -34.0 -34.0 -34.0 -34.0 88

-28.1 -28.1 -28.0 -28.0 -28.0 -28.0 -28.0 -28.0 -28.0 -28.0 -27.9 86

-24.6 -24.5 -24.5 -24.5 -24.5 -24.5 -24.4 -24.4 -24.4 -24.4 -24.4 84

-22.1 -22.0 -22.0 -22.0 -22.0 -21.9 -21.9 -21.9 -21.9 -21.9 -21.9 82

-20.1 -20.1 -20.1 -20.0 -20.0 -20.0 -20.0 -20.0 -19.9 -19.9 -19.9 80

-18.5 -18.5 -18.5 -18.4 -18.4 -18.4 -18.4 -18.3 -18.3 -18.3 -18.3 78

-17.1 -17.1 -17.1 -17.0 -17.0 -17.0 -17.0 -17.0 -17.0 -16.9 -16.9 76

-16.0 -15.9 -15.9 -15.8 -15.8 -15.8 -15.8 -15.8 -15.7 -15.7 -15.7 74

-14.9 -14.9 -14.8 -14.8 -14.7 -14.7 -14.7 -14.7 -14.7 -14.6 -14.6 72

-13.9 -13.9 -13.8 -13.8 -13.8 -13.7 -13.7 -13.7 -13.7 -13.6 -13.6 70

-13.0 -13.0 -12.9 -12.9 -12.8 -12.8 -12.8 -12.8 -12.7 -12.7 -12.7 68

-12.2 -12.1 -12.1 -12.0 -12.0 -11.9 -11.9 -11.9 -11.9 -11.9 -11.8 66

-11.4 -11.3 -11.3 -11.2 -11.2 -11.1 -11.1 -11.1 -11.1 -11.0 -11.0 64

-10.6 -10.6 -10.5 -10.5 -10.4 -10.4 -10.3 -10.3 -10.3 -10.3 -10.2 62

-9.9  -9.8  -9.8  -9.7  -9.7  -9.6  -9.6  -9.5  -9.5  -9.5  -9.5 60

-9.2  -9.1  -9.1  -9.0  -8.9  -8.9  -8.8  -8.8  -8.8  -8.8  -8.7 58

-8.5  -8.4  -8.4  -8.3  -8.2  -8.2  -8.1  -8.1  -8.1  -8.1  -8.0 56

-7.8  -7.7  -7.7  -7.6  -7.5  -7.5  -7.5  -7.4  -7.4  -7.4  -7.3 54

-7.1  -7.1  -7.0  -6.9  -6.9  -6.8  -6.8  -6.8  -6.7  -6.7  -6.7 52

-6.5  -6.4  -6.4  -6.3  -6.2  -6.2  -6.1  -6.1  -6.1  -6.0  -6.0 50

-5.9  -5.8  -5.7  -5.7  -5.6  -5.5  -5.5  -5.5  -5.4  -5.4  -5.4 48

-5.3  -5.2  -5.1  -5.1  -5.0  -4.9  -4.9  -4.9  -4.8  -4.8  -4.8 46

-4.7  -4.6  -4.6  -4.5  -4.4  -4.4  -4.3  -4.3  -4.3  -4.2  -4.2 44

-4.1  -4.1  -4.0  -3.9  -3.9  -3.8  -3.8  -3.7  -3.7  -3.7  -3.7 42

-3.6  -3.5  -3.5  -3.4  -3.4  -3.3  -3.3  -3.2  -3.2  -3.2  -3.2 40

-3.1  -3.1  -3.0  -2.9  -2.9  -2.8  -2.8  -2.8  -2.7  -2.7  -2.7 38

-2.7  -2.6  -2.6  -2.5  -2.4  -2.4  -2.3  -2.3  -2.3  -2.3  -2.2 36

-2.2  -2.2  -2.1  -2.1  -2.0  -2.0  -1.9  -1.9  -1.9  -1.9  -1.8 34

-1.9  -1.8  -1.8  -1.7  -1.7  -1.6  -1.6  -1.6  -1.5  -1.5  -1.5 32

-1.6  -1.5  -1.5  -1.4  -1.4  -1.3  -1.3  -1.3  -1.2  -1.2  -1.2 30

-1.3  -1.3  -1.2  -1.2  -1.1  -1.1  -1.0  -1.0  -1.0  -1.0   -.9 28

-1.1  -1.1  -1.0  -1.0   -.9   -.9   -.9   -.8   -.8   -.8   -.8 26

-1.0  -1.0   -.9   -.9   -.8   -.8   -.7   -.7   -.7   -.7   -.7 24

-1.0   -.9   -.9   -.8   -.8   -.7   -.7   -.7   -.7   -.6   -.6 22

-1.0  -1.0   -.9   -.9   -.8   -.8   -.8   -.7   -.7   -.7   -.7 20

-1.2  -1.1  -1.1  -1.0  -1.0   -.9   -.9   -.9   -.9   -.8   -.8 18

-1.4  -1.4  -1.4  -1.3  -1.3  -1.2  -1.2  -1.2  -1.1  -1.1  -1.1 16

-1.9  -1.8  -1.8  -1.7  -1.7  -1.7  -1.6  -1.6  -1.6  -1.6  -1.5 14

-2.5  -2.4  -2.4  -2.4  -2.3  -2.3  -2.2  -2.2  -2.2  -2.2  -2.2 12

-3.4  -3.3  -3.3  -3.2  -3.2  -3.2  -3.1  -3.1  -3.1  -3.1  -3.1 10

-4.6  -4.6  -4.5  -4.5  -4.4  -4.4  -4.4  -4.4  -4.3  -4.3  -4.3  8

-6.4  -6.4  -6.3  -6.3  -6.2  -6.2  -6.2  -6.2  -6.1  -6.1  -6.1  6

-9.2  -9.2  -9.2  -9.1  -9.1  -9.0  -9.0  -9.0  -9.0  -9.0  -8.9  4

-14.6 -14.5 -14.5 -14.5 -14.4 -14.4 -14.4 -14.3 -14.3 -14.3 -14.3  2

-50.0 -50.0 -50.0 -50.0 -50.0 -50.0 -50.0 -50.0 -50.0 -50.0 -50.0  0

12    13    14    16    18    20    22    24    26    28    30

FREQUENCY IN MEGAHERTZ

ANTENNA EFFICIENCY

.0    .0    .0    .0    .0    .0    .0    .0    .0    .0    .0

12    13    14    16    18    20    22    24    26    28    30

FREQUENCY IN MEGAHERTZ

Figure 9a.    Transmitter antenna pattern continued.

METHOD 15   ICEPAC IC.10   PAGE  10

ITSA 1 ANTENNA PACKAGE             ANTENNA PATTERN

FREQUENCY RANGE  ANTENNA TYPE      HEIGHT    LENGTH     ANGLE

2.0 TO  30.0   VER MONOPOLE        .000     -.250      .000

2     3     4     5     6     7     8     9    10    11

90 -50.6 -50.6 -50.6 -50.6 -50.6 -50.6 -50.6 -50.6 -50.6 -50.6

88 -29.7 -30.0 -30.1 -30.2 -30.2 -30.2 -30.2 -30.2 -30.2 -30.1

86 -23.6 -24.0 -24.1 -24.2 -24.2 -24.2 -24.2 -24.2 -24.1 -24.1

84 -20.1 -20.4 -20.6 -20.7 -20.7 -20.7 -20.7 -20.6 -20.6 -20.6

82 -17.6 -18.0 -18.1 -18.2 -18.2 -18.2 -18.2 -18.1 -18.1 -18.1

80 -15.7 -16.0 -16.2 -16.2 -16.2 -16.2 -16.2 -16.2 -16.2 -16.2

78 -14.1 -14.4 -14.6 -14.7 -14.7 -14.7 -14.6 -14.6 -14.6 -14.6

76 -12.8 -13.1 -13.3 -13.3 -13.3 -13.3 -13.3 -13.3 -13.3 -13.3

74 -11.6 -11.9 -12.1 -12.2 -12.2 -12.2 -12.2 -12.1 -12.1 -12.1

72 -10.6 -10.9 -11.1 -11.2 -11.2 -11.2 -11.2 -11.1 -11.1 -11.1

E 70  -9.7 -10.0 -10.2 -10.3 -10.3 -10.3 -10.3 -10.2 -10.2 -10.2

L 68  -8.9  -9.2  -9.4  -9.4  -9.5  -9.5  -9.4  -9.4  -9.4  -9.4

E 66  -8.1  -8.5  -8.6  -8.7  -8.7  -8.7  -8.7  -8.7  -8.7  -8.7

V 64  -7.4  -7.8  -8.0  -8.0  -8.0  -8.0  -8.0  -8.0  -8.0  -8.0

A 62  -6.8  -7.2  -7.3  -7.4  -7.4  -7.4  -7.4  -7.4  -7.4  -7.4

T 60  -6.2  -6.6  -6.8  -6.8  -6.9  -6.9  -6.9  -6.8  -6.8  -6.8

I 58  -5.7  -6.1  -6.3  -6.3  -6.3  -6.3  -6.3  -6.3  -6.3  -6.3

O 56  -5.2  -5.6  -5.8  -5.8  -5.9  -5.9  -5.9  -5.8  -5.8  -5.8

N 54  -4.8  -5.2  -5.3  -5.4  -5.4  -5.4  -5.4  -5.4  -5.4  -5.4

52  -4.4  -4.7  -4.9  -5.0  -5.0  -5.0  -5.0  -5.0  -5.0  -5.0

A 50  -4.0  -4.4  -4.5  -4.6  -4.6  -4.6  -4.6  -4.6  -4.6  -4.6

N 48  -3.6  -4.0  -4.2  -4.3  -4.3  -4.3  -4.3  -4.3  -4.3  -4.3

G 46  -3.3  -3.7  -3.9  -4.0  -4.0  -4.0  -4.0  -4.0  -4.0  -4.0

L 44  -3.0  -3.4  -3.6  -3.7  -3.7  -3.7  -3.7  -3.7  -3.7  -3.7

E 42  -2.7  -3.2  -3.4  -3.4  -3.5  -3.5  -3.5  -3.5  -3.5  -3.4

40  -2.5  -2.9  -3.1  -3.2  -3.3  -3.3  -3.3  -3.3  -3.2  -3.2

I 38  -2.3  -2.8  -3.0  -3.0  -3.1  -3.1  -3.1  -3.1  -3.1  -3.1

N 36  -2.1  -2.6  -2.8  -2.9  -2.9  -2.9  -2.9  -2.9  -2.9  -2.9

34  -2.0  -2.5  -2.7  -2.8  -2.8  -2.8  -2.8  -2.8  -2.8  -2.8

D 32  -1.9  -2.4  -2.6  -2.7  -2.7  -2.8  -2.8  -2.8  -2.7  -2.7

E 30  -1.8  -2.3  -2.6  -2.7  -2.7  -2.7  -2.7  -2.7  -2.7  -2.7

G 28  -1.8  -2.3  -2.6  -2.7  -2.7  -2.7  -2.7  -2.7  -2.7  -2.7

R 26  -1.8  -2.4  -2.6  -2.7  -2.8  -2.8  -2.8  -2.8  -2.8  -2.8

E 24  -1.9  -2.5  -2.7  -2.9  -2.9  -2.9  -2.9  -2.9  -2.9  -2.9

E 22  -2.0  -2.6  -2.9  -3.0  -3.1  -3.1  -3.1  -3.1  -3.1  -3.1

S 20  -2.2  -2.9  -3.1  -3.3  -3.3  -3.3  -3.4  -3.3  -3.3  -3.3

18  -2.5  -3.2  -3.5  -3.6  -3.7  -3.7  -3.7  -3.7  -3.7  -3.7

16  -2.9  -3.6  -3.9  -4.0  -4.1  -4.1  -4.1  -4.1  -4.1  -4.1

14  -3.4  -4.1  -4.5  -4.6  -4.7  -4.7  -4.7  -4.7  -4.7  -4.7

12  -4.1  -4.8  -5.2  -5.3  -5.4  -5.5  -5.5  -5.5  -5.5  -5.5

10  -5.0  -5.8  -6.1  -6.3  -6.4  -6.4  -6.4  -6.5  -6.4  -6.4

8  -6.2  -7.1  -7.5  -7.6  -7.7  -7.8  -7.8  -7.8  -7.8  -7.8

6  -8.0  -8.9  -9.3  -9.5  -9.6  -9.6  -9.7  -9.7  -9.7  -9.7

4 -10.7 -11.7 -12.2 -12.4 -12.5 -12.5 -12.5 -12.5 -12.5 -12.5

2 -16.0 -17.0 -17.5 -17.7 -17.8 -17.9 -17.9 -17.9 -17.9 -17.9

0 -50.6 -50.6 -50.6 -50.6 -50.6 -50.6 -50.6 -50.6 -50.6 -50.6

2     3     4     5     6     7     8     9    10    11

FREQUENCY IN MEGAHERTZ

ANTENNA EFFICIENCY

-.6   -.6   -.6   -.6   -.6   -.6   -.6   -.6   -.6   -.6

2     3     4     5     6     7     8     9    10    11

FREQUENCY IN MEGAHERTZ

Figure 10.    Receiver-antenna pattern.               (METHOD = 15)

METHOD 15   ICEPAC IC.10   PAGE  11

AZIMUTH     EX(1)     EX(2)     EX(3)     EX(4)  CONDUCT.  DIELECT.

.000     .000      .000      .000      .000     .001     4.000

12    13    14    16    18    20    22    24    26    28    30

-50.6 -50.6 -50.6 -50.6 -50.6 -50.6 -50.6 -50.6 -50.6 -50.6 -50.6 90

-30.1 -30.1 -30.1 -30.0 -30.0 -30.0 -30.0 -30.0 -30.0 -29.9 -29.9 88

-24.1 -24.1 -24.1 -24.0 -24.0 -24.0 -24.0 -23.9 -23.9 -23.9 -23.9 86

-20.6 -20.6 -20.5 -20.5 -20.5 -20.5 -20.4 -20.4 -20.4 -20.4 -20.4 84

-18.1 -18.1 -18.0 -18.0 -18.0 -18.0 -17.9 -17.9 -17.9 -17.9 -17.9 82

-16.1 -16.1 -16.1 -16.1 -16.1 -16.0 -16.0 -16.0 -16.0 -16.0 -16.0 80

-14.6 -14.6 -14.5 -14.5 -14.5 -14.5 -14.4 -14.4 -14.4 -14.4 -14.4 78

-13.2 -13.2 -13.2 -13.2 -13.2 -13.1 -13.1 -13.1 -13.1 -13.1 -13.1 76

-12.1 -12.1 -12.1 -12.0 -12.0 -12.0 -12.0 -11.9 -11.9 -11.9 -11.9 74

-11.1 -11.1 -11.0 -11.0 -11.0 -11.0 -11.0 -10.9 -10.9 -10.9 -10.9 72

-10.2 -10.2 -10.1 -10.1 -10.1 -10.1 -10.1 -10.0 -10.0 -10.0 -10.0 70

-9.4  -9.4  -9.3  -9.3  -9.3  -9.3  -9.2  -9.2  -9.2  -9.2  -9.2 68

-8.6  -8.6  -8.6  -8.6  -8.6  -8.5  -8.5  -8.5  -8.5  -8.5  -8.5 66

-8.0  -7.9  -7.9  -7.9  -7.9  -7.9  -7.8  -7.8  -7.8  -7.8  -7.8 64

-7.4  -7.3  -7.3  -7.3  -7.3  -7.3  -7.2  -7.2  -7.2  -7.2  -7.2 62

-6.8  -6.8  -6.8  -6.7  -6.7  -6.7  -6.7  -6.7  -6.6  -6.6  -6.6 60

-6.3  -6.3  -6.2  -6.2  -6.2  -6.2  -6.2  -6.1  -6.1  -6.1  -6.1 58

-5.8  -5.8  -5.8  -5.7  -5.7  -5.7  -5.7  -5.7  -5.7  -5.6  -5.6 56

-5.4  -5.3  -5.3  -5.3  -5.3  -5.3  -5.2  -5.2  -5.2  -5.2  -5.2 54

-5.0  -4.9  -4.9  -4.9  -4.9  -4.9  -4.8  -4.8  -4.8  -4.8  -4.8 52

-4.6  -4.6  -4.6  -4.5  -4.5  -4.5  -4.5  -4.5  -4.5  -4.4  -4.4 50

-4.2  -4.2  -4.2  -4.2  -4.2  -4.2  -4.1  -4.1  -4.1  -4.1  -4.1 48

-3.9  -3.9  -3.9  -3.9  -3.9  -3.9  -3.8  -3.8  -3.8  -3.8  -3.8 46

-3.7  -3.7  -3.6  -3.6  -3.6  -3.6  -3.6  -3.6  -3.6  -3.5  -3.5 44

-3.4  -3.4  -3.4  -3.4  -3.4  -3.3  -3.3  -3.3  -3.3  -3.3  -3.3 42

-3.2  -3.2  -3.2  -3.2  -3.2  -3.1  -3.1  -3.1  -3.1  -3.1  -3.1 40

-3.0  -3.0  -3.0  -3.0  -3.0  -3.0  -3.0  -2.9  -2.9  -2.9  -2.9 38

-2.9  -2.9  -2.9  -2.9  -2.8  -2.8  -2.8  -2.8  -2.8  -2.8  -2.8 36

-2.8  -2.8  -2.8  -2.7  -2.7  -2.7  -2.7  -2.7  -2.7  -2.7  -2.7 34

-2.7  -2.7  -2.7  -2.7  -2.7  -2.6  -2.6  -2.6  -2.6  -2.6  -2.6 32

-2.7  -2.7  -2.7  -2.6  -2.6  -2.6  -2.6  -2.6  -2.6  -2.6  -2.6 30

-2.7  -2.7  -2.7  -2.7  -2.6  -2.6  -2.6  -2.6  -2.6  -2.6  -2.6 28

-2.8  -2.8  -2.7  -2.7  -2.7  -2.7  -2.7  -2.7  -2.7  -2.7  -2.7 26

-2.9  -2.9  -2.9  -2.8  -2.8  -2.8  -2.8  -2.8  -2.8  -2.8  -2.8 24

-3.1  -3.1  -3.0  -3.0  -3.0  -3.0  -3.0  -3.0  -3.0  -3.0  -3.0 22

-3.3  -3.3  -3.3  -3.3  -3.3  -3.3  -3.2  -3.2  -3.2  -3.2  -3.2 20

-3.7  -3.7  -3.6  -3.6  -3.6  -3.6  -3.6  -3.6  -3.6  -3.6  -3.6 18

-4.1  -4.1  -4.1  -4.1  -4.1  -4.0  -4.0  -4.0  -4.0  -4.0  -4.0 16

-4.7  -4.7  -4.7  -4.7  -4.6  -4.6  -4.6  -4.6  -4.6  -4.6  -4.6 14

-5.4  -5.4  -5.4  -5.4  -5.4  -5.4  -5.4  -5.4  -5.4  -5.4  -5.3 12

-6.4  -6.4  -6.4  -6.4  -6.4  -6.4  -6.4  -6.4  -6.3  -6.3  -6.3 10

-7.8  -7.8  -7.7  -7.7  -7.7  -7.7  -7.7  -7.7  -7.7  -7.7  -7.7  8

-9.6  -9.6  -9.6  -9.6  -9.6  -9.6  -9.6  -9.6  -9.6  -9.6  -9.6  6

-12.5 -12.5 -12.5 -12.5 -12.5 -12.5 -12.5 -12.5 -12.5 -12.4 -12.4  4

-17.9 -17.9 -17.9 -17.9 -17.9 -17.8 -17.8 -17.8 -17.8 -17.8 -17.8  2

-50.6 -50.6 -50.6 -50.6 -50.6 -50.6 -50.6 -50.6 -50.6 -50.6 -50.6  0

12    13    14    16    18    20    22    24    26    28    30

FREQUENCY IN MEGAHERTZ

ANTENNA EFFICIENCY

-.6   -.6   -.6   -.6   -.6   -.6   -.6   -.6   -.6   -.6   -.6

12    13    14    16    18    20    22    24    26    28    30

FREQUENCY IN MEGAHERTZ

Figure 10a.     Receiver-antenna pattern continued.

METHOD 16   ICEPAC IC.10   PAGE  12

JAN    1970          SSN = 100.

BOULDER,COLORADO TO ST. LOUIS,MO.         AZIMUTHS          N. MI.      KM

40.03 N  105.30 W - 38.67 N   90.25 W     91.84  281.42     702.6   1301.1

ITSA 1 Antenna Package        MINIMUM ANGLE    .0 DEGREES

XMTR   2.0  TO  30.0  VER MONOPOLE H     .00 L    -.50 A     .0  OFF AZ     .0

RCVR   2.0  TO  30.0  VER MONOPOLE H     .00 L    -.25 A     .0  OFF AZ     .0

POWER =  30.000 KW  3 MHZ NOISE = -150.0 DBW   REQ. REL = .90  REQ. SNR = 55.0

MULTIPATH POWER TOLERANCE = 10.0 DB   MULTIPATH DELAY TOLERANCE =   .850 MS

UT  MUF

19.0 22.0  2.0  3.0  5.0  7.5 10.0 12.5 15.0 17.5 20.0 25.0 30.0 FREQ

1F2  1 E  1 E  1ES  2F1  1F2  1F2  1F2  1F2  1F2  1F2  1F2 MODE

23.9  3.3  3.3  6.6 26.6 27.0 22.5 21.1 21.0 21.8 23.9 23.9 ANGLE

5.0  4.4  4.4  4.4  5.0  5.2  4.9  4.9  4.9  4.9  5.0  5.0 DELAY

338.  71.  72. 110. 176. 386. 318. 297. 296. 307. 338. 338. V HITE

.50 1.00 1.00 1.00 1.00 1.00 1.00 1.00  .97  .80  .11  .00 F DAYS

141. 209. 203. 183. 162. 139. 136. 135. 134. 135. 155. 202. LOSS

41. -38. -29.  -6.  17.  37.  43.  44.  46.  47.  28. -18. DBU

-95 -162 -157 -137 -111  -92  -88  -89  -89  -88 -110 -157 S DBW

-173 -145 -150 -156 -161 -163 -163 -165 -168 -172 -175 -177 N DBW

78. -18.  -8.  19.  49.  70.  74.  76.  79.  83.  66.  20. SNR

-5.  79.  69.  42.  15.  -9. -13. -15. -18. -18.  15.  60. RPWRG

.95  .00  .00  .00  .23 1.00 1.00 1.00 1.00 1.00  .70  .05 REL

.00  .00  .00  .00  .00  .00  .00  .00  .00  .00  .00  .00 MPROB

7.0  7.6  2.0  3.0  5.0  7.5 10.0 12.5 15.0 17.5 20.0 25.0 30.0 FREQ

1F2  1 E  1F2  1F2  1F2  1F2  1ES  1ES  1F2  1F2  1F2  1F2 MODE

26.0  7.0 19.0 20.2 25.3 26.0  6.6  6.6 26.0 26.0 26.0 26.0 ANGLE

5.1  4.4  4.8  4.8  5.1  5.1  4.4  4.4  5.1  5.1  5.1  5.1 DELAY

370. 115. 268. 284. 360. 371. 110. 110. 371. 371. 371. 371. V HITE

.50  .86 1.00  .99  .52  .06  .14  .07  .00  .00  .00  .00 F DAYS

125. 120. 120. 118. 124. 144. 168. 192. 227. 228. 230. 231. LOSS

52.  47.  46.  52.  52.  32.  15.  -7. -47. -47. -47. -47. DBU

-75  -71  -73  -72  -74  -98 -122 -147 -181 -183 -184 -186 S DBW

-157 -138 -143 -149 -157 -163 -166 -169 -171 -173 -175 -177 N DBW

82.  67.  69.  77.  82.  64.  45.  23. -10. -10.  -9.  -8. SNR

-16.  -4.  -8. -16. -16.   8.  25.  57.  72.  72.  71.  70. RPWRG

1.00  .98 1.00 1.00 1.00  .76  .31  .06  .00  .00  .00  .00 REL

.00  .63  .89  .00  .00  .00  .00  .00  .00  .00  .00  .00 MPROB

Figure 11.    System performance.                     (METHOD = 16)

METHOD 17   ICEPAC IC.10   PAGE  13

JAN    1970          SSN = 100.

BOULDER,COLORADO TO ST. LOUIS,MO.         AZIMUTHS          N. MI.      KM

40.03 N  105.30 W - 38.67 N   90.25 W     91.84  281.42     702.6   1301.1

ITSA 1 Antenna Package        MINIMUM ANGLE    .0 DEGREES

XMTR   2.0  TO  30.0  VER MONOPOLE H     .00 L    -.50 A     .0  OFF AZ     .0

RCVR   2.0  TO  30.0  VER MONOPOLE H     .00 L    -.25 A     .0  OFF AZ     .0

POWER =  30.000 KW  3 MHZ NOISE = -150.0 DBW   REQ. REL = .90  REQ. SNR = 55.0

MULTIPATH POWER TOLERANCE = 10.0 DB   MULTIPATH DELAY TOLERANCE =   .850 MS

UT  MUF

19.0 22.0  2.0  3.0  5.0  7.5 10.0 12.5 15.0 17.5 20.0 25.0 30.0 FREQ

1F2  1 E  1 E  1ES  2F1  1F2  1F2  1F2  1F2  1F2  1F2  1F2 MODE

23.9  3.3  3.3  6.6 26.6 27.0 22.5 21.1 21.0 21.8 23.9 23.9 ANGLE

.50 1.00 1.00 1.00 1.00 1.00 1.00 1.00  .97  .80  .11  .00 F DAYS

41. -38. -29.  -6.  17.  37.  43.  44.  46.  47.  28. -18. DBU

78. -18.  -8.  19.  49.  70.  74.  76.  79.  83.  66.  20. SNR

.95  .00  .00  .00  .23 1.00 1.00 1.00 1.00 1.00  .70  .05 REL

7.0  7.6  2.0  3.0  5.0  7.5 10.0 12.5 15.0 17.5 20.0 25.0 30.0 FREQ

1F2  1 E  1F2  1F2  1F2  1F2  1ES  1ES  1F2  1F2  1F2  1F2 MODE

26.0  7.0 19.0 20.2 25.3 26.0  6.6  6.6 26.0 26.0 26.0 26.0 ANGLE

.50  .86 1.00  .99  .52  .06  .14  .07  .00  .00  .00  .00 F DAYS

52.  47.  46.  52.  52.  32.  15.  -7. -47. -47. -47. -47. DBU

82.  67.  69.  77.  82.  64.  45.  23. -10. -10.  -9.  -8. SNR

1.00  .98 1.00 1.00 1.00  .76  .31  .06  .00  .00  .00  .00 REL

Figure 12.    Condensed system performance,

reliabilities.                          (METHOD = 17)

METHOD 18   ICEPAC IC.10   PAGE  14

JAN    1970          SSN = 100.

BOULDER,COLORADO TO ST. LOUIS,MO.         AZIMUTHS          N. MI.      KM

40.03 N  105.30 W - 38.67 N   90.25 W     91.84  281.42     702.6   1301.1

ITSA 1 Antenna Package        MINIMUM ANGLE    .0 DEGREES

XMTR   2.0  TO  30.0  VER MONOPOLE H     .00 L    -.50 A     .0  OFF AZ     .0

RCVR   2.0  TO  30.0  VER MONOPOLE H     .00 L    -.25 A     .0  OFF AZ     .0

POWER =  30.000 KW  3 MHZ NOISE = -150.0 DBW   REQ. REL = .90  REQ. SNR = 55.0

UT  MUF

19.0 22.0  2.0  3.0  5.0  7.5 10.0 12.5 15.0 17.5 20.0 25.0 30.0 FREQ

1F2  1 E  1 E  1ES  2F1  1F2  1F2  1F2  1F2  1F2  1F2  1F2 MODE

23.9  3.3  3.3  6.6 26.6 27.0 22.5 21.1 21.0 21.8 23.9 23.9 ANGLE

.50 1.00 1.00 1.00 1.00 1.00 1.00 1.00  .97  .80  .11  .00 F DAYS

41. -38. -29.  -6.  17.  37.  43.  44.  46.  47.  28. -18. DBU

78. -18.  -8.  19.  49.  70.  74.  76.  79.  83.  66.  20. SNR

.56  .00  .00  .00  .16  .63  .76  .81  .84  .79  .31  .03 S PRB

7.0  7.6  2.0  3.0  5.0  7.5 10.0 12.5 15.0 17.5 20.0 25.0 30.0 FREQ

1F2  1 E  1F2  1F2  1F2  1F2  1ES  1ES  1F2  1F2  1F2  1F2 MODE

26.0  7.0 19.0 20.2 25.3 26.0  6.6  6.6 26.0 26.0 26.0 26.0 ANGLE

.50  .86 1.00  .99  .52  .06  .14  .07  .00  .00  .00  .00 F DAYS

52.  47.  46.  52.  52.  32.  15.  -7. -47. -47. -47. -47. DBU

82.  67.  69.  77.  82.  64.  45.  23. -10. -10.  -9.  -8. SNR

.73  .53  .64  .84  .74  .34  .13  .04  .00  .00  .00  .00 S PRB

Figure 13.    Condensed system performance,

service probability.                    (METHOD = 18)

METHOD 19   ICEPAC IC.10   PAGE  15

JAN    1970          SSN = 100.

BOULDER,COLORADO TO ST. LOUIS,MO.         AZIMUTHS          N. MI.      KM

40.03 N  105.30 W - 38.67 N   90.25 W     91.84  281.42     702.6   1301.1

MINIMUM ANGLE    .0  DEGREES

XMTR   2.0  TO  30.0  VER MONOPOLE H     .00 L    -.50 A     .0  OFF AZ     .0

UT  MUF

19.0 22.0  2.0  3.0  5.0  7.5 10.0 12.5 15.0 17.5 20.0 25.0 30.0 FREQ

1F2  1 E  1 E  1ES  2F1  1F2  1F2  1F2  1F2  1F2  1F2  1F2 MODE

23.9  3.3  3.3  6.6 26.6 27.0 22.5 21.1 21.0 21.8 23.9 23.9 ANGLE

5.0  4.4  4.4  4.4  5.0  5.2  4.9  4.9  4.9  4.9  5.0  5.0 DELAY

338.  71.  72. 110. 176. 386. 318. 297. 296. 307. 338. 338. V HITE

.50 1.00 1.00 1.00 1.00 1.00 1.00 1.00  .97  .80  .11  .00 F DAYS

7.0  7.6  2.0  3.0  5.0  7.5 10.0 12.5 15.0 17.5 20.0 25.0 30.0 FREQ

1F2  1 E  1F2  1F2  1F2  1F2  1ES  1ES  1F2  1F2  1F2  1F2 MODE

26.0  7.0 19.0 20.2 25.3 26.0  6.6  6.6 26.0 26.0 26.0 26.0 ANGLE

5.1  4.4  4.8  4.8  5.1  5.1  4.4  4.4  5.1  5.1  5.1  5.1 DELAY

370. 115. 268. 284. 360. 371. 110. 110. 371. 371. 371. 371. V HITE

.50  .86 1.00  .99  .52  .06  .14  .07  .00  .00  .00  .00 F DAYS

Figure 14.  Propagation path geometry.                (METHOD = 19)

METHOD 20   ICEPAC IC.10   PAGE  16

JAN    1970          SSN = 100.

BOULDER,COLORADO TO ST. LOUIS,MO.         AZIMUTHS          N. MI.      KM

40.03 N  105.30 W - 38.67 N   90.25 W     91.84  281.42     702.6   1301.1

ITSA 1 Antenna Package        MINIMUM ANGLE    .0 DEGREES

XMTR   2.0  TO  30.0  VER MONOPOLE H     .00 L    -.50 A     .0  OFF AZ     .0

RCVR   2.0  TO  30.0  VER MONOPOLE H     .00 L    -.25 A     .0  OFF AZ     .0

POWER =  30.000 KW  3 MHZ NOISE = -150.0 DBW   REQ. REL = .90  REQ. SNR = 55.0

MULTIPATH POWER TOLERANCE = 10.0 DB   MULTIPATH DELAY TOLERANCE =   .850 MS

UT  MUF

19.0 22.0  2.0  3.0  5.0  7.5 10.0 12.5 15.0 17.5 20.0 25.0 30.0 FREQ

1F2  1 E  1 E  1ES  2F1  1F2  1F2  1F2  1F2  1F2  1F2  1F2 MODE

23.9  3.3  3.3  6.6 26.6 27.0 22.5 21.1 21.0 21.8 23.9 23.9 ANGLE

5.0  4.4  4.4  4.4  5.0  5.2  4.9  4.9  4.9  4.9  5.0  5.0 DELAY

338.  71.  72. 110. 176. 386. 318. 297. 296. 307. 338. 338. V HITE

.50 1.00 1.00 1.00 1.00 1.00 1.00 1.00  .97  .80  .11  .00 F DAYS

141. 209. 203. 183. 162. 139. 136. 135. 134. 135. 155. 202. LOSS

41. -38. -29.  -6.  17.  37.  43.  44.  46.  47.  28. -18. DBU

-95 -162 -157 -137 -111  -92  -88  -89  -89  -88 -110 -157 S DBW

-173 -145 -150 -156 -161 -163 -163 -165 -168 -172 -175 -177 N DBW

78. -18.  -8.  19.  49.  70.  74.  76.  79.  83.  66.  20. SNR

-5.  79.  69.  42.  15.  -9. -13. -15. -18. -18.  15.  60. RPWRG

.95  .00  .00  .00  .23 1.00 1.00 1.00 1.00 1.00  .70  .05 REL

.00  .00  .00  .00  .00  .00  .00  .00  .00  .00  .00  .00 MPROB

.56  .00  .00  .00  .16  .63  .76  .81  .84  .79  .31  .03 S PRB

17.   2.   2.   2.   7.   3.   3.   3.   4.   9.  25.  25. SIG LW

8.   2.   2.   2.   4.   5.   3.   2.   2.   5.  16.  25. SIG UP

12.  10.  10.  10.  10.  10.   9.   8.   9.  10.  19.  27. SNR UP

7.0  7.6  2.0  3.0  5.0  7.5 10.0 12.5 15.0 17.5 20.0 25.0 30.0 FREQ

1F2  1 E  1F2  1F2  1F2  1F2  1ES  1ES  1F2  1F2  1F2  1F2 MODE

26.0  7.0 19.0 20.2 25.3 26.0  6.6  6.6 26.0 26.0 26.0 26.0 ANGLE

5.1  4.4  4.8  4.8  5.1  5.1  4.4  4.4  5.1  5.1  5.1  5.1 DELAY

370. 115. 268. 284. 360. 371. 110. 110. 371. 371. 371. 371. V HITE

.50  .86 1.00  .99  .52  .06  .14  .07  .00  .00  .00  .00 F DAYS

125. 120. 120. 118. 124. 144. 168. 192. 227. 228. 230. 231. LOSS

52.  47.  46.  52.  52.  32.  15.  -7. -47. -47. -47. -47. DBU

-75  -71  -73  -72  -74  -98 -122 -147 -181 -183 -184 -186 S DBW

-157 -138 -143 -149 -157 -163 -166 -169 -171 -173 -175 -177 N DBW

82.  67.  69.  77.  82.  64.  45.  23. -10. -10.  -9.  -8. SNR

-16.  -4.  -8. -16. -16.   8.  25.  57.  72.  72.  71.  70. RPWRG

1.00  .98 1.00 1.00 1.00  .76  .31  .06  .00  .00  .00  .00 REL

.00  .63  .89  .00  .00  .00  .00  .00  .00  .00  .00  .00 MPROB

.73  .53  .64  .84  .74  .34  .13  .04  .00  .00  .00  .00 S PRB

10.   3.   1.   2.  10.  16.  13.  24.   3.   3.   3.   3. SIG LW

6.   2.   4.   1.   6.  16.  25.  25.  11.   1.   1.   0. SIG UP

10.   9.   9.   7.  10.  18.  27.  27.  14.  10.  10.  10. SNR UP

Figure 15.    complete system performance.            (METHOD = 20)

METHOD 21   ICEPAC IC.10   PAGE  17

JAN    1970          SSN = 100.

BOULDER,COLORADO TO ST. LOUIS,MO.         AZIMUTHS          N. MI.      KM

40.03 N  105.30 W - 38.67 N   90.25 W     91.84  281.42     702.6   1301.1

ITSA 1 Antenna Package        MINIMUM ANGLE    .0 DEGREES

XMTR   2.0  TO  30.0  VER MONOPOLE H     .00 L    -.50 A     .0  OFF AZ     .0

RCVR   2.0  TO  30.0  VER MONOPOLE H     .00 L    -.25 A     .0  OFF AZ     .0

POWER =  30.000 KW  3 MHZ NOISE = -150.0 DBW   REQ. REL = .90  REQ. SNR = 55.0

MULTIPATH POWER TOLERANCE = 10.0 DB   MULTIPATH DELAY TOLERANCE =   .850 MS

UT  MUF

19.0 22.0  2.0  3.0  5.0  7.5 10.0 12.5 15.0 17.5 20.0 25.0 30.0 FREQ

F2F2  E E  E E  E E F1 E F1F1 F2F1 F2F2 F2F2 F2F2 F2F2 F2F2 MODE

20.0  3.0  3.0  3.0 34.0 23.8 22.0 20.0 20.0 20.0 23.9 23.9 ANGLE

20.0 18.0 18.0 22.0 16.0 23.8 12.0 20.0 20.0 20.0 23.9 23.9 ANGLE

5.3  4.4  4.4  4.5  4.8  5.4  4.9  5.1  5.0  5.1  5.0  5.0 DELAY

313.  72.  74.  85. 203. 366. 248. 301. 295. 302. 338. 338. V HITE

.50 1.00 1.00 1.00 1.00 1.00 1.00 1.00  .97  .80  .11  .00 F DAYS

141. 235. 212. 182. 143. 132. 134. 132. 133. 135. 154. 201. LOSS

42. -74. -47. -13.  31.  43.  46.  47.  48.  46.  29. -17. DBU

-95 -189 -166 -137  -98  -87  -88  -87  -87  -90 -108 -156 S DBW

-173 -145 -150 -156 -161 -163 -163 -165 -168 -172 -175 -177 N DBW

78. -44. -16.  19.  63.  76.  75.  78.  81.  82.  67.  21. SNR

-5. 106.  78.  42.  -2.  -4.   1. -17. -19. -15.  14.  59. RPWRG

.95  .00  .00  .00  .95  .94  .88 1.00 1.00 1.00  .73  .05 REL

.00  .00  .00  .00  .00  .00  .00  .00  .00  .00  .00  .00 MPROB

.56  .00  .00  .00  .46  .54  .47  .85  .85  .74  .33  .04 S PRB

17.   2.   2.   2.   3.  16.  21.   3.   5.  11.  25.  25. SIG LW

8.   2.   2.   2.   2.   7.  15.   2.   3.   4.  16.  25. SIG UP

7.0  7.6  2.0  3.0  5.0  7.5 10.0 12.5 15.0 17.5 20.0 25.0 30.0 FREQ

F2F2 F2F2 F2F2 F2F2 F2F2 F2F2 F2F2 F2F2 F2F2 F2F2 F2F2 F2F2 MODE

22.0 18.0 18.0 18.0 22.0 26.0 26.0 26.0 26.0 26.0 26.0 26.0 ANGLE

22.0 18.0 18.0 18.0 22.0 26.0 26.0 26.0 26.0 26.0 26.0 26.0 ANGLE

5.3  5.1  5.0  5.2  5.2  5.1  5.1  5.1  5.1  5.1  5.1  5.1 DELAY

334. 276. 267. 282. 332. 371. 371. 371. 371. 371. 371. 371. V HITE

.50 1.00 1.00  .99  .52  .06  .00  .00  .00  .00  .00  .00 F DAYS

134. 152. 143. 134. 133. 149. 182. 220. 229. 229. 230. 232. LOSS

39.   9.  21.  35.  39.  26.  -6. -42. -49. -48. -48. -48. DBU

-89 -107  -98  -89  -88 -103 -137 -175 -183 -184 -185 -186 S DBW

-157 -138 -143 -149 -157 -163 -166 -169 -171 -173 -175 -177 N DBW

68.  32.  45.  60.  69.  59.  29.  -6. -12. -11. -10.  -9. SNR

-2.  31.  17.   1.  -2.  15.  49.  73.  74.  73.  71.  70. RPWRG

.94  .00  .05  .84  .94  .61  .11  .00  .00  .00  .00  .00 REL

.00  .00  .00  .00  .00  .00  .00  .00  .00  .00  .00  .00 MPROB

.50  .02  .11  .41  .51  .28  .06  .00  .00  .00  .00  .00 S PRB

10.   3.   3.   4.  10.  19.  23.  11.   3.   3.   3.   3. SIG LW

6.   1.   1.   1.   6.  16.  25.  25.   2.   1.   1.   1. SIG UP

Figure 16.   forced long-path model.                  (METHOD = 21)

METHOD 22   ICEPAC IC.10   PAGE  18

JAN    1970          SSN = 100.

BOULDER,COLORADO TO ST. LOUIS,MO.         AZIMUTHS          N. MI.      KM

40.03 N  105.30 W - 38.67 N   90.25 W     91.84  281.42     702.6   1301.1

ITSA 1 Antenna Package        MINIMUM ANGLE    .0 DEGREES

XMTR   2.0  TO  30.0  VER MONOPOLE H     .00 L    -.50 A     .0  OFF AZ     .0

RCVR   2.0  TO  30.0  VER MONOPOLE H     .00 L    -.25 A     .0  OFF AZ     .0

POWER =  30.000 KW  3 MHZ NOISE = -150.0 DBW   REQ. REL = .90  REQ. SNR = 55.0

MULTIPATH POWER TOLERANCE = 10.0 DB   MULTIPATH DELAY TOLERANCE =   .850 MS

UT  MUF

19.0 22.0  2.0  3.0  5.0  7.5 10.0 12.5 15.0 17.5 20.0 25.0 30.0 FREQ

1F2  1 E  1 E  1ES  2F1  1F2  1F2  1F2  1F2  1F2  1F2  1F2 MODE

23.9  3.3  3.3  6.6 26.6 27.0 22.5 21.1 21.0 21.8 23.9 23.9 ANGLE

5.0  4.4  4.4  4.4  5.0  5.2  4.9  4.9  4.9  4.9  5.0  5.0 DELAY

338.  71.  72. 110. 176. 386. 318. 297. 296. 307. 338. 338. V HITE

.50 1.00 1.00 1.00 1.00 1.00 1.00 1.00  .97  .80  .11  .00 F DAYS

141. 209. 203. 183. 162. 139. 136. 135. 134. 135. 155. 202. LOSS

41. -38. -29.  -6.  17.  37.  43.  44.  46.  47.  28. -18. DBU

-95 -162 -157 -137 -111  -92  -88  -89  -89  -88 -110 -157 S DBW

-173 -145 -150 -156 -161 -163 -163 -165 -168 -172 -175 -177 N DBW

78. -18.  -8.  19.  49.  70.  74.  76.  79.  83.  66.  20. SNR

-5.  79.  69.  42.  15.  -9. -13. -15. -18. -18.  15.  60. RPWRG

.95  .00  .00  .00  .23 1.00 1.00 1.00 1.00 1.00  .70  .05 REL

.00  .00  .00  .00  .00  .00  .00  .00  .00  .00  .00  .00 MPROB

.56  .00  .00  .00  .16  .63  .76  .81  .84  .79  .31  .03 S PRB

17.   2.   2.   2.   7.   3.   3.   3.   4.   9.  25.  25. SIG LW

8.   2.   2.   2.   4.   5.   3.   2.   2.   5.  16.  25. SIG UP

7.0  7.6  2.0  3.0  5.0  7.5 10.0 12.5 15.0 17.5 20.0 25.0 30.0 FREQ

1F2  1 E  1F2  1F2  1F2  1F2  1ES  1ES  1F2  1F2  1F2  1F2 MODE

26.0  7.0 19.0 20.2 25.3 26.0  6.6  6.6 26.0 26.0 26.0 26.0 ANGLE

5.1  4.4  4.8  4.8  5.1  5.1  4.4  4.4  5.1  5.1  5.1  5.1 DELAY

370. 115. 268. 284. 360. 371. 110. 110. 371. 371. 371. 371. V HITE

.50  .86 1.00  .99  .52  .06  .14  .07  .00  .00  .00  .00 F DAYS

125. 120. 120. 118. 124. 144. 168. 192. 227. 228. 230. 231. LOSS

52.  47.  46.  52.  52.  32.  15.  -7. -47. -47. -47. -47. DBU

-75  -71  -73  -72  -74  -98 -122 -147 -181 -183 -184 -186 S DBW

-157 -138 -143 -149 -157 -163 -166 -169 -171 -173 -175 -177 N DBW

82.  67.  69.  77.  82.  64.  45.  23. -10. -10.  -9.  -8. SNR

-16.  -4.  -8. -16. -16.   8.  25.  57.  72.  72.  71.  70. RPWRG

1.00  .98 1.00 1.00 1.00  .76  .31  .06  .00  .00  .00  .00 REL

.00  .63  .89  .00  .00  .00  .00  .00  .00  .00  .00  .00 MPROB

.73  .53  .64  .84  .74  .34  .13  .04  .00  .00  .00  .00 S PRB

10.   3.   1.   2.  10.  16.  13.  24.   3.   3.   3.   3. SIG LW

6.   2.   4.   1.   6.  16.  25.  25.  11.   1.   1.   0. SIG UP

Figure 17.    forced short-path model.                (MODEL = 22)

METHOD 23   ICEPAC IC.10   PAGE  19

JAN    1970          SSN = 100.

BOULDER,COLORADO TO ST. LOUIS,MO.         AZIMUTHS          N. MI.      KM

40.03 N  105.30 W - 38.67 N   90.25 W     91.84  281.42     702.6   1301.1

ITSA 1 Antenna Package        MINIMUM ANGLE    .0 DEGREES

XMTR   2.0  TO  30.0  VER MONOPOLE H     .00 L    -.50 A     .0  OFF AZ     .0

RCVR   2.0  TO  30.0  VER MONOPOLE H     .00 L    -.25 A     .0  OFF AZ     .0

POWER =  30.000 KW  3 MHZ NOISE = -150.0 DBW   REQ. REL = .90  REQ. SNR = 55.0

UT  MUF

19.0 22.0  2.0  3.0  5.0  7.5 10.0 12.5 15.0 17.5 20.0 25.0 30.0 FREQ

1F2  1 E  1 E  1ES  2F1  1F2  1F2  1F2  1F2  1F2  1F2  1F2 MODE

23.9  3.3  3.3  6.6 26.6 27.0 22.5 21.1 21.0 21.8 23.9 23.9 ANGLE

338.  71.  72. 110. 176. 386. 318. 297. 296. 307. 338. 338. V HITE

78. -18.  -8.  19.  49.  70.  74.  76.  79.  83.  66.  20. SNR

-5.  79.  69.  42.  15.  -9. -13. -15. -18. -18.  15.  60. RPWRG

.95  .00  .00  .00  .23 1.00 1.00 1.00 1.00 1.00  .70  .05 REL

7.0  7.6  2.0  3.0  5.0  7.5 10.0 12.5 15.0 17.5 20.0 25.0 30.0 FREQ

1F2  1 E  1F2  1F2  1F2  1F2  1ES  1ES  1F2  1F2  1F2  1F2 MODE

26.0  7.0 19.0 20.2 25.3 26.0  6.6  6.6 26.0 26.0 26.0 26.0 ANGLE

370. 115. 268. 284. 360. 371. 110. 110. 371. 371. 371. 371. V HITE

82.  67.  69.  77.  82.  64.  45.  23. -10. -10.  -9.  -8. SNR

-16.  -4.  -8. -16. -16.   8.  25.  57.  72.  72.  71.  70. RPWRG

1.00  .98 1.00 1.00 1.00  .76  .31  .06  .00  .00  .00  .00 REL

Figure 18.    User-selected system performance.       (METHOD = 23)

METHOD 24   ICEPAC IC.10   PAGE  20

JAN    1970          SSN = 100.

BOULDER,COLORADO TO ST. LOUIS,MO.         AZIMUTHS          N. MI.      KM

40.03 N  105.30 W - 38.67 N   90.25 W     91.84  281.42     702.6   1301.1

ITSA 1 Antenna Package        MINIMUM ANGLE    .0 DEGREES

XMTR   2.0  TO  30.0  VER MONOPOLE H     .00 L    -.50 A     .0  OFF AZ     .0

RCVR   2.0  TO  30.0  VER MONOPOLE H     .00 L    -.25 A     .0  OFF AZ     .0

POWER =  30.000 KW  3 MHZ NOISE = -150.0 DBW   REQ. REL = .90  REQ. SNR = 55.0

FREQUENCY / RELIABILITY

LT  MUF  2.0  3.0  5.0  7.5 10.0 12.5 15.0 17.5 20.0 25.0 30.0  MUF

1  8.2  .98 1.00 1.00 1.00  .91  .34  .02  .00  .00  .00  .00 1.00

2  8.1  .99 1.00 1.00 1.00  .64  .11  .00  .00  .00  .00  .00  .98

3  7.5  .98 1.00 1.00  .93  .37  .04  .00  .00  .00  .00  .00  .93

4  6.4  .97 1.00 1.00  .69  .28  .02  .00  .00  .00  .00  .00  .89

5  6.3  .93 1.00 1.00  .64  .29  .03  .00  .00  .00  .00  .00  .88

6  8.2  .92  .97 1.00 1.00  .38  .07  .00  .00  .00  .00  .00  .89

7 12.2  .12  .21  .99 1.00 1.00  .76  .14  .00  .00  .00  .00  .82

8 16.4  .00  .01  .33 1.00 1.00 1.00  .99  .71  .19  .00  .00  .87

9 19.5  .00  .00  .09 1.00 1.00 1.00 1.00 1.00  .82  .05  .00  .86

10 20.4  .00  .00  .00  .24 1.00 1.00 1.00 1.00  .97  .37  .00  .93

11 21.2  .00  .00  .00  .26 1.00 1.00 1.00 1.00  .99  .56  .01  .95

12 22.0  .00  .00  .00  .23 1.00 1.00 1.00 1.00 1.00  .70  .05  .95

13 21.8  .00  .00  .00  .19 1.00 1.00 1.00 1.00 1.00  .66  .04  .96

14 21.8  .00  .00  .08  .99 1.00 1.00 1.00 1.00 1.00  .64  .03  .93

15 20.9  .00  .01  .36  .98  .99  .99 1.00 1.00  .97  .46  .00  .87

16 19.3  .15  .22  .93  .98  .99 1.00 1.00  .99  .81  .15  .00  .86

17 16.7  .47  .50  .95  .99 1.00 1.00 1.00  .77  .42  .00  .00  .93

18 13.9  .59  .87  .99 1.00 1.00 1.00  .91  .67  .38  .02  .00  .96

19 11.6  .65  .91  .99 1.00 1.00  .91  .60  .26  .04  .00  .00  .96

20  9.6  .82  .97 1.00 1.00  .95  .62  .20  .02  .00  .00  .00  .97

21  8.0  .98 1.00 1.00 1.00  .81  .34  .10  .00  .00  .00  .00 1.00

22  7.0  .97 1.00 1.00  .97  .55  .33  .13  .00  .00  .00  .00 1.00

23  7.0  .97 1.00 1.00  .97  .58  .34  .12  .00  .00  .00  .00 1.00

24  7.6  .98 1.00 1.00 1.00  .76  .31  .06  .00  .00  .00  .00 1.00

Figure 19.    Reliability table output.               (METHOD = 24)

METHOD 25   ICEPAC IC.10   PAGE  21

JAN    1970          SSN = 100.

BOULDER,COLORADO TO ST. LOUIS,MO.         AZIMUTHS          N. MI.      KM

40.03 N  105.30 W - 38.67 N   90.25 W     91.84  281.42     702.6   1301.1

ITSA 1 Antenna Package        MINIMUM ANGLE    .0 DEGREES

XMTR   2.0  TO  30.0  VER MONOPOLE H     .00 L    -.50 A     .0  OFF AZ     .0

RCVR   2.0  TO  30.0  VER MONOPOLE H     .00 L    -.25 A     .0  OFF AZ     .0

POWER =  30.000 KW  3 MHZ NOISE = -150.0 DBW   REQ. REL = .90  REQ. SNR = 55.0

YE =  20.0     HE = 110.0     HS = 110.0

LAT   LONG    LMT    UT    E     F1    Y1     H1   FH/2 F2Z   Y2

H2     PB    CEN    EB M3000   TCGM   RAT    ZEN FLAG   MAGL

39.6N  97.7W  12.5  19.0  3.46    4.6  50.0  200.0   .7  11.4  95.9

283.9   77.5   76.7  76.0  3.11   12.1   3.0   61.3    6  50.1N

FREQ =   3.0 MHZ  UT =  19.0

1. E      2. E      3. E      1. E

TIME DEL.     4.389     4.473     4.631     4.389

ANGLE         3.315    11.191    18.503     3.315

VIR. HITE    71.602    73.468    77.170    71.602

TRAN.LOSS   203.441   216.021   231.070   203.441

T. GAIN     -11.082    -3.442    -2.002   -11.082

R. GAIN      -13.177    -5.186    -3.081   -13.177

ABSORB       71.177    48.621    37.135

FS. LOSS    104.376   104.542   104.844

FIELD ST.   -28.750   -49.321   -66.475   -28.711

SIG. POW.  -158.000  -171.000  -186.000  -157.000

SNR          -8.108   -20.688   -35.737    -7.891

MODE PROB     1.000     1.000     1.000     1.000

R. PWRG     1000.000  1000.000  1000.000    69.371

RELIABIL        .000      .000      .000      .000

SERV PROB      .000      .000      .000      .000

SIG LOW       2.460     2.460     2.460     2.460

SIG  UP      2.256     2.256     2.256     2.256

NOISE =   -150      S. POWER =   -157

SIGNAL =    2.5    5.1    2.3  /    2.3    3.3     .7

NOISE =     9.7 -150.6    6.0  /     1.5     5.4     1.5

RELIAB =   10.0   -7.9    6.5

SPROB =    29.3  -10.1   29.3

Figure 20.    All modes output (user defined freq.)   (METHOD = 25)

METHOD 25   ICEPAC IC.10   PAGE  22

JAN    1970          SSN = 100.

BOULDER,COLORADO TO ST. LOUIS,MO.         AZIMUTHS          N. MI.      KM

40.03 N  105.30 W - 38.67 N   90.25 W     91.84  281.42     702.6   1301.1

ITSA 1 Antenna Package        MINIMUM ANGLE    .0 DEGREES

XMTR   2.0  TO  30.0  VER MONOPOLE H     .00 L    -.50 A     .0  OFF AZ     .0

RCVR   2.0  TO  30.0  VER MONOPOLE H     .00 L    -.25 A     .0  OFF AZ     .0

POWER =  30.000 KW  3 MHZ NOISE = -150.0 DBW   REQ. REL = .90  REQ. SNR = 55.0

FREQ =  22.0 MHZ  UT =  19.0

1.F2      1.ES      2.ES      1.F2

TIME DEL.     4.990     4.437     4.616     4.990

ANGLE        23.893     6.583    17.069    23.893

VIR. HITE   337.903   110.000   110.000   337.903

TRAN.LOSS   140.511 41260.370 82370.950   140.511

T. GAIN       -.734    -5.604    -1.026     -.734

R. GAIN       -2.819    -8.982    -3.786    -2.819

ABSORB        2.792     5.835     3.598

FS. LOSS    122.808   121.787   122.132

FIELD ST.    41.138 ********* *********    41.139

SIG. POW.   -95.000 ********* *********   -95.000

SNR          78.241 ********* *********    78.240

MODE PROB      .500      .000      .000      .500

R. PWRG     1000.000  1000.000  1000.000    -5.335

RELIABIL        .952      .000      .000      .952

SERV PROB      .000      .001      .001      .000

SIG LOW      17.053     2.460     2.460    17.053

SIG  UP      7.595    25.000     2.256     7.595

NOISE =   -173      S. POWER =    -95

SIGNAL =    2.5    5.1    2.3  /    2.3    3.3     .7

NOISE =     9.5 -174.0    5.5  /     1.4     4.8     1.6

RELIAB =   12.2   78.2   17.9

SPROB =    29.3  -10.1   29.3

Figure 21.    All modes output (MUF)                  (METHOD = 25)

METHOD 26   ICEPAC IC.10   PAGE  23

JAN    1970          SSN = 100.

BOULDER,COLORADO TO ST. LOUIS,MO.         AZIMUTHS          N. MI.      KM

40.03 N  105.30 W - 38.67 N   90.25 W     91.84  281.42     702.6   1301.1

ITSA 1 Antenna Package        MINIMUM ANGLE    .0 DEGREES

XMTR   2.0  TO  30.0  VER MONOPOLE H     .00 L    -.50 A     .0  OFF AZ     .0

RCVR   2.0  TO  30.0  VER MONOPOLE H     .00 L    -.25 A     .0  OFF AZ     .0

POWER =  30.000 KW  3 MHZ NOISE = -150.0 DBW   REQ. REL = .90  REQ. SNR = 55.0

UT    LT    FOT    HPF  ESMUF    MUF    LUF

1.0  18.0  11.00  17.82   5.68  13.92   3.47

3.0  20.0   7.57  12.26   5.45   9.58   2.29

5.0  22.0   5.85   8.94   5.72   7.04   2.00

7.0  24.0   6.28   9.60   5.93   7.56   2.00

9.0   2.0   6.60   9.53   5.75   8.15   2.00

11.0   4.0   5.19   7.50   5.42   6.41   2.00

13.0   6.0   6.97   9.18   6.04   8.19   2.00

15.0   8.0  13.93  18.35   8.19  16.39   7.11

17.0  10.0  17.57  23.28  10.60  20.43   8.33

19.0  12.0  18.94  25.11  11.15  22.03   8.75

21.0  14.0  18.74  24.84   9.41  21.79   8.76

23.0  16.0  16.63  22.05   7.05  19.34   4.67

Figure 22.    LUF-MUF table.                          (METHOD = 26)

METHOD 27   ICEPAC IC.10   PAGE  24

JAN    1970          SSN = 100.

BOULDER,COLORADO TO ST. LOUIS,MO.         AZIMUTHS          N. MI.      KM

40.03 N  105.30 W - 38.67 N   90.25 W     91.84  281.42     702.6   1301.1

ITSA 1 Antenna Package        MINIMUM ANGLE    .0 DEGREES

XMTR   2.0  TO  30.0  VER MONOPOLE H     .00 L    -.50 A     .0  OFF AZ     .0

RCVR   2.0  TO  30.0  VER MONOPOLE H     .00 L    -.25 A     .0  OFF AZ     .0

POWER =  30.000 KW  3 MHZ NOISE = -150.0 DBW   REQ. REL = .90  REQ. SNR = 55.0

LUF(....)       FOT(XXXX)

00  02  04  06  08  10  12  14  16  18  20  22  00

MHZ+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+MHZ

40-                                                   -40

-                                                   -

38-                                                   -38

-                                                   -

36-                                                   -36  GMT   LUF   FOT

-                                                   -

34-                                                   -34  1.0   3.5  11.0

-                                                   -    3.0   2.3   7.6

32-                                                   -32  5.0   2.0   5.8

-                                                   -    7.0   2.0   6.3

30-                                                   -30  9.0   2.0   6.6

-                                                   -   11.0   2.0   5.2

28-                                                   -28 13.0   2.0   7.0

-                                                   -   15.0   7.1  13.9

26-                                                   -26 17.0   8.3  17.6

-                                                   -   19.0   8.8  18.9

24-                                                   -24 21.0   8.8  18.7

-                                                   -   23.0   4.7  16.6

22-                                                   -22

-                                                   -

20-                                                   -20

-                                       X   X       -

18-                                   X               -18

-                                               X   -

16-                                                   -16

-                                                   -

14-                               X                   -14

-                                                   -

12-                                                   -12

-   X                                               -

10-                                                   -10

-                                       .   .       -

08-       X                           .               -08

-                   X       X   .                   -

06-           X   X                                   -06

-                       X                       .   -

04-                                                   -04

-   .                                               -

02-       .   .   .   .   .   .                       -02

-                                                   -

MHZ+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+MHZ

00  02  04  06  08  10  12  14  16  18  20  22  00

UNIVERSAL TIME

Figure 23.    LUF-FOT graph.                          (METHOD = 27)

METHOD 28   ICEPAC IC.10   PAGE  25

JAN    1970          SSN = 100.

BOULDER,COLORADO TO ST. LOUIS,MO.         AZIMUTHS          N. MI.      KM

40.03 N  105.30 W - 38.67 N   90.25 W     91.84  281.42     702.6   1301.1

ITSA 1 Antenna Package        MINIMUM ANGLE    .0 DEGREES

XMTR   2.0  TO  30.0  VER MONOPOLE H     .00 L    -.50 A     .0  OFF AZ     .0

RCVR   2.0  TO  30.0  VER MONOPOLE H     .00 L    -.25 A     .0  OFF AZ     .0

POWER =  30.000 KW  3 MHZ NOISE = -150.0 DBW   REQ. REL = .90  REQ. SNR = 55.0

MUF(....)       FOT(XXXX)       LUF(++++)

00  02  04  06  08  10  12  14  16  18  20  22  00

MHZ+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+MHZ

40-                                                   -40

-                                                   -

38-                                                   -38

-                                                   -

36-                                                   -36  GMT   MUF   FOT   LUF

-                                                   -

34-                                                   -34  1.0  13.9  11.0   3.5

-                                                   -    3.0   9.6   7.6   2.3

32-                                                   -32  5.0   7.0   5.8   2.0

-                                                   -    7.0   7.6   6.3   2.0

30-                                                   -30  9.0   8.1   6.6   2.0

-                                                   -   11.0   6.4   5.2   2.0

28-                                                   -28 13.0   8.2   7.0   2.0

-                                                   -   15.0  16.4  13.9   7.1

26-                                                   -26 17.0  20.4  17.6   8.3

-                                                   -   19.0  22.0  18.9   8.8

24-                                                   -24 21.0  21.8  18.7   8.8

-                                                   -   23.0  19.3  16.6   4.7

22-                                       .   .       -22

-                                                   -

20-                                   .               -20

-                                       X   X   .   -

18-                                   X               -18

-                                               X   -

16-                               .                   -16

-                                                   -

14-   .                           X                   -14

-                                                   -

12-                                                   -12

-   X                                               -

10-       .                                           -10

-                                       +   +       -

08-       X       .   .       .       +               -08

-           .       X       X   +                   -

06-           X   X       .                           -06

-                       X                       +   -

04-                                                   -04

-   +                                               -

02-       +   +   +   +   +   +                       -02

-                                                   -

MHZ+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+MHZ

00  02  04  06  08  10  12  14  16  18  20  22  00

UNIVERSAL TIME

Figure 24.    LUF MUF FOT graph.                      (METHOD = 28)

METHOD 29   ICEPAC IC.10   PAGE  26

JAN    1970          SSN = 100.

BOULDER,COLORADO TO ST. LOUIS,MO.         AZIMUTHS          N. MI.      KM

40.03 N  105.30 W - 38.67 N   90.25 W     91.84  281.42     702.6   1301.1

ITSA 1 Antenna Package        MINIMUM ANGLE    .0 DEGREES

XMTR   2.0  TO  30.0  VER MONOPOLE H     .00 L    -.50 A     .0  OFF AZ     .0

RCVR   2.0  TO  30.0  VER MONOPOLE H     .00 L    -.25 A     .0  OFF AZ     .0

POWER =  30.000 KW  3 MHZ NOISE = -150.0 DBW   REQ. REL = .90  REQ. SNR = 55.0

MUF(....)       LUF(XXXX)

00  02  04  06  08  10  12  14  16  18  20  22  00

MHZ+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+MHZ

40-                                                   -40

-                                                   -

38-                                                   -38

-                                                   -

36-                                                   -36  GMT   MUF   LUF

-                                                   -

34-                                                   -34  1.0  13.9   3.5

-                                                   -    3.0   9.6   2.3

32-                                                   -32  5.0   7.0   2.0

-                                                   -    7.0   7.6   2.0

30-                                                   -30  9.0   8.1   2.0

-                                                   -   11.0   6.4   2.0

28-                                                   -28 13.0   8.2   2.0

-                                                   -   15.0  16.4   7.1

26-                                                   -26 17.0  20.4   8.3

-                                                   -   19.0  22.0   8.8

24-                                                   -24 21.0  21.8   8.8

-                                                   -   23.0  19.3   4.7

22-                                       .   .       -22

-                                                   -

20-                                   .               -20

-                                               .   -

18-                                                   -18

-                                                   -

16-                               .                   -16

-                                                   -

14-   .                                               -14

-                                                   -

12-                                                   -12

-                                                   -

10-       .                                           -10

-                                       X   X       -

08-               .   .       .       X               -08

-           .                   X                   -

06-                       .                           -06

-                                               X   -

04-                                                   -04

-   X                                               -

02-       X   X   X   X   X   X                       -02

-                                                   -

MHZ+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+MHZ

00  02  04  06  08  10  12  14  16  18  20  22  00

UNIVERSAL TIME

Figure 25.    LUF MUF graph.                          (METHOD = 29)

METHOD  7   ICEPAC IC.10   PAGE  27

JAN    1970          SSN = 100.

BOULDER,COLORADO TO AUCKLAND,N. Z.        AZIMUTHS          N. MI.      KM

40.03 N  105.30 W - 36.92 S  174.75 E    235.07   51.75    6377.8  11810.7

MINIMUM ANGLE    .0  DEGREES

XMTR   2.0  TO  30.0  VER MONOPOLE H     .00 L    -.50 A     .0  OFF AZ     .0

UT    LT            FOT   MUF   HPF ANGLE VIRTL  TRUE  FVERT

13.0   6.0  E-LAYER   4.7   5.4   6.1   1.7  148.  103.   1.0

F1LAYER   4.7   5.4   6.1   1.7  148.  103.   1.0

F2LAYER   8.6  11.3  14.8   2.1  395.  292.   3.3

ESLAYER   3.0   4.7  10.6   1.9  110.  110.    .9

19.0  12.0  E-LAYER  13.0  14.9  16.8   1.7  147.  103.   2.7

F1LAYER  13.0  14.9  16.8   1.7  147.  103.   2.7

F2LAYER  25.5  29.3  33.4   2.8  424.  293.   8.7

ESLAYER   9.4  13.6  17.7   1.9  110.  110.   2.5

1.0  18.0  E-LAYER   6.8   7.8   8.8   1.7  147.  103.   1.4

F1LAYER   6.8   7.8   8.8   1.7  147.  103.   1.4

F2LAYER  24.9  29.2  34.5   1.5  372.  271.   8.3

ESLAYER   5.0   8.2  15.1   1.9  110.  110.   1.5

Figure 26.    MUF complete output table

(long path example).                    (METHOD = 7)

METHOD 23   ICEPAC IC.10   PAGE  28

JAN    1970          SSN = 100.

BOULDER,COLORADO TO AUCKLAND,N. Z.        AZIMUTHS          N. MI.      KM

40.03 N  105.30 W - 36.92 S  174.75 E    235.07   51.75    6377.8  11810.7

ITSA 1 Antenna Package        MINIMUM ANGLE    .0 DEGREES

XMTR   2.0  TO  30.0  VER MONOPOLE H     .00 L    -.50 A     .0  OFF AZ     .0

RCVR   2.0  TO  30.0  VER MONOPOLE H     .00 L    -.25 A     .0  OFF AZ     .0

POWER =  30.000 KW  3 MHZ NOISE = -150.0 DBW   REQ. REL = .90  REQ. SNR = 55.0

UT  MUF

13.0 11.3  2.0  3.0  5.0  7.5 10.0 12.5 15.0 17.5 20.0 25.0 30.0 FREQ

F2F2  EF2  EF2 F2F2 F2F2 F2F2 F2F2 F2F2 F2F2 F2F2 F2F2 F2F2 MODE

3.0  3.0  4.0 10.0 12.0 10.0  2.1  2.1  2.1  2.1  2.1  2.1 ANGLE

14.0 12.0 12.0 12.0 12.0 14.0 14.0  3.0  3.0  3.0  4.3  2.1 ANGLE

374. 206. 197. 301. 318. 365. 373. 372. 380. 389. 484. 395. V HITE

36. -23.  -7.  26.  39.  45.  30.   4. -23. -57. **** **** SNR

45.  85.  69.  36.  32.  36.  51.  77. 104. 138. 182. 198. RPWRG

.16  .00  .00  .00  .01  .22  .11  .01  .00  .00  .00  .00 REL

19.0 29.3  2.0  3.0  5.0  7.5 10.0 12.5 15.0 17.5 20.0 25.0 30.0 FREQ

F2F2  E E  E E  E E  E E  EF2 F1F2 F1F2 F2F2 F2F2 F2F2 F2F2 MODE

13.7   .5  1.0  1.0  1.0 18.0 13.0 12.8  8.5 12.0 12.0 12.8 ANGLE

5.0  1.0  1.0  1.0  4.0 12.2 10.0 10.0 12.0 12.0  8.0  3.0 ANGLE

451.  71.  72.  74.  82. 238. 231. 338. 415. 314. 335. 455. V HITE

39. **** **** **** **** -73. -31.  10.  27.  31.  41.  33. SNR

42. **** 926. 550. 350. 137. 110.  70.  36.  33.  39.  48. RPWRG

.19  .00  .00  .00  .00  .00  .00  .02  .00  .00  .06  .14 REL

1.0 29.2  2.0  3.0  5.0  7.5 10.0 12.5 15.0 17.5 20.0 25.0 30.0 FREQ

F2F2  E E  E E F2 E F2 E F2 E F2 E F2F1 F2F1 F2F2 F2F2 F2F2 MODE

3.0  3.0  3.0 13.3  8.0 18.0 18.0 10.0 10.0 10.0 10.0  1.0 ANGLE

16.0   .5   .5   .5  1.0  1.0 16.0 14.0 12.0 10.0 14.0 17.5 ANGLE

413.  71.  74. 182. 161. 166. 210. 227. 275. 324. 355. 534. V HITE

35. **** **** **** **** **** -48.  -9.  15.  29.  42.  27. SNR

46. **** **** 456. 284. 216. 112.  73.  49.  37.  39.  54. RPWRG

.16  .00  .00  .00  .00  .00  .00  .00  .00  .00  .10  .08 REL

Figure 27.    User selected system performance

(long path example).              (METHOD = 23)

ICEPAC  Version IC.10

1         2         3         4         5         6         7         8

12345678901234567890123456789012345678901234567890123456789012345678901234567890

COMMENT   **********************************************************************

COMMENT   ITSA-1 Antenna Package

COMMENT   **********************************************************************

METHOD       13

COMMENT

COMMENT   Terminated Rhombic Transmitter Antenna

COMMENT   Bearing of Antenna = 0 Degrees East of North,

COMMENT   Ground Conductivity = 0.001 MHOS/M, Dielectric Constant = 4,

COMMENT   Tilt Angle (Half of Large Interior Angle) = 67.5 Degrees,

COMMENT   Antenna Leg Length = 88.39 Meters, Antenna Height = 16.76 Meters,

COMMENT   Ending Frequency = 30 Mhz (Default) , Number of Antennas = 1 (Default)

ANTENNA       1    1      .001  4. 67.5 88.3916.76

EXECUTE

COMMENT

COMMENT   Vertical Monopole Receiver Antenna

COMMENT   Ground Conductivity = 0.001 MHOS/M, Dielectric Constant = 4,

COMMENT   Antenna Height = 1/4 Wavelength, Gain Above Diploe = 0 db,

COMMENT   Ending Frequency = 30 Mhz , Number of Antennas = 1

ANTENNA       2    2      .001  4.      -0.25                          30.    1

EXECUTE

COMMENT

COMMENT   Horizontal Dipole Transmitter Antenna

COMMENT   Bearing of Antenna = 0 Degrees East of North,

COMMENT   Ground Conductivity = 0.001 MHOS/M, Dielectric Constant = 4,

COMMENT   Antenna Length = 1/2 Wavelength, Antenna Height = 1/4 Wavelength,

COMMENT   Gain  Above Half Wavelength Horizontal Dipole = 0.0 db,

COMMENT   Ending Frequency = 30 Mhz , Number of Antennas = 1

ANTENNA       1    3      .001  4.      -0.5 -0.25                     30.    1

EXECUTE

COMMENT   Horizontal Yagi Receiver Antenna

COMMENT   Bearing of Antenna = 0 Degrees East of North,

COMMENT   Ground Conductivity = 0.001 MHOS/M, Dielectric Constant = 4,

COMMENT   Antenna Length = 1/2 Wavelength, Antenna Height = 1/4 Wavelength,

COMMENT   Gain  Above Half Wavelength Horizontal Dipole = 3.0 db,

COMMENT   Ending Frequency = 30 Mhz (Default) , Number of Antennas = 1 (Default)

ANTENNA       2    4      .001  4.      -0.5 -.25  3.0

EXECUTE

COMMENT

COMMENT   Vertical Log Periodic Array of Monopoles Transmitter Antenna

COMMENT   Ground Conductivity = 0.001 MHOS/M, Dielectric Constant = 4,

COMMENT   Antenna Height = 1/4 Wavelength,

COMMENT   Gain Above Quarter Wavelength Vertical Monopole = 2 db,

COMMENT   Ending Frequency = 30 Mhz (Default) , Number of Antennas = 1 (Default)

ANTENNA       1    5      .001  4.      -0.25      2.

EXECUTE

COMMENT   Curtain Receiver Antenna

COMMENT   Bearing of Antenna = 0 Degrees East of North,

COMMENT   Ground Conductivity = 0.001 MHOS/M, Dielectric Constant = 4,

COMMENT   Number of Bays = 2 Antenna Element Length = 22 Meters,

COMMENT   Height to 1st Element = 16 Meters, Number of Elements Per Bay = 4,

COMMENT   Distance Between Element Centers = 26 Meters,

COMMENT   Vertical Spacing of Elements=13 Meters, Distance From Screen=7 Meters,

COMMENT   Ending Frequency = 30 Mhz , Number of Antennas = 1

ANTENNA       2    6      .001  4.   2.0 22.0 16.0  4.0 26.0 13.0  7.0  30.    1

EXECUTE

Figure 28.  Antenna pattern input commands.

COMMENT

COMMENT   Terminated Sloping Vee Transmitter Antenna

COMMENT   Bearing of Antenna = 0 Degrees East of North,

COMMENT   Ground Conductivity = 0.001 MHOS/M, Dielectric Constant = 4,

COMMENT   Half Apex Angle = 22.5 Degrees, Antenna Leg Length = 121.9 Meters,

COMMENT   Antenna Height = 15.24 Meters, Terminated Height = 1.829 Meters,

COMMENT   Ending Frequency = 30 Mhz , Number of Antennas = 1

ANTENNA       1    7      .001  4.  22.5121.915.241.829                  30.   1

EXECUTE

COMMENT

COMMENT   Inverted L Receiver Antenna

COMMENT   Bearing of Antenna = 0 Degrees East of North,

COMMENT   Ground Conductivity = 0.001 MHOS/M, Dielectric Constant = 4,

COMMENT   Antenna Length = 21.34 Meters, Antenna Height = 10 Meters,

COMMENT   Ending Frequency = 30 Mhz , Number of Antennas = 1

COMMENT

ANTENNA       2    8      .001  4.      21.34 10.0                       30.   1

EXECUTE

COMMENT

COMMENT   Terminated Sloping Rhombic Transmitter Antenna

COMMENT   Bearing of Antenna = 0 Degrees East of North,

COMMENT   Ground Conductivity = 0.001 MHOS/M, Dielectric Constant = 4,

COMMENT   Half Large Interior Angle = 22.5 Degrees, Leg Length = 88.39 Meters,

COMMENT   Antenna Height = 16.76 Meters, Terminated Height = 8.382 Meters,

COMMENT   Ending Frequency = 30 Mhz (Default) , Number of Antennas = 1 (Default)

ANTENNA       1    9      .001  4.  22.588.3916.768.382

EXECUTE

COMMENT

COMMENT   Interlaced Rhombic Receiver Antenna

COMMENT   Bearing of Antenna = 0 Degrees East of North,

COMMENT   Ground Conductivity = 0.001 MHOS/M, Dielectric Constant = 4,

COMMENT   Half Large Interior Angle = 70 Degrees, Leg Length = 114 Meters,

COMMENT   Lower Antenna Height = 20 Meters, Vertical Displacement = 4 Meters,

COMMENT   Ending Frequency = 30 Mhz (Default) , Number of Antennas = 1 (Default)

ANTENNA       2   11      .001  4.  70.0114.0 20.0  4.0 33.0

EXECUTE

COMMENT

COMMENT   Constant Gain Transmitter Antenna

COMMENT   Gain Above an Isotropic = 10 db,

COMMENT   Ground Conductivity = 0.001 MHOS/M, Dielectric Constant = 4,

COMMENT   Antenna Efficiency = 0 db,

COMMENT   Ending Frequency = 30 Mhz (Default) , Number of Antennas = 1 (Default)

ANTENNA       1   12 10.  .001  4.             0.

EXECUTE

COMMENT

COMMENT   Constant Gain Receiver Antenna

COMMENT   Gain Above an Isotropic = 10 db,

COMMENT   Ground Conductivity = 0.001 MHOS/M, Dielectric Constant = 4,

COMMENT   Antenna Efficiency = -1.9 db,

COMMENT   Ending Frequency = 30 Mhz (Default) , Number of Antennas = 1 (Default)

ANTENNA       2   12 10.  .001  4.           -1.9

EXECUTE

QUIT

Figure 28a.  Antenna pattern input commands (continued).

0

METHOD 13   ICEPAC IC.10   PAGE   1

ITSA 1 ANTENNA PACKAGE             ANTENNA PATTERN

FREQUENCY RANGE  ANTENNA TYPE      HEIGHT    LENGTH     ANGLE

2.0 TO  30.0   TER. RHOMBIC      16.760    88.390    67.500

2     3     4     5     6     7     8     9    10    11

90  -3.7 -11.7 -11.7  -2.4 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7

88  -3.1 -11.7 -11.7  -2.5  -7.7 -11.7 -11.7  -8.8 -11.7 -11.7

86  -2.7 -11.7 -11.7  -3.0  -5.4 -11.7 -11.7  -8.2 -11.7 -11.7

84  -2.2 -11.7 -11.7  -4.0  -3.7 -11.7 -11.7  -8.8  -8.0 -11.7

82  -1.9  -7.8 -11.7  -5.4  -2.7 -11.7 -11.7 -11.7  -5.8 -11.7

80  -1.5  -5.9 -11.7  -7.4  -2.1  -9.5 -11.7 -11.7  -5.3  -7.6

78  -1.3  -4.2 -11.7 -11.7  -2.1  -6.4 -11.7 -11.7  -6.3  -4.0

76  -1.1  -2.8 -11.7 -11.7  -2.7  -4.3 -11.7 -11.7  -9.1  -2.6

74   -.9  -1.6 -11.7 -11.7  -3.8  -3.0 -11.7 -11.7 -11.7  -3.1

72   -.8   -.5 -11.7 -11.7  -5.6  -2.3  -8.0 -11.7 -11.7  -5.4

E 70   -.8    .5  -8.0 -11.7  -8.3  -2.4  -5.4 -11.7 -11.7 -11.7

L 68   -.8   1.2  -5.1 -11.7 -11.7  -3.1  -3.7 -11.7 -11.7 -11.7

E 66   -.8   1.9  -2.7 -11.7 -11.7  -4.6  -2.9  -7.3 -11.7 -11.7

V 64   -.9   2.5   -.7 -11.7 -11.7  -7.0  -2.9  -5.1 -11.7 -11.7

A 62  -1.1   2.9    .9  -8.9 -11.7 -11.7  -3.7  -3.8  -7.1 -11.7

T 60  -1.3   3.2   2.3  -4.9 -11.7 -11.7  -5.4  -3.4  -5.0  -8.4

I 58  -1.6   3.5   3.4  -1.8 -11.7 -11.7  -8.4  -3.9  -3.9  -5.0

O 56  -1.9   3.6   4.4    .7  -9.9 -11.7 -11.7  -5.4  -3.8  -3.4

N 54  -2.2   3.7   5.1   2.7  -4.9 -11.7 -11.7  -8.2  -4.5  -3.0

52  -2.6   3.7   5.7   4.3  -1.1 -11.7 -11.7 -11.7  -6.3  -3.7

A 50  -3.1   3.6   6.2   5.6   1.8  -7.0 -11.7 -11.7  -9.5  -5.3

N 48  -3.6   3.4   6.5   6.7   4.1  -2.0 -11.7 -11.7 -11.7  -8.0

G 46  -4.2   3.1   6.7   7.5   6.0   1.7  -7.0 -11.7 -11.7 -11.7

L 44  -4.8   2.8   6.7   8.2   7.5   4.5  -1.5 -11.7 -11.7 -11.7

E 42  -5.5   2.3   6.6   8.6   8.6   6.8   2.6  -4.9 -11.7 -11.7

40  -6.2   1.8   6.5   8.9   9.5   8.5   5.7    .6  -8.2 -11.7

I 38  -7.0   1.3   6.2   9.0  10.1   9.9   8.1   4.6  -1.2 -11.7

N 36  -7.8    .6   5.8   8.9  10.5  10.9   9.9   7.6   3.7  -2.6

34  -8.7   -.1   5.3   8.7  10.8  11.6  11.3   9.9   7.3   3.1

D 32  -9.7   -.9   4.7   8.4  10.8  12.1  12.4  11.7  10.0   7.2

E 30 -11.7  -1.8   4.0   7.9  10.6  12.3  13.1  13.0  12.1  10.3

G 28 -11.7  -2.8   3.2   7.3  10.3  12.3  13.5  13.9  13.6  12.6

R 26 -11.7  -3.8   2.3   6.6   9.8  12.1  13.6  14.5  14.7  14.3

E 24 -11.7  -5.0   1.3   5.8   9.2  11.7  13.5  14.7  15.4  15.5

E 22 -11.7  -6.2    .2   4.9   8.4  11.1  13.2  14.7  15.7  16.2

S 20 -11.7  -7.5  -1.0   3.8   7.5  10.4  12.7  14.4  15.7  16.6

18 -11.7  -8.9  -2.3   2.6   6.4   9.5  11.9  13.9  15.4  16.6

16 -11.7 -11.7  -3.8   1.3   5.2   8.4  11.0  13.1  14.9  16.3

14 -11.7 -11.7  -5.3   -.2   3.8   7.1   9.9  12.2  14.0  15.6

12 -11.7 -11.7  -7.0  -1.9   2.3   5.7   8.5  10.9  12.9  14.7

10 -11.7 -11.7  -9.0  -3.7    .5   3.9   6.9   9.4  11.5  13.4

8 -11.7 -11.7 -11.7  -5.9  -1.7   1.9   4.9   7.4   9.7  11.6

6 -11.7 -11.7 -11.7  -8.6  -4.3   -.7   2.3   5.0   7.3   9.3

4 -11.7 -11.7 -11.7 -11.7  -7.9  -4.3  -1.2   1.5   3.8   5.9

2 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7  -7.2  -4.4  -2.1    .0

0 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7

2     3     4     5     6     7     8     9    10    11

FREQUENCY IN MEGAHERTZ

ANTENNA EFFICIENCY

-1.7  -1.7  -1.7  -1.7  -1.7  -1.7  -1.7  -1.7  -1.7  -1.7

2     3     4     5     6     7     8     9    10    11

FREQUENCY IN MEGAHERTZ

Figure 30.  Horizontal rhombic pattern.

METHOD 13   ICEPAC IC.10   PAGE   2

AZIMUTH     EX(1)     EX(2)     EX(3)     EX(4)  CONDUCT.  DIELECT.

.000     .000      .000      .000      .000     .001     4.000

12    13    14    16    18    20    22    24    26    28    30

-3.0 -11.7 -11.7 -11.7 -11.7 -11.7  -2.5 -11.7 -11.7 -11.7 -11.7 90

-3.1  -6.1 -11.7  -5.8 -11.7  -8.0  -6.8 -11.7  -8.2 -11.7  -3.1 88

-5.6  -2.4 -11.7  -5.7 -11.7  -3.9 -11.7  -4.4 -11.7 -11.7  -7.3 86

-11.7  -1.6  -6.7 -11.7 -11.7  -6.4 -11.7  -3.7 -11.7  -6.1 -11.7 84

-11.7  -3.4  -2.2 -11.7  -8.8 -11.7  -3.4 -11.7  -9.2 -11.7  -6.1 82

-11.7  -8.8  -1.1 -11.7  -7.4 -11.7  -1.2 -11.7  -5.9 -11.7  -2.5 80

-11.7 -11.7  -3.0 -11.7 -11.7 -11.7  -7.3  -5.6 -11.7  -8.7 -11.7 78

-6.9 -11.7  -9.0  -4.9 -11.7  -3.9 -11.7   -.7 -11.7  -6.8 -11.7 76

-2.4 -11.7 -11.7  -1.9 -11.7  -4.4 -11.7  -4.9  -7.4 -11.7  -2.9 74

-.6  -5.0 -11.7  -2.6 -11.7 -11.7  -1.7 -11.7  -2.2 -11.7  -5.3 72

-.9   -.6 -11.7  -7.8  -6.2 -11.7   -.3 -11.7  -7.5  -7.2 -11.7 70

-3.4   1.0  -2.2 -11.7  -3.8 -11.7  -6.5    .0 -11.7  -5.2 -11.7 68

-8.7    .2   1.4 -11.7  -5.9  -7.3 -11.7   1.2  -7.4 -11.7  -4.1 66

-11.7  -3.1   2.1  -8.0 -11.7  -4.5 -11.7  -6.4   1.4 -11.7  -9.8 64

-11.7 -11.7    .2   -.2 -11.7  -6.6  -3.4 -11.7    .3  -2.8 -11.7 62

-11.7 -11.7  -4.9   3.0 -11.7 -11.7  -1.2  -7.4 -11.7   1.1  -9.4 60

-10.0 -11.7 -11.7   3.0  -2.2 -11.7  -5.2   1.1 -11.7  -5.5  -1.0 58

-4.6 -11.7 -11.7   -.2   2.6 -11.7 -11.7   1.3    .2 -11.7  -3.7 56

-2.1  -4.6 -11.7  -8.3   3.5  -2.7 -11.7  -5.8   4.2  -2.6 -11.7 54

-1.3   -.8  -4.6 -11.7    .8   2.2 -11.7 -11.7    .5   5.0  -5.9 52

-2.0    .6    .2 -11.7  -7.3   3.0  -1.9 -11.7 -11.7   3.7   4.9 50

-3.8    .2   2.2  -3.5 -11.7   -.6   1.8  -5.4 -11.7  -7.9   5.2 48

-6.7  -1.8   2.1   2.5 -11.7 -11.7   1.1   -.7  -5.0 -11.7  -4.9 46

-11.7  -5.1    .3   5.1   -.2 -11.7  -5.4    .4  -1.1  -3.3 -11.7 44

-11.7 -10.0  -3.4   5.2   5.4  -5.0 -11.7  -3.4   -.9   1.2  -1.2 42

-11.7 -11.7  -8.9   3.2   7.6   4.4 -11.7 -11.7  -3.7   -.6   3.8 40

-11.7 -11.7 -11.7  -1.0   7.2   8.5   3.0 -11.7 -11.7  -4.9   1.1 38

-11.7 -11.7 -11.7  -7.9   4.4   9.4   8.9   2.0 -11.7 -11.7  -6.2 36

-3.3 -11.7 -11.7 -11.7  -1.0   7.8  10.8   9.2   1.8 -11.7 -11.7 34

3.0  -3.2 -11.7 -11.7 -11.7   3.4   9.9  11.9   9.7   2.6 -11.7 32

7.5   3.4  -2.4 -11.7 -11.7  -4.6   6.2  11.4  12.8  10.7   4.4 30

10.9   8.2   4.4  -8.3 -11.7 -11.7  -1.1   7.9  12.4  13.7  12.0 28

13.3  11.6   9.2   1.3 -11.7 -11.7 -11.7    .7   8.9  13.1  14.5 26

15.1  14.2  12.7   7.6  -1.4 -11.7 -11.7 -11.7   1.3   9.1  13.4 24

16.3  16.0  15.2  12.1   6.4  -3.1 -11.7 -11.7 -11.7    .8   8.7 22

17.1  17.2  16.9  15.2  11.7   6.0  -3.4 -11.7 -11.7 -11.7  -1.1 20

17.4  17.9  18.0  17.4  15.4  11.9   6.3  -2.5 -11.7 -11.7 -11.7 18

17.3  18.1  18.6  18.8  17.9  15.9  12.5   7.4   -.5 -11.7 -11.7 16

16.9  17.9  18.7  19.5  19.4  18.5  16.5  13.4   8.9   2.1  -8.8 14

16.1  17.3  18.3  19.6  20.2  20.0  19.0  17.2  14.5  10.5   4.6 12

14.9  16.3  17.4  19.1  20.2  20.5  20.3  19.4  17.8  15.3  11.8 10

13.3  14.8  16.0  18.1  19.4  20.2  20.5  20.2  19.3  17.8  15.6  8

11.1  12.6  14.0  16.2  17.8  18.9  19.5  19.7  19.3  18.4  16.9  6

7.7   9.3  10.8  13.1  15.0  16.3  17.1  17.5  17.5  17.0  16.0  4

1.9   3.5   5.0   7.5   9.4  10.8  11.8  12.3  12.5  12.2  11.5  2

-11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7  0

12    13    14    16    18    20    22    24    26    28    30

FREQUENCY IN MEGAHERTZ

ANTENNA EFFICIENCY

-1.7  -1.7  -1.7  -1.7  -1.7  -1.7  -1.7  -1.7  -1.7  -1.7  -1.7

12    13    14    16    18    20    22    24    26    28    30

FREQUENCY IN MEGAHERTZ

Figure 30a.  Horizontal rhombic pattern continued.

0

METHOD 13   ICEPAC IC.10   PAGE   3

ITSA 1 ANTENNA PACKAGE             ANTENNA PATTERN

FREQUENCY RANGE  ANTENNA TYPE      HEIGHT    LENGTH     ANGLE

2.0 TO  30.0   VER MONOPOLE        .000     -.250      .000

2     3     4     5     6     7     8     9    10    11

90 -50.6 -50.6 -50.6 -50.6 -50.6 -50.6 -50.6 -50.6 -50.6 -50.6

88 -29.7 -30.0 -30.1 -30.2 -30.2 -30.2 -30.2 -30.2 -30.2 -30.1

86 -23.6 -24.0 -24.1 -24.2 -24.2 -24.2 -24.2 -24.2 -24.1 -24.1

84 -20.1 -20.4 -20.6 -20.7 -20.7 -20.7 -20.7 -20.6 -20.6 -20.6

82 -17.6 -18.0 -18.1 -18.2 -18.2 -18.2 -18.2 -18.1 -18.1 -18.1

80 -15.7 -16.0 -16.2 -16.2 -16.2 -16.2 -16.2 -16.2 -16.2 -16.2

78 -14.1 -14.4 -14.6 -14.7 -14.7 -14.7 -14.6 -14.6 -14.6 -14.6

76 -12.8 -13.1 -13.3 -13.3 -13.3 -13.3 -13.3 -13.3 -13.3 -13.3

74 -11.6 -11.9 -12.1 -12.2 -12.2 -12.2 -12.2 -12.1 -12.1 -12.1

72 -10.6 -10.9 -11.1 -11.2 -11.2 -11.2 -11.2 -11.1 -11.1 -11.1

E 70  -9.7 -10.0 -10.2 -10.3 -10.3 -10.3 -10.3 -10.2 -10.2 -10.2

L 68  -8.9  -9.2  -9.4  -9.4  -9.5  -9.5  -9.4  -9.4  -9.4  -9.4

E 66  -8.1  -8.5  -8.6  -8.7  -8.7  -8.7  -8.7  -8.7  -8.7  -8.7

V 64  -7.4  -7.8  -8.0  -8.0  -8.0  -8.0  -8.0  -8.0  -8.0  -8.0

A 62  -6.8  -7.2  -7.3  -7.4  -7.4  -7.4  -7.4  -7.4  -7.4  -7.4

T 60  -6.2  -6.6  -6.8  -6.8  -6.9  -6.9  -6.9  -6.8  -6.8  -6.8

I 58  -5.7  -6.1  -6.3  -6.3  -6.3  -6.3  -6.3  -6.3  -6.3  -6.3

O 56  -5.2  -5.6  -5.8  -5.8  -5.9  -5.9  -5.9  -5.8  -5.8  -5.8

N 54  -4.8  -5.2  -5.3  -5.4  -5.4  -5.4  -5.4  -5.4  -5.4  -5.4

52  -4.4  -4.7  -4.9  -5.0  -5.0  -5.0  -5.0  -5.0  -5.0  -5.0

A 50  -4.0  -4.4  -4.5  -4.6  -4.6  -4.6  -4.6  -4.6  -4.6  -4.6

N 48  -3.6  -4.0  -4.2  -4.3  -4.3  -4.3  -4.3  -4.3  -4.3  -4.3

G 46  -3.3  -3.7  -3.9  -4.0  -4.0  -4.0  -4.0  -4.0  -4.0  -4.0

L 44  -3.0  -3.4  -3.6  -3.7  -3.7  -3.7  -3.7  -3.7  -3.7  -3.7

E 42  -2.7  -3.2  -3.4  -3.4  -3.5  -3.5  -3.5  -3.5  -3.5  -3.4

40  -2.5  -2.9  -3.1  -3.2  -3.3  -3.3  -3.3  -3.3  -3.2  -3.2

I 38  -2.3  -2.8  -3.0  -3.0  -3.1  -3.1  -3.1  -3.1  -3.1  -3.1

N 36  -2.1  -2.6  -2.8  -2.9  -2.9  -2.9  -2.9  -2.9  -2.9  -2.9

34  -2.0  -2.5  -2.7  -2.8  -2.8  -2.8  -2.8  -2.8  -2.8  -2.8

D 32  -1.9  -2.4  -2.6  -2.7  -2.7  -2.8  -2.8  -2.8  -2.7  -2.7

E 30  -1.8  -2.3  -2.6  -2.7  -2.7  -2.7  -2.7  -2.7  -2.7  -2.7

G 28  -1.8  -2.3  -2.6  -2.7  -2.7  -2.7  -2.7  -2.7  -2.7  -2.7

R 26  -1.8  -2.4  -2.6  -2.7  -2.8  -2.8  -2.8  -2.8  -2.8  -2.8

E 24  -1.9  -2.5  -2.7  -2.9  -2.9  -2.9  -2.9  -2.9  -2.9  -2.9

E 22  -2.0  -2.6  -2.9  -3.0  -3.1  -3.1  -3.1  -3.1  -3.1  -3.1

S 20  -2.2  -2.9  -3.1  -3.3  -3.3  -3.3  -3.4  -3.3  -3.3  -3.3

18  -2.5  -3.2  -3.5  -3.6  -3.7  -3.7  -3.7  -3.7  -3.7  -3.7

16  -2.9  -3.6  -3.9  -4.0  -4.1  -4.1  -4.1  -4.1  -4.1  -4.1

14  -3.4  -4.1  -4.5  -4.6  -4.7  -4.7  -4.7  -4.7  -4.7  -4.7

12  -4.1  -4.8  -5.2  -5.3  -5.4  -5.5  -5.5  -5.5  -5.5  -5.5

10  -5.0  -5.8  -6.1  -6.3  -6.4  -6.4  -6.4  -6.5  -6.4  -6.4

8  -6.2  -7.1  -7.5  -7.6  -7.7  -7.8  -7.8  -7.8  -7.8  -7.8

6  -8.0  -8.9  -9.3  -9.5  -9.6  -9.6  -9.7  -9.7  -9.7  -9.7

4 -10.7 -11.7 -12.2 -12.4 -12.5 -12.5 -12.5 -12.5 -12.5 -12.5

2 -16.0 -17.0 -17.5 -17.7 -17.8 -17.9 -17.9 -17.9 -17.9 -17.9

0 -50.6 -50.6 -50.6 -50.6 -50.6 -50.6 -50.6 -50.6 -50.6 -50.6

2     3     4     5     6     7     8     9    10    11

FREQUENCY IN MEGAHERTZ

ANTENNA EFFICIENCY

-.6   -.6   -.6   -.6   -.6   -.6   -.6   -.6   -.6   -.6

2     3     4     5     6     7     8     9    10    11

FREQUENCY IN MEGAHERTZ

Figure 32.  Vertical monopole pattern.

METHOD 13   ICEPAC IC.10   PAGE   4

AZIMUTH     EX(1)     EX(2)     EX(3)     EX(4)  CONDUCT.  DIELECT.

.000     .000      .000      .000      .000     .001     4.000

12    13    14    16    18    20    22    24    26    28    30

-50.6 -50.6 -50.6 -50.6 -50.6 -50.6 -50.6 -50.6 -50.6 -50.6 -50.6 90

-30.1 -30.1 -30.1 -30.0 -30.0 -30.0 -30.0 -30.0 -30.0 -29.9 -29.9 88

-24.1 -24.1 -24.1 -24.0 -24.0 -24.0 -24.0 -23.9 -23.9 -23.9 -23.9 86

-20.6 -20.6 -20.5 -20.5 -20.5 -20.5 -20.4 -20.4 -20.4 -20.4 -20.4 84

-18.1 -18.1 -18.0 -18.0 -18.0 -18.0 -17.9 -17.9 -17.9 -17.9 -17.9 82

-16.1 -16.1 -16.1 -16.1 -16.1 -16.0 -16.0 -16.0 -16.0 -16.0 -16.0 80

-14.6 -14.6 -14.5 -14.5 -14.5 -14.5 -14.4 -14.4 -14.4 -14.4 -14.4 78

-13.2 -13.2 -13.2 -13.2 -13.2 -13.1 -13.1 -13.1 -13.1 -13.1 -13.1 76

-12.1 -12.1 -12.1 -12.0 -12.0 -12.0 -12.0 -11.9 -11.9 -11.9 -11.9 74

-11.1 -11.1 -11.0 -11.0 -11.0 -11.0 -11.0 -10.9 -10.9 -10.9 -10.9 72

-10.2 -10.2 -10.1 -10.1 -10.1 -10.1 -10.1 -10.0 -10.0 -10.0 -10.0 70

-9.4  -9.4  -9.3  -9.3  -9.3  -9.3  -9.2  -9.2  -9.2  -9.2  -9.2 68

-8.6  -8.6  -8.6  -8.6  -8.6  -8.5  -8.5  -8.5  -8.5  -8.5  -8.5 66

-8.0  -7.9  -7.9  -7.9  -7.9  -7.9  -7.8  -7.8  -7.8  -7.8  -7.8 64

-7.4  -7.3  -7.3  -7.3  -7.3  -7.3  -7.2  -7.2  -7.2  -7.2  -7.2 62

-6.8  -6.8  -6.8  -6.7  -6.7  -6.7  -6.7  -6.7  -6.6  -6.6  -6.6 60

-6.3  -6.3  -6.2  -6.2  -6.2  -6.2  -6.2  -6.1  -6.1  -6.1  -6.1 58

-5.8  -5.8  -5.8  -5.7  -5.7  -5.7  -5.7  -5.7  -5.7  -5.6  -5.6 56

-5.4  -5.3  -5.3  -5.3  -5.3  -5.3  -5.2  -5.2  -5.2  -5.2  -5.2 54

-5.0  -4.9  -4.9  -4.9  -4.9  -4.9  -4.8  -4.8  -4.8  -4.8  -4.8 52

-4.6  -4.6  -4.6  -4.5  -4.5  -4.5  -4.5  -4.5  -4.5  -4.4  -4.4 50

-4.2  -4.2  -4.2  -4.2  -4.2  -4.2  -4.1  -4.1  -4.1  -4.1  -4.1 48

-3.9  -3.9  -3.9  -3.9  -3.9  -3.9  -3.8  -3.8  -3.8  -3.8  -3.8 46

-3.7  -3.7  -3.6  -3.6  -3.6  -3.6  -3.6  -3.6  -3.6  -3.5  -3.5 44

-3.4  -3.4  -3.4  -3.4  -3.4  -3.3  -3.3  -3.3  -3.3  -3.3  -3.3 42

-3.2  -3.2  -3.2  -3.2  -3.2  -3.1  -3.1  -3.1  -3.1  -3.1  -3.1 40

-3.0  -3.0  -3.0  -3.0  -3.0  -3.0  -3.0  -2.9  -2.9  -2.9  -2.9 38

-2.9  -2.9  -2.9  -2.9  -2.8  -2.8  -2.8  -2.8  -2.8  -2.8  -2.8 36

-2.8  -2.8  -2.8  -2.7  -2.7  -2.7  -2.7  -2.7  -2.7  -2.7  -2.7 34

-2.7  -2.7  -2.7  -2.7  -2.7  -2.6  -2.6  -2.6  -2.6  -2.6  -2.6 32

-2.7  -2.7  -2.7  -2.6  -2.6  -2.6  -2.6  -2.6  -2.6  -2.6  -2.6 30

-2.7  -2.7  -2.7  -2.7  -2.6  -2.6  -2.6  -2.6  -2.6  -2.6  -2.6 28

-2.8  -2.8  -2.7  -2.7  -2.7  -2.7  -2.7  -2.7  -2.7  -2.7  -2.7 26

-2.9  -2.9  -2.9  -2.8  -2.8  -2.8  -2.8  -2.8  -2.8  -2.8  -2.8 24

-3.1  -3.1  -3.0  -3.0  -3.0  -3.0  -3.0  -3.0  -3.0  -3.0  -3.0 22

-3.3  -3.3  -3.3  -3.3  -3.3  -3.3  -3.2  -3.2  -3.2  -3.2  -3.2 20

-3.7  -3.7  -3.6  -3.6  -3.6  -3.6  -3.6  -3.6  -3.6  -3.6  -3.6 18

-4.1  -4.1  -4.1  -4.1  -4.1  -4.0  -4.0  -4.0  -4.0  -4.0  -4.0 16

-4.7  -4.7  -4.7  -4.7  -4.6  -4.6  -4.6  -4.6  -4.6  -4.6  -4.6 14

-5.4  -5.4  -5.4  -5.4  -5.4  -5.4  -5.4  -5.4  -5.4  -5.4  -5.3 12

-6.4  -6.4  -6.4  -6.4  -6.4  -6.4  -6.4  -6.4  -6.3  -6.3  -6.3 10

-7.8  -7.8  -7.7  -7.7  -7.7  -7.7  -7.7  -7.7  -7.7  -7.7  -7.7  8

-9.6  -9.6  -9.6  -9.6  -9.6  -9.6  -9.6  -9.6  -9.6  -9.6  -9.6  6

-12.5 -12.5 -12.5 -12.5 -12.5 -12.5 -12.5 -12.5 -12.5 -12.4 -12.4  4

-17.9 -17.9 -17.9 -17.9 -17.9 -17.8 -17.8 -17.8 -17.8 -17.8 -17.8  2

-50.6 -50.6 -50.6 -50.6 -50.6 -50.6 -50.6 -50.6 -50.6 -50.6 -50.6  0

12    13    14    16    18    20    22    24    26    28    30

FREQUENCY IN MEGAHERTZ

ANTENNA EFFICIENCY

-.6   -.6   -.6   -.6   -.6   -.6   -.6   -.6   -.6   -.6   -.6

12    13    14    16    18    20    22    24    26    28    30

FREQUENCY IN MEGAHERTZ

Figure 32a.  Vertical monopole pattern continued.

0

METHOD 13   ICEPAC IC.10   PAGE   5

ITSA 1 ANTENNA PACKAGE             ANTENNA PATTERN

FREQUENCY RANGE  ANTENNA TYPE      HEIGHT    LENGTH     ANGLE

2.0 TO  30.0   HORZ. DIPOLE       -.250     -.500      .000

2     3     4     5     6     7     8     9    10    11

90   6.1   5.7   5.4   5.3   5.1   5.1   5.0   4.9   4.9   4.9

88   6.1   5.7   5.4   5.3   5.1   5.1   5.0   4.9   4.9   4.9

86   6.1   5.7   5.5   5.3   5.1   5.1   5.0   4.9   4.9   4.9

84   6.1   5.7   5.5   5.3   5.2   5.1   5.0   5.0   4.9   4.9

82   6.2   5.8   5.5   5.3   5.2   5.1   5.0   5.0   4.9   4.9

80   6.2   5.8   5.5   5.3   5.2   5.1   5.0   5.0   5.0   4.9

78   6.2   5.8   5.5   5.4   5.2   5.1   5.1   5.0   5.0   5.0

76   6.2   5.8   5.6   5.4   5.3   5.2   5.1   5.0   5.0   5.0

74   6.3   5.9   5.6   5.4   5.3   5.2   5.1   5.1   5.0   5.0

72   6.3   5.9   5.6   5.5   5.3   5.2   5.2   5.1   5.1   5.0

E 70   6.3   5.9   5.7   5.5   5.4   5.3   5.2   5.1   5.1   5.1

L 68   6.4   6.0   5.7   5.5   5.4   5.3   5.2   5.2   5.1   5.1

E 66   6.4   6.0   5.8   5.6   5.4   5.3   5.3   5.2   5.2   5.1

V 64   6.4   6.1   5.8   5.6   5.5   5.4   5.3   5.2   5.2   5.1

A 62   6.5   6.1   5.8   5.6   5.5   5.4   5.3   5.3   5.2   5.2

T 60   6.5   6.1   5.9   5.7   5.5   5.4   5.3   5.3   5.2   5.2

I 58   6.5   6.1   5.9   5.7   5.6   5.5   5.4   5.3   5.3   5.2

O 56   6.5   6.2   5.9   5.7   5.6   5.5   5.4   5.3   5.3   5.2

N 54   6.5   6.2   5.9   5.7   5.6   5.5   5.4   5.3   5.3   5.2

52   6.5   6.2   5.9   5.7   5.6   5.5   5.4   5.3   5.3   5.2

A 50   6.5   6.1   5.9   5.7   5.6   5.5   5.4   5.3   5.3   5.2

N 48   6.4   6.1   5.9   5.7   5.6   5.4   5.4   5.3   5.2   5.2

G 46   6.4   6.1   5.8   5.7   5.5   5.4   5.3   5.3   5.2   5.1

L 44   6.3   6.0   5.8   5.6   5.5   5.4   5.3   5.2   5.1   5.1

E 42   6.2   5.9   5.7   5.5   5.4   5.3   5.2   5.1   5.1   5.0

40   6.1   5.8   5.6   5.4   5.3   5.2   5.1   5.0   5.0   4.9

I 38   5.9   5.7   5.5   5.3   5.2   5.1   5.0   4.9   4.8   4.8

N 36   5.7   5.5   5.3   5.2   5.0   4.9   4.8   4.8   4.7   4.7

34   5.5   5.3   5.1   5.0   4.9   4.7   4.7   4.6   4.5   4.5

D 32   5.3   5.1   4.9   4.8   4.6   4.5   4.4   4.4   4.3   4.3

E 30   5.0   4.8   4.6   4.5   4.4   4.3   4.2   4.1   4.1   4.0

G 28   4.6   4.5   4.3   4.2   4.1   4.0   3.9   3.8   3.8   3.7

R 26   4.2   4.1   4.0   3.8   3.7   3.6   3.6   3.5   3.4   3.4

E 24   3.8   3.7   3.5   3.4   3.3   3.2   3.1   3.1   3.0   3.0

E 22   3.2   3.1   3.0   2.9   2.8   2.7   2.7   2.6   2.5   2.5

S 20   2.6   2.6   2.5   2.4   2.3   2.2   2.1   2.1   2.0   1.9

18   1.9   1.9   1.8   1.7   1.6   1.5   1.5   1.4   1.3   1.3

16   1.1   1.1   1.0    .9    .8    .8    .7    .6    .6    .5

14    .1    .1    .0    .0   -.1   -.2   -.2   -.3   -.3   -.4

12  -1.0  -1.1  -1.1  -1.2  -1.2  -1.3  -1.3  -1.4  -1.4  -1.5

10  -2.5  -2.5  -2.5  -2.5  -2.6  -2.6  -2.7  -2.8  -2.8  -2.8

8  -4.3  -4.2  -4.2  -4.3  -4.3  -4.4  -4.4  -4.5  -4.5  -4.6

6  -6.6  -6.6  -6.6  -6.6  -6.6  -6.7  -6.7  -6.8  -6.8  -6.9

4 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0

2 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0

0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0

2     3     4     5     6     7     8     9    10    11

FREQUENCY IN MEGAHERTZ

ANTENNA EFFICIENCY

.0    .0    .0    .0    .0    .0    .0    .0    .0    .0

2     3     4     5     6     7     8     9    10    11

FREQUENCY IN MEGAHERTZ

Figure 34.  Horizontal dipole pattern.

METHOD 13   ICEPAC IC.10   PAGE   6

AZIMUTH     EX(1)     EX(2)     EX(3)     EX(4)  CONDUCT.  DIELECT.

.000     .000      .000      .000      .000     .001     4.000

12    13    14    16    18    20    22    24    26    28    30

4.9   4.8   4.8   4.8   4.8   4.8   4.8   4.8   4.8   4.8   4.8 90

4.9   4.8   4.8   4.8   4.8   4.8   4.8   4.8   4.8   4.8   4.8 88

4.9   4.8   4.8   4.8   4.8   4.8   4.8   4.8   4.8   4.8   4.8 86

4.9   4.9   4.8   4.8   4.8   4.8   4.8   4.8   4.8   4.8   4.8 84

4.9   4.9   4.9   4.8   4.8   4.8   4.8   4.8   4.8   4.8   4.8 82

4.9   4.9   4.9   4.8   4.8   4.8   4.8   4.8   4.8   4.8   4.8 80

4.9   4.9   4.9   4.9   4.9   4.8   4.8   4.8   4.8   4.8   4.8 78

5.0   4.9   4.9   4.9   4.9   4.9   4.8   4.8   4.8   4.8   4.8 76

5.0   5.0   4.9   4.9   4.9   4.9   4.9   4.9   4.9   4.8   4.8 74

5.0   5.0   5.0   4.9   4.9   4.9   4.9   4.9   4.9   4.9   4.9 72

5.0   5.0   5.0   5.0   4.9   4.9   4.9   4.9   4.9   4.9   4.9 70

5.1   5.0   5.0   5.0   5.0   5.0   4.9   4.9   4.9   4.9   4.9 68

5.1   5.1   5.0   5.0   5.0   5.0   5.0   5.0   4.9   4.9   4.9 66

5.1   5.1   5.1   5.0   5.0   5.0   5.0   5.0   5.0   5.0   4.9 64

5.1   5.1   5.1   5.1   5.0   5.0   5.0   5.0   5.0   5.0   5.0 62

5.2   5.1   5.1   5.1   5.1   5.0   5.0   5.0   5.0   5.0   5.0 60

5.2   5.2   5.1   5.1   5.1   5.0   5.0   5.0   5.0   5.0   5.0 58

5.2   5.2   5.1   5.1   5.1   5.0   5.0   5.0   5.0   5.0   5.0 56

5.2   5.2   5.1   5.1   5.1   5.0   5.0   5.0   5.0   5.0   5.0 54

5.2   5.2   5.1   5.1   5.1   5.0   5.0   5.0   5.0   5.0   5.0 52

5.2   5.1   5.1   5.1   5.0   5.0   5.0   5.0   5.0   4.9   4.9 50

5.1   5.1   5.1   5.0   5.0   5.0   5.0   4.9   4.9   4.9   4.9 48

5.1   5.1   5.0   5.0   5.0   4.9   4.9   4.9   4.9   4.9   4.8 46

5.0   5.0   5.0   4.9   4.9   4.9   4.8   4.8   4.8   4.8   4.8 44

5.0   4.9   4.9   4.9   4.8   4.8   4.8   4.7   4.7   4.7   4.7 42

4.9   4.8   4.8   4.8   4.7   4.7   4.7   4.6   4.6   4.6   4.6 40

4.8   4.7   4.7   4.6   4.6   4.6   4.5   4.5   4.5   4.5   4.5 38

4.6   4.6   4.5   4.5   4.4   4.4   4.4   4.4   4.3   4.3   4.3 36

4.4   4.4   4.4   4.3   4.3   4.2   4.2   4.2   4.2   4.1   4.1 34

4.2   4.2   4.1   4.1   4.0   4.0   4.0   4.0   3.9   3.9   3.9 32

4.0   3.9   3.9   3.8   3.8   3.8   3.7   3.7   3.7   3.7   3.7 30

3.7   3.6   3.6   3.5   3.5   3.5   3.4   3.4   3.4   3.4   3.4 28

3.3   3.3   3.3   3.2   3.2   3.1   3.1   3.1   3.0   3.0   3.0 26

2.9   2.9   2.9   2.8   2.8   2.7   2.7   2.7   2.6   2.6   2.6 24

2.5   2.4   2.4   2.3   2.3   2.2   2.2   2.2   2.2   2.1   2.1 22

1.9   1.9   1.8   1.8   1.7   1.7   1.7   1.6   1.6   1.6   1.6 20

1.3   1.2   1.2   1.1   1.1   1.0   1.0   1.0   1.0    .9    .9 18

.5    .5    .4    .4    .3    .3    .3    .2    .2    .2    .2 16

-.4   -.5   -.5   -.5   -.6   -.6   -.7   -.7   -.7   -.7   -.7 14

-1.5  -1.6  -1.6  -1.6  -1.7  -1.7  -1.7  -1.8  -1.8  -1.8  -1.8 12

-2.9  -2.9  -2.9  -3.0  -3.0  -3.1  -3.1  -3.1  -3.1  -3.2  -3.2 10

-4.6  -4.6  -4.7  -4.7  -4.8  -4.8  -4.8  -4.8  -4.9  -4.9  -4.9  8

-6.9  -6.9  -6.9  -7.0  -7.0  -7.1  -7.1  -7.1  -7.2  -7.2  -7.2  6

-10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0  4

-10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0  2

-10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0  0

12    13    14    16    18    20    22    24    26    28    30

FREQUENCY IN MEGAHERTZ

ANTENNA EFFICIENCY

.0    .0    .0    .0    .0    .0    .0    .0    .0    .0    .0

12    13    14    16    18    20    22    24    26    28    30

FREQUENCY IN MEGAHERTZ

Figure 34a.  Horizontal dipole pattern continued.

0

METHOD 13   ICEPAC IC.10   PAGE   7

ITSA 1 ANTENNA PACKAGE             ANTENNA PATTERN

FREQUENCY RANGE  ANTENNA TYPE      HEIGHT    LENGTH     ANGLE

2.0 TO  30.0   HORZ. YAGI         -.258     -.500      .000

2     3     4     5     6     7     8     9    10    11

90   6.0   5.6   5.4   5.2   5.1   5.0   4.9   4.9   4.9   4.8

88   6.0   5.6   5.4   5.2   5.1   5.0   4.9   4.9   4.9   4.8

86   6.0   5.6   5.4   5.2   5.1   5.0   4.9   4.9   4.9   4.8

84   6.1   5.7   5.4   5.2   5.1   5.0   5.0   4.9   4.9   4.9

82   6.1   5.7   5.4   5.2   5.1   5.0   5.0   4.9   4.9   4.9

80   6.1   5.7   5.4   5.3   5.1   5.1   5.0   4.9   4.9   4.9

78   6.1   5.7   5.5   5.3   5.2   5.1   5.0   5.0   4.9   4.9

76   6.2   5.8   5.5   5.3   5.2   5.1   5.1   5.0   5.0   4.9

74   6.2   5.8   5.5   5.4   5.2   5.2   5.1   5.0   5.0   5.0

72   6.3   5.9   5.6   5.4   5.3   5.2   5.1   5.1   5.0   5.0

E 70   6.3   5.9   5.6   5.5   5.3   5.2   5.2   5.1   5.1   5.0

L 68   6.3   6.0   5.7   5.5   5.4   5.3   5.2   5.2   5.1   5.1

E 66   6.4   6.0   5.7   5.5   5.4   5.3   5.3   5.2   5.2   5.1

V 64   6.4   6.0   5.8   5.6   5.5   5.4   5.3   5.2   5.2   5.2

A 62   6.5   6.1   5.8   5.6   5.5   5.4   5.3   5.3   5.2   5.2

T 60   6.5   6.1   5.9   5.7   5.5   5.4   5.4   5.3   5.3   5.2

I 58   6.5   6.2   5.9   5.7   5.6   5.5   5.4   5.3   5.3   5.2

O 56   6.5   6.2   5.9   5.7   5.6   5.5   5.4   5.4   5.3   5.3

N 54   6.5   6.2   6.0   5.8   5.6   5.5   5.4   5.4   5.3   5.3

52   6.5   6.2   6.0   5.8   5.6   5.5   5.5   5.4   5.3   5.3

A 50   6.5   6.2   6.0   5.8   5.6   5.5   5.5   5.4   5.3   5.3

N 48   6.5   6.2   6.0   5.8   5.6   5.5   5.4   5.4   5.3   5.3

G 46   6.4   6.2   5.9   5.7   5.6   5.5   5.4   5.3   5.3   5.2

L 44   6.4   6.1   5.9   5.7   5.6   5.5   5.4   5.3   5.2   5.2

E 42   6.3   6.0   5.8   5.6   5.5   5.4   5.3   5.2   5.2   5.1

40   6.2   5.9   5.7   5.5   5.4   5.3   5.2   5.1   5.1   5.0

I 38   6.0   5.8   5.6   5.4   5.3   5.2   5.1   5.0   5.0   4.9

N 36   5.9   5.6   5.5   5.3   5.2   5.1   5.0   4.9   4.8   4.8

34   5.7   5.5   5.3   5.1   5.0   4.9   4.8   4.7   4.7   4.6

D 32   5.4   5.2   5.1   4.9   4.8   4.7   4.6   4.5   4.5   4.4

E 30   5.1   5.0   4.8   4.7   4.5   4.4   4.4   4.3   4.2   4.2

G 28   4.8   4.6   4.5   4.4   4.3   4.2   4.1   4.0   3.9   3.9

R 26   4.4   4.3   4.1   4.0   3.9   3.8   3.7   3.7   3.6   3.6

E 24   4.0   3.8   3.7   3.6   3.5   3.4   3.3   3.3   3.2   3.2

E 22   3.5   3.3   3.2   3.1   3.0   2.9   2.9   2.8   2.7   2.7

S 20   2.8   2.8   2.7   2.6   2.5   2.4   2.3   2.3   2.2   2.1

18   2.1   2.1   2.0   1.9   1.8   1.7   1.7   1.6   1.6   1.5

16   1.3   1.3   1.2   1.1   1.0   1.0    .9    .8    .8    .7

14    .3    .3    .3    .2    .1    .0    .0   -.1   -.1   -.2

12   -.8   -.8   -.9   -.9  -1.0  -1.1  -1.1  -1.2  -1.2  -1.3

10  -2.2  -2.2  -2.3  -2.3  -2.4  -2.4  -2.5  -2.5  -2.6  -2.6

8  -4.0  -4.0  -4.0  -4.1  -4.1  -4.2  -4.2  -4.3  -4.3  -4.3

6  -6.4  -6.4  -6.4  -6.4  -6.4  -6.5  -6.5  -6.6  -6.6  -6.6

4  -9.8  -9.7  -9.7  -9.7  -9.8  -9.8  -9.8  -9.9  -9.9 -10.0

2 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0

0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0

2     3     4     5     6     7     8     9    10    11

FREQUENCY IN MEGAHERTZ

ANTENNA EFFICIENCY

.0    .0    .0    .0    .0    .0    .0    .0    .0    .0

2     3     4     5     6     7     8     9    10    11

FREQUENCY IN MEGAHERTZ

Figure 36.  Horizontal Yagi Pattern

METHOD 13   ICEPAC IC.10   PAGE   8

AZIMUTH     EX(1)     EX(2)     EX(3)     EX(4)  CONDUCT.  DIELECT.

.000     .000      .000      .000      .000     .001     4.000

12    13    14    16    18    20    22    24    26    28    30

4.8   4.8   4.8   4.8   4.8   4.7   4.7   4.7   4.7   4.7   4.7 90

4.8   4.8   4.8   4.8   4.8   4.7   4.7   4.7   4.7   4.7   4.7 88

4.8   4.8   4.8   4.8   4.8   4.8   4.7   4.7   4.7   4.7   4.7 86

4.8   4.8   4.8   4.8   4.8   4.8   4.8   4.8   4.8   4.7   4.7 84

4.8   4.8   4.8   4.8   4.8   4.8   4.8   4.8   4.8   4.8   4.8 82

4.9   4.9   4.8   4.8   4.8   4.8   4.8   4.8   4.8   4.8   4.8 80

4.9   4.9   4.9   4.8   4.8   4.8   4.8   4.8   4.8   4.8   4.8 78

4.9   4.9   4.9   4.9   4.9   4.8   4.8   4.8   4.8   4.8   4.8 76

5.0   4.9   4.9   4.9   4.9   4.9   4.9   4.9   4.8   4.8   4.8 74

5.0   5.0   5.0   4.9   4.9   4.9   4.9   4.9   4.9   4.9   4.9 72

5.0   5.0   5.0   5.0   4.9   4.9   4.9   4.9   4.9   4.9   4.9 70

5.1   5.0   5.0   5.0   5.0   5.0   4.9   4.9   4.9   4.9   4.9 68

5.1   5.1   5.1   5.0   5.0   5.0   5.0   5.0   5.0   5.0   4.9 66

5.1   5.1   5.1   5.1   5.0   5.0   5.0   5.0   5.0   5.0   5.0 64

5.2   5.1   5.1   5.1   5.1   5.0   5.0   5.0   5.0   5.0   5.0 62

5.2   5.2   5.1   5.1   5.1   5.1   5.1   5.0   5.0   5.0   5.0 60

5.2   5.2   5.2   5.1   5.1   5.1   5.1   5.1   5.0   5.0   5.0 58

5.2   5.2   5.2   5.2   5.1   5.1   5.1   5.1   5.1   5.1   5.0 56

5.3   5.2   5.2   5.2   5.1   5.1   5.1   5.1   5.1   5.1   5.0 54

5.3   5.2   5.2   5.2   5.1   5.1   5.1   5.1   5.1   5.0   5.0 52

5.3   5.2   5.2   5.2   5.1   5.1   5.1   5.1   5.0   5.0   5.0 50

5.2   5.2   5.2   5.1   5.1   5.1   5.0   5.0   5.0   5.0   5.0 48

5.2   5.2   5.1   5.1   5.1   5.0   5.0   5.0   5.0   5.0   5.0 46

5.2   5.1   5.1   5.0   5.0   5.0   5.0   4.9   4.9   4.9   4.9 44

5.1   5.1   5.0   5.0   4.9   4.9   4.9   4.9   4.8   4.8   4.8 42

5.0   5.0   4.9   4.9   4.8   4.8   4.8   4.8   4.8   4.7   4.7 40

4.9   4.9   4.8   4.8   4.7   4.7   4.7   4.7   4.6   4.6   4.6 38

4.8   4.7   4.7   4.6   4.6   4.6   4.5   4.5   4.5   4.5   4.5 36

4.6   4.5   4.5   4.5   4.4   4.4   4.4   4.3   4.3   4.3   4.3 34

4.4   4.3   4.3   4.3   4.2   4.2   4.2   4.1   4.1   4.1   4.1 32

4.1   4.1   4.1   4.0   4.0   3.9   3.9   3.9   3.9   3.8   3.8 30

3.9   3.8   3.8   3.7   3.7   3.6   3.6   3.6   3.6   3.6   3.5 28

3.5   3.5   3.4   3.4   3.3   3.3   3.3   3.3   3.2   3.2   3.2 26

3.1   3.1   3.0   3.0   2.9   2.9   2.9   2.9   2.8   2.8   2.8 24

2.7   2.6   2.6   2.5   2.5   2.4   2.4   2.4   2.4   2.3   2.3 22

2.1   2.1   2.0   2.0   1.9   1.9   1.9   1.8   1.8   1.8   1.8 20

1.5   1.4   1.4   1.3   1.3   1.3   1.2   1.2   1.2   1.2   1.1 18

.7    .7    .6    .6    .5    .5    .5    .4    .4    .4    .4 16

-.2   -.2   -.3   -.3   -.4   -.4   -.4   -.5   -.5   -.5   -.5 14

-1.3  -1.3  -1.4  -1.4  -1.5  -1.5  -1.5  -1.6  -1.6  -1.6  -1.6 12

-2.7  -2.7  -2.7  -2.8  -2.8  -2.8  -2.9  -2.9  -2.9  -2.9  -3.0 10

-4.4  -4.4  -4.4  -4.5  -4.5  -4.6  -4.6  -4.6  -4.6  -4.7  -4.7  8

-6.7  -6.7  -6.7  -6.8  -6.8  -6.8  -6.9  -6.9  -6.9  -6.9  -7.0  6

-10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0  4

-10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0  2

-10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0  0

12    13    14    16    18    20    22    24    26    28    30

FREQUENCY IN MEGAHERTZ

ANTENNA EFFICIENCY

.0    .0    .0    .0    .0    .0    .0    .0    .0    .0    .0

12    13    14    16    18    20    22    24    26    28    30

FREQUENCY IN MEGAHERTZ

Figure 36a.  Horizontal Yagi Pattern continued.

0

METHOD 13   ICEPAC IC.10   PAGE   9

ITSA 1 ANTENNA PACKAGE             ANTENNA PATTERN

FREQUENCY RANGE  ANTENNA TYPE      HEIGHT    LENGTH     ANGLE

2.0 TO  30.0   V LOG PERIOD        .000     -.250      .000

2     3     4     5     6     7     8     9    10    11

90 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6

88 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6

86 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6

84 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6

82 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6

80 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6

78 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6

76 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6

74 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6

72 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6

E 70  -9.7 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6

L 68  -8.9  -9.2  -9.4  -9.4  -9.5  -9.5  -9.4  -9.4  -9.4  -9.4

E 66  -8.1  -8.5  -8.6  -8.7  -8.7  -8.7  -8.7  -8.7  -8.7  -8.7

V 64  -7.4  -7.8  -8.0  -8.0  -8.0  -8.0  -8.0  -8.0  -8.0  -8.0

A 62  -6.8  -7.2  -7.3  -7.4  -7.4  -7.4  -7.4  -7.4  -7.4  -7.4

T 60  -6.2  -6.6  -6.8  -6.8  -6.9  -6.9  -6.9  -6.8  -6.8  -6.8

I 58  -5.7  -6.1  -6.3  -6.3  -6.3  -6.3  -6.3  -6.3  -6.3  -6.3

O 56  -5.2  -5.6  -5.8  -5.8  -5.9  -5.9  -5.9  -5.8  -5.8  -5.8

N 54  -4.8  -5.2  -5.3  -5.4  -5.4  -5.4  -5.4  -5.4  -5.4  -5.4

52  -4.4  -4.7  -4.9  -5.0  -5.0  -5.0  -5.0  -5.0  -5.0  -5.0

A 50  -4.0  -4.4  -4.5  -4.6  -4.6  -4.6  -4.6  -4.6  -4.6  -4.6

N 48  -3.6  -4.0  -4.2  -4.3  -4.3  -4.3  -4.3  -4.3  -4.3  -4.3

G 46  -3.3  -3.7  -3.9  -4.0  -4.0  -4.0  -4.0  -4.0  -4.0  -4.0

L 44  -3.0  -3.4  -3.6  -3.7  -3.7  -3.7  -3.7  -3.7  -3.7  -3.7

E 42  -2.7  -3.2  -3.4  -3.4  -3.5  -3.5  -3.5  -3.5  -3.5  -3.4

40  -2.5  -2.9  -3.1  -3.2  -3.3  -3.3  -3.3  -3.3  -3.2  -3.2

I 38  -2.3  -2.8  -3.0  -3.0  -3.1  -3.1  -3.1  -3.1  -3.1  -3.1

N 36  -2.1  -2.6  -2.8  -2.9  -2.9  -2.9  -2.9  -2.9  -2.9  -2.9

34  -2.0  -2.5  -2.7  -2.8  -2.8  -2.8  -2.8  -2.8  -2.8  -2.8

D 32  -1.9  -2.4  -2.6  -2.7  -2.7  -2.8  -2.8  -2.8  -2.7  -2.7

E 30  -1.8  -2.3  -2.6  -2.7  -2.7  -2.7  -2.7  -2.7  -2.7  -2.7

G 28  -1.8  -2.3  -2.6  -2.7  -2.7  -2.7  -2.7  -2.7  -2.7  -2.7

R 26  -1.8  -2.4  -2.6  -2.7  -2.8  -2.8  -2.8  -2.8  -2.8  -2.8

E 24  -1.9  -2.5  -2.7  -2.9  -2.9  -2.9  -2.9  -2.9  -2.9  -2.9

E 22  -2.0  -2.6  -2.9  -3.0  -3.1  -3.1  -3.1  -3.1  -3.1  -3.1

S 20  -2.2  -2.9  -3.1  -3.3  -3.3  -3.3  -3.4  -3.3  -3.3  -3.3

18  -2.5  -3.2  -3.5  -3.6  -3.7  -3.7  -3.7  -3.7  -3.7  -3.7

16  -2.9  -3.6  -3.9  -4.0  -4.1  -4.1  -4.1  -4.1  -4.1  -4.1

14  -3.4  -4.1  -4.5  -4.6  -4.7  -4.7  -4.7  -4.7  -4.7  -4.7

12  -4.1  -4.8  -5.2  -5.3  -5.4  -5.5  -5.5  -5.5  -5.5  -5.5

10  -5.0  -5.8  -6.1  -6.3  -6.4  -6.4  -6.4  -6.5  -6.4  -6.4

8  -6.2  -7.1  -7.5  -7.6  -7.7  -7.8  -7.8  -7.8  -7.8  -7.8

6  -8.0  -8.9  -9.3  -9.5  -9.6  -9.6  -9.7  -9.7  -9.7  -9.7

4 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6

2 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6

0 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6

2     3     4     5     6     7     8     9    10    11

FREQUENCY IN MEGAHERTZ

ANTENNA EFFICIENCY

-.6   -.6   -.6   -.6   -.6   -.6   -.6   -.6   -.6   -.6

2     3     4     5     6     7     8     9    10    11

FREQUENCY IN MEGAHERTZ

Figure 38.  Vertical log periodic pattern.

METHOD 13   ICEPAC IC.10   PAGE  10

AZIMUTH     EX(1)     EX(2)     EX(3)     EX(4)  CONDUCT.  DIELECT.

.000     .000      .000      .000      .000     .001     4.000

12    13    14    16    18    20    22    24    26    28    30

-10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 90

-10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 88

-10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 86

-10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 84

-10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 82

-10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 80

-10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 78

-10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 76

-10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 74

-10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 72

-10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 70

-9.4  -9.4  -9.3  -9.3  -9.3  -9.3  -9.2  -9.2  -9.2  -9.2  -9.2 68

-8.6  -8.6  -8.6  -8.6  -8.6  -8.5  -8.5  -8.5  -8.5  -8.5  -8.5 66

-8.0  -7.9  -7.9  -7.9  -7.9  -7.9  -7.8  -7.8  -7.8  -7.8  -7.8 64

-7.4  -7.3  -7.3  -7.3  -7.3  -7.3  -7.2  -7.2  -7.2  -7.2  -7.2 62

-6.8  -6.8  -6.8  -6.7  -6.7  -6.7  -6.7  -6.7  -6.6  -6.6  -6.6 60

-6.3  -6.3  -6.2  -6.2  -6.2  -6.2  -6.2  -6.1  -6.1  -6.1  -6.1 58

-5.8  -5.8  -5.8  -5.7  -5.7  -5.7  -5.7  -5.7  -5.7  -5.6  -5.6 56

-5.4  -5.3  -5.3  -5.3  -5.3  -5.3  -5.2  -5.2  -5.2  -5.2  -5.2 54

-5.0  -4.9  -4.9  -4.9  -4.9  -4.9  -4.8  -4.8  -4.8  -4.8  -4.8 52

-4.6  -4.6  -4.6  -4.5  -4.5  -4.5  -4.5  -4.5  -4.5  -4.4  -4.4 50

-4.2  -4.2  -4.2  -4.2  -4.2  -4.2  -4.1  -4.1  -4.1  -4.1  -4.1 48

-3.9  -3.9  -3.9  -3.9  -3.9  -3.9  -3.8  -3.8  -3.8  -3.8  -3.8 46

-3.7  -3.7  -3.6  -3.6  -3.6  -3.6  -3.6  -3.6  -3.6  -3.5  -3.5 44

-3.4  -3.4  -3.4  -3.4  -3.4  -3.3  -3.3  -3.3  -3.3  -3.3  -3.3 42

-3.2  -3.2  -3.2  -3.2  -3.2  -3.1  -3.1  -3.1  -3.1  -3.1  -3.1 40

-3.0  -3.0  -3.0  -3.0  -3.0  -3.0  -3.0  -2.9  -2.9  -2.9  -2.9 38

-2.9  -2.9  -2.9  -2.9  -2.8  -2.8  -2.8  -2.8  -2.8  -2.8  -2.8 36

-2.8  -2.8  -2.8  -2.7  -2.7  -2.7  -2.7  -2.7  -2.7  -2.7  -2.7 34

-2.7  -2.7  -2.7  -2.7  -2.7  -2.6  -2.6  -2.6  -2.6  -2.6  -2.6 32

-2.7  -2.7  -2.7  -2.6  -2.6  -2.6  -2.6  -2.6  -2.6  -2.6  -2.6 30

-2.7  -2.7  -2.7  -2.7  -2.6  -2.6  -2.6  -2.6  -2.6  -2.6  -2.6 28

-2.8  -2.8  -2.7  -2.7  -2.7  -2.7  -2.7  -2.7  -2.7  -2.7  -2.7 26

-2.9  -2.9  -2.9  -2.8  -2.8  -2.8  -2.8  -2.8  -2.8  -2.8  -2.8 24

-3.1  -3.1  -3.0  -3.0  -3.0  -3.0  -3.0  -3.0  -3.0  -3.0  -3.0 22

-3.3  -3.3  -3.3  -3.3  -3.3  -3.3  -3.2  -3.2  -3.2  -3.2  -3.2 20

-3.7  -3.7  -3.6  -3.6  -3.6  -3.6  -3.6  -3.6  -3.6  -3.6  -3.6 18

-4.1  -4.1  -4.1  -4.1  -4.1  -4.0  -4.0  -4.0  -4.0  -4.0  -4.0 16

-4.7  -4.7  -4.7  -4.7  -4.6  -4.6  -4.6  -4.6  -4.6  -4.6  -4.6 14

-5.4  -5.4  -5.4  -5.4  -5.4  -5.4  -5.4  -5.4  -5.4  -5.4  -5.3 12

-6.4  -6.4  -6.4  -6.4  -6.4  -6.4  -6.4  -6.4  -6.3  -6.3  -6.3 10

-7.8  -7.8  -7.7  -7.7  -7.7  -7.7  -7.7  -7.7  -7.7  -7.7  -7.7  8

-9.6  -9.6  -9.6  -9.6  -9.6  -9.6  -9.6  -9.6  -9.6  -9.6  -9.6  6

-10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6  4

-10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6  2

-10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6 -10.6  0

12    13    14    16    18    20    22    24    26    28    30

FREQUENCY IN MEGAHERTZ

ANTENNA EFFICIENCY

-.6   -.6   -.6   -.6   -.6   -.6   -.6   -.6   -.6   -.6   -.6

12    13    14    16    18    20    22    24    26    28    30

FREQUENCY IN MEGAHERTZ

Figure 38a.  Vertical log periodic pattern continued.

METHOD 13   ICEPAC IC.10   PAGE  11

ITSA 1 ANTENNA PACKAGE             ANTENNA PATTERN

FREQUENCY RANGE  ANTENNA TYPE      HEIGHT    LENGTH     ANGLE

2.0 TO  30.0   CURTAIN ANT.      16.000    22.000     2.000

2     3     4     5     6     7     8     9    10    11

90 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0

88 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0

86 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0  -9.2 -10.0

84 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0  -8.6  -5.7 -10.0

82 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0  -6.3  -3.1  -8.9

80 -10.0 -10.0 -10.0 -10.0 -10.0  -9.8  -8.6  -4.7  -1.1  -6.1

78 -10.0 -10.0 -10.0 -10.0 -10.0  -8.4  -6.8  -3.4    .6  -3.7

76 -10.0 -10.0 -10.0 -10.0 -10.0  -7.3  -5.3  -2.5   1.9  -1.4

74 -10.0 -10.0 -10.0 -10.0 -10.0  -6.4  -3.9  -1.9   3.0    .6

72 -10.0 -10.0 -10.0 -10.0 -10.0  -5.8  -2.6  -1.4   3.9   2.5

E 70 -10.0 -10.0 -10.0 -10.0 -10.0  -5.5  -1.4  -1.1   4.6   4.2

L 68 -10.0 -10.0 -10.0 -10.0 -10.0  -5.4   -.4   -.7   5.1   5.7

E 66 -10.0 -10.0 -10.0 -10.0 -10.0  -5.6    .5   -.2   5.3   6.9

V 64 -10.0 -10.0 -10.0 -10.0 -10.0  -6.3   1.3    .5   5.2   7.9

A 62 -10.0  -9.0 -10.0 -10.0 -10.0  -7.6   1.8   1.6   4.9   8.7

T 60 -10.0  -8.2  -9.1  -9.6  -8.4  -9.8   2.1   2.8   4.3   9.1

I 58 -10.0  -7.3  -7.7  -9.4  -6.8 -10.0   2.2   3.9   3.7   9.1

O 56 -10.0  -6.5  -6.3  -9.0  -5.5 -10.0   1.8   5.0   3.5   8.8

N 54 -10.0  -5.7  -4.9  -8.5  -4.5 -10.0   1.0   5.8   4.2   7.9

52 -10.0  -5.0  -3.6  -7.6  -3.9  -9.7   -.5   6.3   5.5   6.7

A 50 -10.0  -4.3  -2.3  -6.2  -3.5  -5.5  -3.3   6.3   6.9   5.6

N 48 -10.0  -3.6  -1.1  -4.5  -3.4  -2.8  -9.0   5.8   8.1   5.7

G 46 -10.0  -3.0    .1  -2.5  -3.6  -1.0 -10.0   4.5   8.9   7.2

L 44 -10.0  -2.5   1.2   -.5  -3.7    .1  -7.4   2.0   8.9   9.0

E 42 -10.0  -2.0   2.2   1.4  -3.1    .6  -1.7  -3.0   8.2  10.3

40 -10.0  -1.5   3.1   3.3  -1.3    .4   1.3 -10.0   6.4  10.7

I 38 -10.0  -1.1   4.0   4.9   1.3   -.2   2.9  -4.1   2.5  10.2

N 36 -10.0   -.7   4.8   6.4   4.1   -.3   3.4   1.9  -8.0   8.3

34 -10.0   -.4   5.5   7.8   6.6   1.5   2.9   4.6  -4.4   3.8

D 32 -10.0   -.2   6.1   9.0   8.8   4.9   1.9   5.5   3.5 -10.0

E 30 -10.0   -.1   6.6  10.1  10.8   8.4   2.9   4.8   6.4    .4

G 28 -10.0    .0   7.0  11.0  12.4  11.3   7.0   3.3   6.9   6.3

R 26 -10.0    .0   7.3  11.7  13.8  13.8  11.2   5.5   5.2   7.9

E 24 -10.0   -.1   7.5  12.3  15.0  15.8  14.6  10.9   4.8   6.5

E 22 -10.0   -.2   7.6  12.8  15.9  17.4  17.3  15.3  10.9   4.8

S 20 -10.0   -.5   7.6  13.0  16.6  18.7  19.4  18.6  16.2  11.6

18 -10.0  -1.0   7.4  13.1  17.1  19.6  20.9  21.1  20.0  17.5

16 -10.0  -1.5   7.0  13.0  17.3  20.2  22.1  22.9  22.7  21.6

14 -10.0  -2.3   6.4  12.6  17.2  20.5  22.7  24.1  24.6  24.3

12 -10.0  -3.2   5.6  12.0  16.8  20.4  23.0  24.7  25.7  26.1

10 -10.0  -4.5   4.5  11.1  16.0  19.8  22.7  24.8  26.2  26.9

8 -10.0  -6.1   2.9   9.6  14.8  18.8  21.9  24.2  25.9  27.0

6 -10.0  -8.4    .8   7.6  12.9  17.0  20.2  22.8  24.6  26.0

4 -10.0 -10.0  -2.5   4.4   9.8  14.0  17.4  20.0  22.1  23.5

2 -10.0 -10.0  -8.3  -1.3   4.1   8.4  11.8  14.5  16.7  18.2

0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0

2     3     4     5     6     7     8     9    10    11

FREQUENCY IN MEGAHERTZ

ANTENNA EFFICIENCY

.0    .0    .0    .0    .0    .0    .0    .0    .0    .0

2     3     4     5     6     7     8     9    10    11

FREQUENCY IN MEGAHERTZ

Figure 40.  Curtain pattern.

METHOD 13   ICEPAC IC.10   PAGE  12

AZIMUTH     EX(1)     EX(2)     EX(3)     EX(4)  CONDUCT.  DIELECT.

.000    4.000    26.000    13.000     7.000     .001     4.000

12    13    14    16    18    20    22    24    26    28    30

-10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 90

-10.0 -10.0  -8.8 -10.0 -10.0  -4.8  -5.1 -10.0 -10.0 -10.0 -10.0 88

-10.0 -10.0  -2.9  -6.8  -9.3   1.2    .7  -4.6 -10.0 -10.0 -10.0 86

-10.0  -7.8    .4  -2.9  -6.8   4.7   3.7   -.9 -10.0 -10.0 -10.0 84

-10.0  -6.0   2.6    .1  -6.0   7.1   5.6   1.7 -10.0 -10.0 -10.0 82

-10.0  -4.8   4.0   2.7  -6.4   8.8   6.6   3.8 -10.0 -10.0 -10.0 80

-10.0  -4.2   5.0   4.9  -8.5  10.1   7.0   5.5 -10.0 -10.0 -10.0 78

-10.0  -3.9   5.5   6.9 -10.0  10.9   7.0   6.7 -10.0 -10.0 -10.0 76

-10.0  -3.9   5.6   8.7 -10.0  11.3   6.9   7.6 -10.0 -10.0 -10.0 74

-10.0  -4.0   5.3  10.2 -10.0  11.2   7.1   8.0 -10.0 -10.0 -10.0 72

-9.1  -4.3   4.6  11.5  -7.3  10.5   7.9   7.9 -10.0 -10.0 -10.0 70

-3.9  -5.1   3.4  12.5  -3.2   9.1   9.1   7.3  -9.2 -10.0  -7.2 68

.1  -7.1   1.9  13.1    .6   6.6  10.1   5.8  -7.7 -10.0  -3.8 66

3.2 -10.0    .4  13.4   4.3   2.6  10.6   3.6  -7.0 -10.0  -1.9 64

5.8 -10.0   -.8  13.1   7.6  -4.3  10.3   1.2  -7.1 -10.0  -1.6 62

7.8  -4.8  -2.1  12.3  10.2 -10.0   9.0    .6  -8.2 -10.0  -3.0 60

9.4   1.6  -5.0  10.7  12.1  -9.5   6.5   1.8 -10.0 -10.0  -5.5 58

10.6   5.7 -10.0   8.3  13.2  -3.3   2.1   2.4 -10.0 -10.0  -4.6 56

11.2   8.6  -6.2   4.9  13.5   2.1  -5.6   1.7 -10.0 -10.0  -1.9 54

11.3  10.7   2.6   1.4  12.8   6.3 -10.0   -.8 -10.0 -10.0  -1.3 52

10.7  12.0   7.4   -.8  11.0   8.9 -10.0  -5.7 -10.0 -10.0  -3.4 50

9.5  12.5  10.4  -4.2   7.7  10.0  -6.3 -10.0 -10.0 -10.0  -9.1 48

7.6  12.1  12.2 -10.0   2.7   9.6   -.6 -10.0 -10.0 -10.0 -10.0 46

6.3  10.8  12.9    .8  -2.0   7.5   2.3 -10.0 -10.0 -10.0 -10.0 44

7.5   8.5  12.4   7.4  -4.5   3.3   3.0 -10.0 -10.0 -10.0 -10.0 42

9.7   6.5  10.7  10.7 -10.0  -3.6   1.3 -10.0 -10.0 -10.0 -10.0 40

11.3   8.1   7.6  12.0  -1.0  -8.2  -3.1 -10.0 -10.0 -10.0 -10.0 38

11.8  10.7   6.4  11.5   5.9 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 36

10.9  12.2   9.5   8.8   8.6  -8.7 -10.0 -10.0 -10.0 -10.0 -10.0 34

8.0  12.0  11.8   4.4   8.6    .2 -10.0 -10.0 -10.0 -10.0 -10.0 32

.5   9.9  12.2   6.4   5.6   2.9 -10.0 -10.0 -10.0 -10.0  -8.0 30

-4.8   3.9  10.4  10.0   -.1   1.8  -8.8 -10.0 -10.0 -10.0  -8.2 28

5.8 -10.0   4.5  10.7   3.7  -3.7  -8.9 -10.0 -10.0 -10.0 -10.0 26

8.3   5.6 -10.0   8.2   6.8  -5.9 -10.0 -10.0 -10.0 -10.0 -10.0 24

6.9   8.4   5.9  -1.5   5.9   -.7 -10.0 -10.0 -10.0 -10.0 -10.0 22

5.0   6.3   7.9   -.2   -.9  -1.1 -10.0 -10.0 -10.0 -10.0  -9.2 20

13.2   6.4   4.1   5.7  -7.4  -8.2 -10.0 -10.0 -10.0 -10.0  -9.3 18

19.3  15.5   9.7   2.4   1.2 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 16

23.2  21.1  18.0   7.2  -3.6  -7.5 -10.0 -10.0 -10.0 -10.0  -7.7 14

25.7  24.6  22.8  16.6   5.4 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 12

27.1  26.6  25.6  21.4  14.0    .8 -10.0 -10.0 -10.0 -10.0 -10.0 10

27.5  27.4  26.8  23.9  18.0   7.0 -10.0  -6.9 -10.0 -10.0 -10.0  8

26.7  27.0  26.7  24.5  19.6   9.5  -5.9    .0 -10.0 -10.0   1.5  6

24.5  24.9  24.9  23.1  18.7   9.1  -3.9   1.9  -9.7 -10.0   7.2  4

19.3  19.8  19.9  18.4  14.3   4.9  -6.9  -1.1 -10.0 -10.0   5.7  2

-10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0  0

12    13    14    16    18    20    22    24    26    28    30

FREQUENCY IN MEGAHERTZ

ANTENNA EFFICIENCY

.0    .0    .0    .0    .0    .0    .0    .0    .0    .0    .0

12    13    14    16    18    20    22    24    26    28    30

FREQUENCY IN MEGAHERTZ

Figure 40a.  Curtain pattern continued.

0

METHOD 13   ICEPAC IC.10   PAGE  13

ITSA 1 ANTENNA PACKAGE             ANTENNA PATTERN

FREQUENCY RANGE  ANTENNA TYPE      HEIGHT    LENGTH     ANGLE

2.0 TO  30.0   TER SLOP VEE      15.240   121.900    22.500

2     3     4     5     6     7     8     9    10    11

90 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7

88 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7

86 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7

84 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7

82 -11.7 -11.7  -9.8 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7

80 -11.7 -11.7  -9.6 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7

78 -11.7 -11.7  -9.5 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7

76 -11.7 -11.7  -9.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7

74 -11.7 -11.7 -11.7  -9.1 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7

72 -11.7 -11.7 -11.7  -8.3 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7

E 70 -11.7 -11.7 -11.7  -7.8 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7

L 68 -10.0 -11.7 -11.7  -7.7  -9.4 -11.7 -11.7 -11.7 -11.7 -11.7

E 66  -9.6 -11.7 -11.7  -7.7  -8.0 -11.7 -11.7 -11.7 -11.7 -11.7

V 64  -9.2 -11.7 -11.7  -8.1  -7.0 -11.7 -11.7 -11.7 -11.7 -11.7

A 62  -8.9  -9.3 -11.7  -8.7  -6.5  -9.2 -11.7 -11.7 -11.7 -11.7

T 60  -8.6  -8.3 -11.7  -9.4  -6.3  -7.3 -11.7 -11.7 -11.7 -11.7

I 58  -8.4  -7.5 -11.7 -11.7  -6.4  -6.1 -11.7 -11.7 -11.7 -11.7

O 56  -8.2  -6.8  -9.2 -11.7  -6.9  -5.5  -7.6 -11.7 -11.7 -11.7

N 54  -8.0  -6.2  -7.8 -11.7  -7.6  -5.3  -5.9 -11.7 -11.7 -11.7

52  -7.9  -5.6  -6.6  -9.5  -8.5  -5.5  -4.9  -7.4 -11.7 -11.7

A 50  -7.8  -5.2  -5.5  -8.0  -9.0  -6.1  -4.5  -5.4  -9.9 -11.7

N 48  -7.8  -4.8  -4.5  -6.4  -8.7  -6.9  -4.6  -4.3  -6.5 -11.7

G 46  -7.8  -4.4  -3.7  -5.0  -7.5  -7.5  -5.1  -3.8  -4.6  -8.0

L 44  -7.8  -4.2  -3.0  -3.7  -5.8  -7.5  -5.9  -4.0  -3.6  -5.1

E 42  -7.9  -4.0  -2.5  -2.6  -4.1  -6.3  -6.5  -4.7  -3.4  -3.6

40  -8.0  -3.9  -2.0  -1.7  -2.6  -4.5  -6.2  -5.5  -3.8  -3.0

I 38  -8.1  -3.8  -1.7   -.9  -1.3  -2.7  -4.7  -5.7  -4.6  -3.2

N 36  -8.3  -3.9  -1.4   -.3   -.3  -1.1  -2.7  -4.6  -5.1  -3.9

34  -8.6  -3.9  -1.3    .1    .6    .2   -.9  -2.6  -4.2  -4.5

D 32  -8.8  -4.1  -1.2    .5   1.2   1.3    .7   -.5  -2.2  -3.8

E 30  -9.2  -4.2  -1.2    .7   1.7   2.2   2.0   1.3    .0  -1.7

G 28  -9.5  -4.5  -1.3    .8   2.1   2.8   3.0   2.7   1.9    .7

R 26 -10.0  -4.8  -1.5    .7   2.3   3.2   3.7   3.8   3.5   2.7

E 24 -11.7  -5.2  -1.8    .6   2.3   3.5   4.3   4.6   4.6   4.3

E 22 -11.7  -5.7  -2.2    .4   2.2   3.6   4.5   5.2   5.5   5.5

S 20 -11.7  -6.3  -2.7    .0   2.0   3.5   4.6   5.5   6.0   6.3

18 -11.7  -7.0  -3.3   -.5   1.6   3.2   4.5   5.5   6.2   6.7

16 -11.7  -7.8  -4.0  -1.2   1.0   2.8   4.2   5.3   6.2   6.9

14 -11.7  -8.7  -4.9  -2.0    .3   2.1   3.6   4.9   5.9   6.7

12 -11.7  -9.9  -6.0  -3.0   -.7   1.2   2.8   4.1   5.3   6.2

10 -11.7 -11.7  -7.3  -4.3  -2.0    .0   1.7   3.1   4.3   5.3

8 -11.7 -11.7  -9.1  -6.0  -3.6  -1.6    .1   1.6   2.8   3.9

6 -11.7 -11.7 -11.7  -8.3  -5.9  -3.8  -2.0   -.5    .8   1.9

4 -11.7 -11.7 -11.7 -11.7  -9.2  -7.1  -5.3  -3.7  -2.4  -1.2

2 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7  -9.5  -8.2  -7.0

0 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7

2     3     4     5     6     7     8     9    10    11

FREQUENCY IN MEGAHERTZ

ANTENNA EFFICIENCY

-1.7  -1.7  -1.7  -1.7  -1.7  -1.7  -1.7  -1.7  -1.7  -1.7

2     3     4     5     6     7     8     9    10    11

FREQUENCY IN MEGAHERTZ

Figure 42.  Sloping vee pattern.

METHOD 13   ICEPAC IC.10   PAGE  14

AZIMUTH     EX(1)     EX(2)     EX(3)     EX(4)  CONDUCT.  DIELECT.

.000    1.829      .000      .000      .000     .001     4.000

12    13    14    16    18    20    22    24    26    28    30

-11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 90

-11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7  -9.3 -11.7 -11.7 -11.7 88

-11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 86

-11.7 -11.7 -11.7  -9.8 -11.7 -11.7 -11.7 -11.7  -9.6 -11.7 -11.7 84

-11.7 -11.7  -9.3 -11.7 -11.7 -11.7 -11.7  -8.5 -11.7 -11.7 -11.7 82

-11.7 -11.7 -10.0 -11.7 -11.7 -11.7  -9.7 -11.7 -11.7 -11.7 -11.7 80

-11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7  -8.3 -11.7 -11.7 78

-11.7 -11.7 -11.7  -9.3 -11.7 -11.7 -11.7  -8.2  -9.7 -11.7 -11.7 76

-11.7  -8.9 -11.7  -8.0 -11.7 -11.7 -11.7  -9.1 -11.7  -8.4 -11.7 74

-11.7 -11.7  -8.6 -11.7 -11.7 -11.7 -11.7 -11.7  -7.1 -11.7 -11.7 72

-11.7 -11.7  -7.7 -11.7  -8.9 -11.7 -11.7  -8.9  -8.2 -11.7  -8.6 70

-11.7 -11.7  -9.5 -11.7  -8.0 -11.7 -11.7  -7.3 -11.7  -6.2 -11.7 68

-9.9 -11.7 -11.7  -7.1 -11.7 -11.7  -9.5 -11.7  -6.5 -11.7  -9.6 66

-11.7  -9.6 -11.7  -6.0 -11.7  -8.2 -11.7 -11.7  -6.4 -11.7  -6.8 64

-11.7  -8.2 -11.7  -8.1  -8.5 -11.7 -11.7  -7.1 -11.7  -4.4 -11.7 62

-11.7  -9.0  -7.2 -11.7  -5.2 -11.7  -9.0 -11.7  -7.4  -8.9  -5.4 60

-11.7 -11.7  -6.8 -11.7  -5.9  -9.2 -11.7 -11.7  -5.2 -11.7  -5.2 58

-11.7 -11.7  -8.8  -6.5 -11.7  -4.8 -11.7  -8.4 -11.7  -3.6 -11.7 56

-11.7 -11.7 -11.7  -4.4 -11.7  -5.5  -8.1  -9.7 -11.7  -6.3  -3.4 54

-11.7 -11.7 -11.7  -5.1  -5.3 -11.7  -4.6 -11.7  -6.2 -11.7  -3.5 52

-11.7  -9.6 -11.7  -8.8  -2.7 -11.7  -6.8  -6.3  -9.5  -4.9 -11.7 50

-11.7 -11.7  -8.9 -11.7  -3.2  -3.3 -11.7  -5.2  -9.4  -5.3  -4.1 48

-11.7 -11.7  -8.5 -11.7  -7.0  -1.3  -5.5 -11.7  -5.3 -11.7  -2.8 46

-9.5 -11.7 -11.7  -7.0 -11.7  -2.6   -.9  -9.1  -8.9  -6.2 -11.7 44

-5.7 -11.7 -11.7  -5.7  -8.5  -8.1   -.6  -1.2 -11.7  -7.1  -7.1 42

-3.6  -6.1 -11.7  -7.0  -4.2 -11.7  -4.0    .4  -1.6 -11.7  -5.7 40

-2.7  -3.6  -6.2 -11.7  -3.4  -3.8 -11.7  -1.9   1.0  -1.7 -11.7 38

-2.8  -2.5  -3.5  -9.9  -5.1  -1.4  -3.7  -8.5   -.8   1.4  -1.3 36

-3.5  -2.5  -2.3  -5.5  -8.6  -1.9   -.1  -3.4  -7.3   -.3   1.8 34

-4.1  -3.2  -2.3  -2.8  -7.7  -4.8    .2   1.0  -2.5  -6.9   -.3 32

-3.2  -3.7  -3.0  -1.8  -3.8  -7.9  -2.0   1.7   2.1  -1.1  -6.7 30

-.9  -2.5  -3.2  -2.0  -1.7  -4.6  -6.2   -.2   2.9   3.3    .7 28

1.5    .0  -1.6  -2.6  -1.3  -1.7  -4.9  -4.6    .9   3.9   4.4 26

3.6   2.5   1.1  -1.7  -1.7   -.7  -1.4  -4.5  -3.5   1.6   4.5 24

5.2   4.5   3.6   1.0  -1.3   -.8    .1   -.7  -3.3  -3.0   1.7 22

6.3   6.0   5.5   3.7   1.2   -.4    .3   1.1    .5  -1.6  -2.5 20

7.0   7.0   6.9   5.9   4.1   1.9    .8   1.6   2.4   2.1    .6 18

7.3   7.6   7.7   7.3   6.3   4.6   2.9   2.4   3.1   3.9   4.0 16

7.3   7.8   8.1   8.2   7.7   6.7   5.3   4.1   4.0   4.8   5.6 14

6.9   7.5   8.0   8.5   8.5   8.0   7.1   6.1   5.4   5.6   6.4 12

6.1   6.9   7.4   8.2   8.5   8.4   8.0   7.3   6.7   6.5   7.0 10

4.9   5.7   6.3   7.3   7.9   8.1   8.0   7.7   7.2   7.0   7.2  8

2.9   3.8   4.5   5.6   6.4   6.8   7.0   6.9   6.6   6.5   6.6  6

-.2    .7   1.5   2.7   3.6   4.2   4.5   4.6   4.5   4.5   4.6  4

-5.9  -5.0  -4.2  -2.9  -1.9  -1.2   -.9   -.7   -.6   -.6   -.5  2

-11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7  0

12    13    14    16    18    20    22    24    26    28    30

FREQUENCY IN MEGAHERTZ

ANTENNA EFFICIENCY

-1.7  -1.7  -1.7  -1.7  -1.7  -1.7  -1.7  -1.7  -1.7  -1.7  -1.7

12    13    14    16    18    20    22    24    26    28    30

FREQUENCY IN MEGAHERTZ

Figure 42a.  Sloping vee pattern continued.

0

METHOD 13   ICEPAC IC.10   PAGE  15

ITSA 1 ANTENNA PACKAGE             ANTENNA PATTERN

FREQUENCY RANGE  ANTENNA TYPE      HEIGHT    LENGTH     ANGLE

2.0 TO  30.0   INVERTED L        10.000    21.340      .000

2     3     4     5     6     7     8     9    10    11

90   -.9  15.3  13.3  15.7  16.3  15.0  12.2   7.9   1.8  -7.6

88  -2.5   7.8  13.3  15.7  16.3  15.0  12.1   7.6    .9  -8.6

86  -2.2   7.9  13.3  15.7  16.3  15.0  12.2   7.6    .9  -8.6

84  -1.9   7.9  13.3  15.7  16.3  15.0  12.2   7.6   1.0  -8.5

82  -1.4   7.9  13.3  15.7  16.3  15.0  12.2   7.7   1.0  -8.3

80   -.9   8.0  13.3  15.7  16.3  15.0  12.2   7.7   1.1  -8.2

78   -.3   8.0  13.3  15.7  16.3  15.0  12.3   7.8   1.2  -8.0

76    .3   8.1  13.3  15.7  16.3  15.1  12.3   7.8   1.3  -7.8

74    .8   8.1  13.2  15.7  16.3  15.1  12.4   7.9   1.4  -7.5

72   1.4   8.2  13.2  15.7  16.3  15.1  12.4   8.0   1.5  -7.3

E 70   1.9   8.3  13.2  15.6  16.3  15.1  12.5   8.1   1.6  -7.1

L 68   2.4   8.3  13.2  15.6  16.3  15.2  12.5   8.2   1.8  -6.8

E 66   2.9   8.4  13.1  15.6  16.3  15.2  12.6   8.3   1.9  -6.6

V 64   3.4   8.5  13.1  15.6  16.2  15.2  12.6   8.4   2.1  -6.4

A 62   3.8   8.5  13.1  15.5  16.2  15.2  12.7   8.5   2.2  -6.2

T 60   4.2   8.6  13.0  15.5  16.2  15.2  12.7   8.6   2.4  -5.9

I 58   4.6   8.7  13.0  15.4  16.2  15.2  12.8   8.7   2.6  -5.7

O 56   4.9   8.7  12.9  15.4  16.2  15.2  12.8   8.8   2.7  -5.5

N 54   5.2   8.8  12.8  15.3  16.1  15.2  12.9   8.9   2.9  -5.3

52   5.5   8.8  12.8  15.2  16.1  15.2  12.9   9.0   3.0  -5.2

A 50   5.8   8.9  12.7  15.1  16.0  15.2  12.9   9.1   3.2  -5.0

N 48   6.0   8.9  12.6  15.0  15.9  15.2  13.0   9.1   3.3  -4.8

G 46   6.2   8.9  12.4  14.9  15.8  15.1  12.9   9.2   3.4  -4.7

L 44   6.4   8.9  12.3  14.7  15.7  15.0  12.9   9.2   3.5  -4.6

E 42   6.6   8.9  12.2  14.6  15.6  14.9  12.9   9.3   3.6  -4.5

40   6.7   8.9  12.0  14.4  15.4  14.8  12.8   9.3   3.7  -4.4

I 38   6.8   8.8  11.8  14.2  15.3  14.7  12.7   9.2   3.7  -4.3

N 36   6.9   8.8  11.6  14.0  15.1  14.5  12.6   9.2   3.8  -4.3

34   6.9   8.7  11.4  13.7  14.8  14.4  12.5   9.1   3.8  -4.3

D 32   7.0   8.6  11.1  13.4  14.6  14.1  12.3   9.0   3.7  -4.3

E 30   6.9   8.4  10.8  13.1  14.3  13.9  12.1   8.8   3.6  -4.4

G 28   6.9   8.3  10.5  12.7  13.9  13.6  11.8   8.6   3.5  -4.5

R 26   6.8   8.0  10.1  12.3  13.5  13.2  11.5   8.4   3.3  -4.6

E 24   6.7   7.8   9.6  11.8  13.1  12.8  11.2   8.0   3.0  -4.8

E 22   6.5   7.5   9.2  11.3  12.6  12.3  10.7   7.7   2.7  -5.1

S 20   6.2   7.1   8.6  10.7  12.0  11.7  10.2   7.2   2.3  -5.5

18   5.9   6.6   8.0  10.0  11.3  11.1   9.6   6.6   1.8  -5.9

16   5.4   6.1   7.2   9.2  10.5  10.3   8.9   6.0   1.2  -6.5

14   4.9   5.4   6.3   8.2   9.5   9.4   8.0   5.1    .5  -7.2

12   4.2   4.6   5.3   7.0   8.4   8.3   6.9   4.1   -.5  -8.1

10   3.3   3.6   4.0   5.6   7.0   6.9   5.6   2.9  -1.7  -9.3

8   2.0   2.2   2.4   3.9   5.2   5.2   3.9   1.2  -3.2 -10.0

6    .2    .3    .2   1.6   2.9   2.9   1.7   -.9  -5.4 -10.0

4  -2.6  -2.7  -3.0  -1.8   -.5   -.4  -1.6  -4.2  -8.5 -10.0

2  -7.8  -8.0  -8.6  -7.6  -6.3  -6.2  -7.3  -9.9 -10.0 -10.0

0 -15.2 -12.2 -10.8 -10.3 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0

2     3     4     5     6     7     8     9    10    11

FREQUENCY IN MEGAHERTZ

ANTENNA EFFICIENCY

-5.2  -2.2   -.8   -.3    .0    .0    .0    .0    .0    .0

2     3     4     5     6     7     8     9    10    11

FREQUENCY IN MEGAHERTZ

Figure 44.  Inverted L pattern.

METHOD 13   ICEPAC IC.10   PAGE  16

AZIMUTH     EX(1)     EX(2)     EX(3)     EX(4)  CONDUCT.  DIELECT.

.000     .000      .000      .000      .000     .001     4.000

12    13    14    16    18    20    22    24    26    28    30

-3.9 -10.0 -10.0  -5.3   1.2  11.5  11.9   3.2 -10.0 -10.0  -9.9 90

-10.0 -10.0 -10.0 -10.0    .0  11.3  11.8   2.1 -10.0 -10.0 -10.0 88

-10.0 -10.0 -10.0 -10.0    .0  11.3  11.8   2.1 -10.0 -10.0 -10.0 86

-10.0 -10.0 -10.0 -10.0    .0  11.3  11.8   2.2 -10.0 -10.0 -10.0 84

-10.0 -10.0 -10.0 -10.0    .0  11.2  11.8   2.3 -10.0 -10.0 -10.0 82

-10.0 -10.0 -10.0 -10.0    .0  11.1  11.9   2.4 -10.0 -10.0 -10.0 80

-10.0 -10.0 -10.0 -10.0    .0  11.1  11.9   2.5  -9.1 -10.0  -9.4 78

-10.0 -10.0 -10.0 -10.0    .0  11.0  11.9   2.6  -7.9 -10.0  -8.2 76

-10.0 -10.0 -10.0  -9.3    .0  10.8  11.9   2.8  -6.9  -9.1  -7.3 74

-10.0 -10.0 -10.0  -8.4   -.1  10.7  11.9   2.9  -6.0  -7.9  -6.5 72

-10.0 -10.0 -10.0  -7.6   -.1  10.5  11.8   3.0  -5.3  -6.8  -5.9 70

-10.0 -10.0 -10.0  -7.0    .0  10.2  11.8   3.2  -4.6  -5.7  -5.4 68

-10.0 -10.0 -10.0  -6.4    .0   9.9  11.7   3.3  -4.1  -4.8  -5.0 66

-9.8  -9.4 -10.0  -6.0    .0   9.6  11.6   3.4  -3.6  -3.9  -4.7 64

-9.4  -8.7 -10.0  -5.5    .1   9.1  11.4   3.5  -3.3  -3.1  -4.4 62

-8.9  -8.1  -9.4  -5.2    .2   8.7  11.2   3.5  -3.0  -2.3  -4.3 60

-8.5  -7.5  -8.7  -4.9    .3   8.1  10.9   3.6  -2.7  -1.6  -4.1 58

-8.2  -7.0  -8.0  -4.7    .5   7.5  10.5   3.6  -2.6   -.9  -4.0 56

-7.9  -6.5  -7.3  -4.5    .7   6.8  10.1   3.5  -2.5   -.3  -3.9 54

-7.7  -6.0  -6.7  -4.4   1.0   6.1   9.6   3.4  -2.4    .2  -3.7 52

-7.5  -5.6  -6.0  -4.3   1.3   5.5   9.0   3.3  -2.5    .6  -3.4 50

-7.3  -5.2  -5.4  -4.3   1.6   5.1   8.4   3.1  -2.5   1.0  -3.1 48

-7.2  -4.8  -4.9  -4.2   2.0   5.2   7.8   2.9  -2.7   1.4  -2.8 46

-7.1  -4.5  -4.4  -4.2   2.4   5.6   7.2   2.8  -2.8   1.6  -2.4 44

-7.0  -4.2  -3.9  -4.2   2.8   6.5   7.0   2.6  -3.0   1.8  -2.0 42

-7.0  -3.9  -3.4  -4.1   3.3   7.5   7.1   2.4  -3.2   1.9  -1.7 40

-7.0  -3.7  -3.0  -4.1   3.7   8.6   7.5   2.4  -3.4   1.8  -1.4 38

-7.1  -3.5  -2.6  -4.0   4.0   9.7   8.4   2.5  -3.6   1.8  -1.2 36

-7.2  -3.4  -2.2  -3.8   4.4  10.7   9.3   2.7  -3.7   1.6  -1.2 34

-7.3  -3.3  -1.9  -3.7   4.7  11.6  10.3   3.0  -3.8   1.3  -1.2 32

-7.5  -3.2  -1.7  -3.5   4.9  12.4  11.3   3.4  -3.8    .9  -1.5 30

-7.7  -3.2  -1.5  -3.3   5.1  13.0  12.1   3.9  -3.8    .5  -1.9 28

-8.0  -3.2  -1.4  -3.1   5.2  13.5  12.8   4.4  -3.8    .1  -2.4 26

-8.3  -3.3  -1.3  -2.9   5.3  13.9  13.4   4.8  -3.8   -.4  -3.1 24

-8.7  -3.4  -1.3  -2.8   5.2  14.1  13.8   5.1  -3.9   -.9  -4.0 22

-9.1  -3.7  -1.4  -2.8   5.0  14.2  14.0   5.3  -4.0  -1.3  -4.9 20

-9.7  -4.0  -1.6  -2.8   4.7  14.1  14.1   5.4  -4.1  -1.8  -5.8 18

-10.0  -4.4  -1.9  -3.0   4.3  13.8  14.0   5.3  -4.4  -2.1  -6.5 16

-10.0  -5.0  -2.4  -3.3   3.7  13.4  13.6   4.9  -4.9  -2.6  -7.1 14

-10.0  -5.7  -3.0  -3.8   2.8  12.6  13.0   4.4  -5.5  -3.1  -7.5 12

-10.0  -6.7  -3.9  -4.6   1.7  11.6  12.0   3.4  -6.4  -3.8  -8.0 10

-10.0  -8.0  -5.2  -5.8    .2  10.2  10.6   2.1  -7.7  -4.8  -8.8  8

-10.0  -9.9  -7.0  -7.5  -2.0   8.1   8.6    .1  -9.5  -6.5 -10.0  6

-10.0 -10.0  -9.9 -10.0  -5.1   4.9   5.5  -3.0 -10.0  -9.2 -10.0  4

-10.0 -10.0 -10.0 -10.0 -10.0   -.8   -.2  -8.7 -10.0 -10.0 -10.0  2

-10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0 -10.0  0

12    13    14    16    18    20    22    24    26    28    30

FREQUENCY IN MEGAHERTZ

ANTENNA EFFICIENCY

.0    .0    .0    .0    .0    .0    .0    .0    .0    .0    .0

12    13    14    16    18    20    22    24    26    28    30

FREQUENCY IN MEGAHERTZ

Figure 44a.  Inverted L pattern continued.

0

METHOD 13   ICEPAC IC.10   PAGE  17

ITSA 1 ANTENNA PACKAGE             ANTENNA PATTERN

FREQUENCY RANGE  ANTENNA TYPE      HEIGHT    LENGTH     ANGLE

2.0 TO  30.0   TER SLOP RHM      16.760    88.390    22.500

2     3     4     5     6     7     8     9    10    11

90  -5.5 -11.7 -11.7  -2.4 -11.7 -11.7  -4.9 -11.7 -11.7  -6.6

88  -4.9 -11.7 -11.7  -2.3  -7.8 -11.7  -5.8  -8.5 -11.7  -9.1

86  -4.4 -11.7 -11.7  -2.5  -5.2 -11.7  -7.7  -6.2 -11.7 -11.7

84  -3.9 -11.7 -11.7  -3.2  -3.3 -11.7 -11.7  -4.9 -11.7 -11.7

82  -3.5  -9.8 -11.7  -4.3  -2.0 -11.7 -11.7  -4.8 -11.7 -11.7

80  -3.2  -7.8 -11.7  -5.9  -1.2  -8.6 -11.7  -6.0  -7.0 -11.7

78  -2.9  -6.1 -11.7  -8.1  -1.0  -5.3 -11.7  -8.7  -4.7 -11.7

76  -2.6  -4.6 -11.7 -11.7  -1.2  -2.9 -11.7 -11.7  -4.3 -11.7

74  -2.4  -3.3 -11.7 -11.7  -1.9  -1.3 -10.0 -11.7  -5.6  -5.4

72  -2.3  -2.2 -11.7 -11.7  -3.3   -.3  -6.1 -11.7  -9.2  -3.6

E 70  -2.2  -1.2  -9.8 -11.7  -5.3    .0  -3.1 -11.7 -11.7  -4.1

L 68  -2.2   -.4  -6.9 -11.7  -8.2   -.4  -1.0  -8.7 -11.7  -7.1

E 66  -2.3    .3  -4.4 -11.7 -11.7  -1.4    .2  -5.0 -11.7 -11.7

V 64  -2.3    .9  -2.4 -11.7 -11.7  -3.2    .7  -2.0  -9.4 -11.7

A 62  -2.5   1.4   -.7 -11.7 -11.7  -5.8    .3    .0  -6.0 -11.7

T 60  -2.6   1.7    .7  -6.4 -11.7  -9.7   -.8   1.0  -2.6  -8.8

I 58  -2.9   2.0   1.9  -3.3 -11.7 -11.7  -2.9   1.2   -.1  -6.1

O 56  -3.1   2.2   2.9   -.8 -11.7 -11.7  -6.1    .4   1.3  -2.5

N 54  -3.5   2.2   3.6   1.2  -6.0 -11.7 -11.7  -1.5   1.7    .3

52  -3.8   2.2   4.3   2.9  -2.3 -11.7 -11.7  -4.6    .9   1.8

A 50  -4.3   2.2   4.7   4.3    .6  -7.4 -11.7  -9.1  -1.1   2.0

N 48  -4.7   2.0   5.0   5.3   2.9  -2.8 -11.7 -11.7  -4.4   1.0

G 46  -5.3   1.7   5.2   6.2   4.8    .8  -6.7 -11.7  -9.4  -1.5

L 44  -5.8   1.4   5.3   6.8   6.3   3.6  -1.8 -11.7 -11.7  -5.7

E 42  -6.5   1.0   5.2   7.2   7.4   5.8   2.0  -4.2 -11.7 -11.7

40  -7.1    .5   5.0   7.5   8.2   7.5   5.0    .6  -5.9 -11.7

I 38  -7.8    .0   4.8   7.6   8.8   8.7   7.3   4.3   -.5  -6.7

N 36  -8.6   -.6   4.4   7.5   9.2   9.7   9.0   7.1   3.8  -1.0

34  -9.4  -1.3   3.9   7.3   9.4  10.4  10.3   9.3   7.1   3.7

D 32 -11.7  -2.1   3.3   7.0   9.4  10.8  11.3  10.9   9.6   7.4

E 30 -11.7  -2.9   2.7   6.5   9.2  10.9  11.9  12.0  11.5  10.1

G 28 -11.7  -3.8   1.9   5.9   8.8  10.9  12.2  12.8  12.8  12.1

R 26 -11.7  -4.8   1.0   5.2   8.3  10.6  12.2  13.2  13.7  13.6

E 24 -11.7  -5.8    .1   4.4   7.7  10.2  12.1  13.4  14.2  14.5

E 22 -11.7  -6.9   -.9   3.5   6.9   9.6  11.7  13.2  14.3  15.0

S 20 -11.7  -8.1  -2.1   2.5   6.0   8.8  11.1  12.9  14.2  15.2

18 -11.7  -9.4  -3.3   1.3   5.0   7.9  10.3  12.3  13.8  15.0

16 -11.7 -11.7  -4.6    .0   3.8   6.8   9.4  11.4  13.2  14.6

14 -11.7 -11.7  -6.1  -1.4   2.4   5.6   8.2  10.4  12.3  13.8

12 -11.7 -11.7  -7.7  -2.9    .9   4.1   6.8   9.1  11.1  12.8

10 -11.7 -11.7  -9.5  -4.7   -.8   2.4   5.2   7.6   9.6  11.4

8 -11.7 -11.7 -11.7  -6.9  -2.9    .4   3.2   5.6   7.8   9.6

6 -11.7 -11.7 -11.7  -9.5  -5.5  -2.2    .7   3.2   5.3   7.3

4 -11.7 -11.7 -11.7 -11.7  -9.1  -5.7  -2.8   -.3   1.9   3.9

2 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7  -8.8  -6.2  -4.0  -2.0

0 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7

2     3     4     5     6     7     8     9    10    11

FREQUENCY IN MEGAHERTZ

ANTENNA EFFICIENCY

-1.7  -1.7  -1.7  -1.7  -1.7  -1.7  -1.7  -1.7  -1.7  -1.7

2     3     4     5     6     7     8     9    10    11

FREQUENCY IN MEGAHERTZ

Figure 46.  Sloping rhombic pattern.

METHOD 13   ICEPAC IC.10   PAGE  18

AZIMUTH     EX(1)     EX(2)     EX(3)     EX(4)  CONDUCT.  DIELECT.

.000    8.382      .000      .000      .000     .001     4.000

12    13    14    16    18    20    22    24    26    28    30

-11.7 -11.7  -8.3 -11.7  -5.6 -11.7 -11.7  -8.1 -11.7  -6.9  -6.4 90

-8.0 -11.7 -11.7  -9.3  -5.8 -11.7  -5.8 -11.7 -11.7  -4.2 -11.7 88

-5.5 -11.7 -11.7  -5.6  -8.9 -11.7  -7.3 -11.7  -5.3 -11.7  -7.4 86

-6.2  -8.3 -11.7  -4.5 -11.7  -5.5 -11.7  -9.6  -9.3 -11.7  -2.4 84

-11.7  -4.9 -11.7  -6.4 -11.7  -4.6 -11.7  -5.1 -11.7  -4.2 -11.7 82

-11.7  -4.9  -6.9 -11.7  -6.0  -8.2 -11.7 -11.7 -11.7  -4.4 -11.7 80

-11.7  -8.7  -4.0 -11.7  -3.7 -11.7  -3.8 -11.7  -4.3 -11.7  -2.2 78

-11.7 -11.7  -4.2  -9.8  -4.3 -11.7  -6.3 -11.7  -9.1 -11.7  -3.6 76

-11.7 -11.7  -8.5  -5.9  -7.6  -4.9 -11.7  -3.9 -11.7  -2.7 -11.7 74

-11.7 -11.7 -11.7  -2.9 -11.7  -2.6 -11.7  -6.1 -11.7  -5.7  -7.0 72

-5.3 -11.7 -11.7  -2.6  -7.8  -4.2  -7.1 -11.7  -3.2 -11.7   -.5 70

-2.9  -9.6 -11.7  -5.7  -3.6  -7.9  -1.9 -11.7  -7.6  -8.8  -6.6 68

-3.3  -3.7 -11.7 -11.7  -1.4  -8.6  -3.2  -6.8 -11.7  -2.1 -11.7 66

-6.8  -1.9  -5.3  -9.2  -2.1  -4.3  -6.9  -1.2 -11.7  -9.1  -2.8 64

-11.7  -3.2  -1.7  -7.8  -5.7  -1.0  -8.2  -4.4  -3.2 -11.7  -1.5 62

-11.7  -8.8  -1.4  -5.8  -8.1   -.7  -4.3  -7.1  -1.6  -9.6  -8.6 60

-9.0 -11.7  -4.8  -1.6  -6.0  -3.1   -.2  -8.4  -9.1   -.6 -11.7 58

-8.0  -9.4 -11.7    .3  -3.6  -5.9    .2  -2.6  -6.6  -7.7  -1.0 56

-5.5  -7.4 -11.7  -1.2   -.3  -4.4  -2.1   1.1  -7.9  -6.7  -3.4 54

-1.5  -7.7  -6.4  -8.6   1.5  -1.3  -4.4    .4    .8 -11.7  -9.2 52

1.3  -3.8  -7.9 -11.7    .1   1.6  -2.3  -2.3   1.7   -.3 -11.7 50

2.5    .4  -6.4  -4.6  -6.9   2.5   1.2  -2.8   -.6   2.4  -1.2 48

2.1   2.5   -.6  -5.8  -8.3    .1   3.2    .7  -2.1    .1   2.8 46

.3   2.9   2.5 -11.7  -2.5  -7.6   2.7   3.5    .1  -1.1    .0 44

-3.3   1.5   3.4  -1.8  -4.7  -3.1  -1.7   4.0   3.6   -.1   -.3 42

-8.8  -1.7   2.3   2.9 -11.7   -.7  -3.8   1.4   4.8   3.8   -.1 40

-11.7  -7.1   -.8   4.3    .0  -6.6    .7  -2.4   3.1   5.4   4.1 38

-6.5 -11.7  -6.2   3.2   4.4  -4.9   -.9   1.4   -.4   4.2   6.0 36

-.8  -5.7  -9.2   -.3   5.1   3.6  -7.0   1.6   2.4   1.3   5.0 34

4.1   -.1  -4.4  -6.3   3.1   6.0   2.9  -3.6   3.2   3.5   2.6 32

7.9   4.9   1.1  -6.2  -1.9   5.1   6.5   2.8   -.8   4.4   4.8 30

10.8   8.8   6.0   -.8  -6.5   1.0   6.4   7.1   3.6   1.0   5.2 28

12.9  11.7   9.9   4.6  -1.6  -5.2   2.9   7.3   7.8   4.8   2.1 26

14.4  13.8  12.7   9.1   3.8  -1.7  -3.7   3.9   7.9   8.6   6.3 24

15.3  15.2  14.7  12.5   8.7   3.7  -1.4  -2.6   4.4   8.3   9.4 22

15.8  16.1  16.1  15.0  12.6   8.7   4.0   -.6  -2.0   4.4   8.5 20

15.9  16.5  16.9  16.7  15.3  12.8   9.1   4.7    .3  -1.6   3.9 18

15.7  16.6  17.2  17.6  17.2  15.7  13.2   9.7   5.5   1.3  -1.1 16

15.1  16.2  17.0  18.0  18.2  17.6  16.1  13.8  10.5   6.3   2.1 14

14.2  15.4  16.4  17.9  18.6  18.6  17.9  16.5  14.3  11.1   7.1 12

13.0  14.3  15.4  17.2  18.3  18.8  18.7  17.9  16.6  14.5  11.5 10

11.3  12.7  13.9  15.9  17.3  18.1  18.5  18.3  17.5  16.1  14.1  8

9.0  10.5  11.8  13.9  15.5  16.6  17.2  17.4  17.1  16.2  14.8  6

5.6   7.2   8.5  10.8  12.5  13.8  14.6  15.0  15.0  14.5  13.5  4

-.3   1.3   2.7   5.1   6.9   8.3   9.2   9.7   9.8   9.5   8.7  2

-11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7  0

12    13    14    16    18    20    22    24    26    28    30

FREQUENCY IN MEGAHERTZ

ANTENNA EFFICIENCY

-1.7  -1.7  -1.7  -1.7  -1.7  -1.7  -1.7  -1.7  -1.7  -1.7  -1.7

12    13    14    16    18    20    22    24    26    28    30

FREQUENCY IN MEGAHERTZ

Figure 46a.  Sloping rhombic pattern continued.

0

METHOD 13   ICEPAC IC.10   PAGE  19

ITSA 1 ANTENNA PACKAGE             ANTENNA PATTERN

FREQUENCY RANGE  ANTENNA TYPE      HEIGHT    LENGTH     ANGLE

2.0 TO  30.0   TER INTR RHM      20.000   114.000    70.000

2     3     4     5     6     7     8     9    10    11

90 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7  -6.3 -11.7 -11.7

88  -9.1 -11.7  -9.7 -11.7 -11.7 -11.7 -11.7  -9.4  -7.3 -11.7

86  -7.6 -11.7  -9.1 -11.7 -11.7 -11.7 -11.7 -11.7  -4.9 -11.7

84  -6.2 -11.7  -9.1 -11.7 -11.7 -11.7 -11.7 -11.7  -5.5 -11.7

82  -4.9 -11.7  -9.5 -11.7 -11.7 -11.7 -11.7 -11.7  -9.2  -6.1

80  -3.9 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7  -4.8

78  -2.9 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7  -7.0

76  -2.1 -11.7 -11.7  -9.3 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7

74  -1.3 -11.7 -11.7  -8.6 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7

72   -.7 -11.7 -11.7  -8.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7

E 70   -.2  -7.4 -11.7  -9.5 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7

L 68    .3  -4.9 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7

E 66    .6  -2.8 -11.7 -11.7  -9.8 -11.7 -11.7 -11.7 -11.7 -11.7

V 64    .9  -1.0 -11.7 -11.7  -9.6 -11.7 -11.7 -11.7 -11.7 -11.7

A 62   1.0    .5 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7

T 60   1.1   1.8  -7.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7

I 58   1.1   2.8  -4.0 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7

O 56   1.1   3.7  -1.1 -11.7 -11.7 -11.7 -11.7  -9.5 -11.7 -11.7

N 54    .9   4.3   1.2 -11.7 -11.7 -11.7 -11.7  -8.4  -7.8 -11.7

52    .7   4.9   3.1  -5.6 -11.7 -11.7 -11.7  -8.9  -4.6  -8.2

A 50    .4   5.2   4.6  -1.5 -11.7 -11.7 -11.7 -11.7  -3.6  -2.9

N 48    .1   5.5   5.9   1.6  -9.5 -11.7 -11.7 -11.7  -4.6   -.6

G 46   -.4   5.5   6.8   4.1  -3.6 -11.7 -11.7 -11.7  -7.6   -.5

L 44   -.9   5.5   7.5   6.1    .7 -11.7 -11.7 -11.7 -11.7  -2.6

E 42  -1.4   5.4   8.1   7.6   4.0  -3.9 -11.7 -11.7 -11.7  -6.9

40  -2.1   5.1   8.4   8.8   6.5   1.1  -9.4 -11.7 -11.7 -11.7

I 38  -2.8   4.7   8.5   9.7   8.5   4.8  -2.1 -11.7 -11.7 -11.7

N 36  -3.6   4.2   8.5  10.3  10.0   7.7   3.0  -5.2 -11.7 -11.7

34  -4.5   3.6   8.3  10.6  11.1   9.9   6.8   1.3  -7.8 -11.7

D 32  -5.4   2.9   7.9  10.7  11.9  11.5   9.6   6.0    .1  -9.3

E 30  -6.4   2.1   7.4  10.6  12.3  12.7  11.8   9.5   5.6   -.4

G 28  -7.5   1.2   6.8  10.4  12.5  13.4  13.3  12.0   9.6   5.7

R 26  -8.7    .2   6.0   9.9  12.4  13.9  14.4  13.9  12.5  10.0

E 24 -11.7   -.9   5.1   9.3  12.1  14.0  15.0  15.2  14.6  13.2

E 22 -11.7  -2.1   4.1   8.4  11.6  13.8  15.2  16.0  16.0  15.5

S 20 -11.7  -3.5   2.9   7.5  10.9  13.4  15.2  16.3  16.9  17.0

18 -11.7  -4.9   1.6   6.3   9.9  12.7  14.8  16.3  17.3  17.9

16 -11.7  -6.5    .1   5.0   8.8  11.8  14.1  15.9  17.3  18.2

14 -11.7  -8.2  -1.5   3.5   7.5  10.6  13.2  15.2  16.8  18.1

12 -11.7 -11.7  -3.3   1.9   5.9   9.2  11.9  14.2  16.0  17.5

10 -11.7 -11.7  -5.3   -.1   4.1   7.5  10.4  12.8  14.8  16.5

8 -11.7 -11.7  -7.6  -2.3   2.0   5.5   8.4  10.9  13.1  14.9

6 -11.7 -11.7 -11.7  -5.0   -.7   2.9   5.9   8.5  10.8  12.7

4 -11.7 -11.7 -11.7  -8.7  -4.3   -.7   2.4   5.1   7.4   9.4

2 -11.7 -11.7 -11.7 -11.7 -11.7  -6.7  -3.6   -.9   1.5   3.6

0 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7

2     3     4     5     6     7     8     9    10    11

FREQUENCY IN MEGAHERTZ

ANTENNA EFFICIENCY

-1.7  -1.7  -1.7  -1.7  -1.7  -1.7  -1.7  -1.7  -1.7  -1.7

2     3     4     5     6     7     8     9    10    11

FREQUENCY IN MEGAHERTZ

Figure 48.  Interlaced rhombic pattern.

METHOD 13   ICEPAC IC.10   PAGE  20

AZIMUTH     EX(1)     EX(2)     EX(3)     EX(4)  CONDUCT.  DIELECT.

.000    4.000    33.000      .000      .000     .001     4.000

12    13    14    16    18    20    22    24    26    28    30

-7.8 -11.7 -11.7 -11.7 -11.7  -6.3  -8.3 -11.7 -11.7  -9.0  -9.9 90

-7.3 -11.7 -11.7 -11.7 -11.7  -6.9 -11.7 -11.7 -11.7 -11.7 -11.7 88

-11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7  -6.3 -11.7 -11.7 -11.7 86

-11.7  -8.2 -11.7  -8.3 -11.7 -11.7  -5.5 -11.7 -11.7 -11.7 -11.7 84

-11.7 -11.7  -9.7 -11.7 -11.7  -8.8 -11.7 -11.7  -5.9 -11.7 -11.7 82

-11.7 -11.7  -7.1 -11.7 -11.7 -11.7 -11.7  -9.7  -8.9 -11.7 -11.7 80

-7.3 -11.7 -11.7 -11.7  -6.3 -11.7 -11.7 -11.7 -11.7  -3.4 -11.7 78

-4.8 -11.7 -11.7  -7.3  -8.6 -11.7 -11.7 -11.7  -6.7  -9.2 -11.7 76

-6.4  -6.8 -11.7  -3.1 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7  -3.2 74

-11.7  -4.4 -11.7  -5.9 -11.7  -8.1 -11.7 -11.7 -11.7  -2.6 -11.7 72

-11.7  -6.5  -4.7 -11.7  -3.5 -11.7 -11.7 -11.7 -11.7 -11.7  -7.8 70

-11.7 -11.7  -3.8 -11.7  -1.9 -11.7  -8.6 -11.7 -11.7 -11.7  -4.2 68

-11.7 -11.7  -7.9  -9.1  -9.0  -5.3 -11.7  -7.0 -11.7 -11.7 -11.7 66

-11.7 -11.7 -11.7  -3.0 -11.7  -1.3 -11.7  -5.2 -11.7 -11.7 -11.7 64

-11.7 -11.7 -11.7  -3.4 -11.7  -7.4  -3.8 -11.7  -2.1 -11.7 -11.7 62

-11.7 -11.7 -11.7 -11.7  -4.6 -11.7   -.7 -11.7  -7.6  -3.7 -11.7 60

-11.7 -11.7 -11.7 -11.7  -3.8 -11.7  -9.6    .2 -11.7  -2.9  -6.9 58

-11.7 -11.7  -9.3 -11.7  -9.5  -6.8 -11.7  -1.2  -2.4 -11.7  -1.1 56

-11.7 -11.7  -8.6  -9.9 -11.7  -8.1  -9.4 -11.7   1.5  -6.7 -11.7 54

-11.7 -11.7 -11.7  -7.0 -11.7 -11.7  -8.4 -11.7  -9.5   1.3 -11.7 52

-9.4 -11.7 -11.7  -7.4  -8.8 -11.7 -11.7  -3.4 -11.7  -5.7   -.7 50

-2.1 -11.7 -11.7  -9.6  -6.8 -11.7 -11.7  -8.2  -1.5 -11.7  -6.2 48

1.4  -1.5 -11.7 -11.7  -9.3  -6.6 -11.7 -11.7  -1.9  -1.1 -11.7 46

2.2   2.9   -.6 -11.7 -11.7  -7.6  -6.9 -11.7 -11.7    .6   -.9 44

.8   4.2   4.2  -7.8 -11.7 -11.7  -3.0  -6.8 -11.7 -10.0   1.2 42

-3.0   3.2   5.8   2.4 -11.7 -11.7  -8.6    .4  -5.8 -11.7 -11.7 40

-11.7   -.5   4.8   7.0  -1.4 -11.7 -11.7  -1.8   3.0  -3.7 -11.7 38

-11.7  -7.4   1.1   8.2   6.4  -5.9 -11.7 -11.7   1.9   5.1   -.9 36

-11.7 -11.7  -6.3   6.5   9.6   5.3  -9.7 -11.7  -9.4   4.0   6.9 34

-11.7 -11.7 -11.7   1.6   9.4  10.2   4.4 -11.7 -11.7  -8.2   4.7 32

-9.6 -11.7 -11.7  -8.2   5.9  11.0  10.5   4.2 -11.7 -11.7 -11.7 30

-.1  -8.6 -11.7 -11.7  -1.9   8.4  12.1  11.0   5.0  -8.6 -11.7 28

6.3   1.0  -6.5 -11.7 -11.7   1.4   9.8  12.8  11.9   6.7  -4.0 26

10.9   7.5   2.8 -11.7 -11.7 -11.7   2.9  10.4  13.4  13.0   9.1 24

14.2  12.2   9.2   -.1 -11.7 -11.7 -11.7   3.1  10.5  13.8  14.2 22

16.5  15.4  13.7   8.1  -1.7 -11.7 -11.7 -11.7   2.0   9.8  13.8 20

17.9  17.6  16.7  13.5   7.8  -1.6 -11.7 -11.7 -11.7   -.5   8.3 18

18.7  18.9  18.7  17.2  14.0   8.6    .2 -11.7 -11.7 -11.7  -5.1 16

19.0  19.6  19.8  19.5  17.9  15.0  10.4   3.4  -7.2 -11.7 -11.7 14

18.7  19.6  20.2  20.8  20.4  19.0  16.4  12.6   7.0  -1.2 -11.7 12

17.9  19.0  20.0  21.2  21.6  21.2  20.0  17.9  14.8  10.4   4.1 10

16.5  17.9  19.0  20.7  21.7  22.0  21.7  20.7  19.1  16.5  12.9  8

14.5  15.9  17.2  19.3  20.7  21.5  21.8  21.5  20.7  19.3  17.1  6

11.2  12.8  14.2  16.5  18.2  19.3  20.0  20.2  19.9  19.1  17.7  4

5.5   7.1   8.6  11.0  12.8  14.2  15.0  15.5  15.5  15.0  14.1  2

-11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7 -11.7  0

12    13    14    16    18    20    22    24    26    28    30

FREQUENCY IN MEGAHERTZ

ANTENNA EFFICIENCY

-1.7  -1.7  -1.7  -1.7  -1.7  -1.7  -1.7  -1.7  -1.7  -1.7  -1.7

12    13    14    16    18    20    22    24    26    28    30

FREQUENCY IN MEGAHERTZ

Figure 48a.  Interlaced rhombic pattern continued.

0

METHOD 13   ICEPAC IC.10   PAGE  21

ITSA 1 ANTENNA PACKAGE             ANTENNA PATTERN

FREQUENCY RANGE  ANTENNA TYPE      HEIGHT    LENGTH     ANGLE

2.0 TO  30.0   CONST. GAIN         .000      .000      .000

2     3     4     5     6     7     8     9    10    11

90  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0

88  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0

86  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0

84  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0

82  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0

80  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0

78  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0

76  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0

74  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0

72  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0

E 70  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0

L 68  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0

E 66  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0

V 64  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0

A 62  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0

T 60  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0

I 58  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0

O 56  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0

N 54  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0

52  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0

A 50  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0

N 48  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0

G 46  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0

L 44  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0

E 42  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0

40  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0

I 38  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0

N 36  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0

34  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0

D 32  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0

E 30  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0

G 28  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0

R 26  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0

E 24  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0

E 22  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0

S 20  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0

18  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0

16  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0

14  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0

12  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0

10  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0

8  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0

6  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0

4  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0

2  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0

0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0

2     3     4     5     6     7     8     9    10    11

FREQUENCY IN MEGAHERTZ

ANTENNA EFFICIENCY

.0    .0    .0    .0    .0    .0    .0    .0    .0    .0

2     3     4     5     6     7     8     9    10    11

FREQUENCY IN MEGAHERTZ

Figure 50.  Constant gain pattern.

METHOD 13   ICEPAC IC.10   PAGE  22

AZIMUTH     EX(1)     EX(2)     EX(3)     EX(4)  CONDUCT.  DIELECT.

.000     .000      .000      .000      .000     .001     4.000

12    13    14    16    18    20    22    24    26    28    30

10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0 90

10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0 88

10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0 86

10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0 84

10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0 82

10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0 80

10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0 78

10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0 76

10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0 74

10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0 72

10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0 70

10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0 68

10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0 66

10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0 64

10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0 62

10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0 60

10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0 58

10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0 56

10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0 54

10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0 52

10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0 50

10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0 48

10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0 46

10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0 44

10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0 42

10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0 40

10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0 38

10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0 36

10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0 34

10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0 32

10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0 30

10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0 28

10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0 26

10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0 24

10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0 22

10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0 20

10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0 18

10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0 16

10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0 14

10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0 12

10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0 10

10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  8

10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  6

10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  4

10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  2

10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  10.0  0

12    13    14    16    18    20    22    24    26    28    30

FREQUENCY IN MEGAHERTZ

ANTENNA EFFICIENCY

.0    .0    .0    .0    .0    .0    .0    .0    .0    .0    .0

12    13    14    16    18    20    22    24    26    28    30

FREQUENCY IN MEGAHERTZ

Figure 50a.  Constant gain pattern continued.

METHOD 13   ICEPAC IC.10   PAGE  23

ITSA 1 ANTENNA PACKAGE             ANTENNA PATTERN

FREQUENCY RANGE  ANTENNA TYPE      HEIGHT    LENGTH     ANGLE

2.0 TO  30.0   CONST. GAIN       -1.900      .000      .000

2     3     4     5     6     7     8     9    10    11

90   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1

88   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1

86   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1

84   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1

82   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1

80   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1

78   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1

76   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1

74   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1

72   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1

E 70   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1

L 68   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1

E 66   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1

V 64   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1

A 62   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1

T 60   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1

I 58   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1

O 56   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1

N 54   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1

52   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1

A 50   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1

N 48   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1

G 46   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1

L 44   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1

E 42   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1

40   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1

I 38   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1

N 36   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1

34   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1

D 32   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1

E 30   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1

G 28   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1

R 26   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1

E 24   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1

E 22   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1

S 20   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1

18   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1

16   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1

14   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1

12   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1

10   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1

8   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1

6   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1

4   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1

2   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1

0   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1

2     3     4     5     6     7     8     9    10    11

FREQUENCY IN MEGAHERTZ

ANTENNA EFFICIENCY

-1.9  -1.9  -1.9  -1.9  -1.9  -1.9  -1.9  -1.9  -1.9  -1.9

2     3     4     5     6     7     8     9    10    11

FREQUENCY IN MEGAHERTZ

Figure 51.  Constant gain receiver antenna with

antenna efficiency.

METHOD 13   ICEPAC IC.10   PAGE  24

AZIMUTH     EX(1)     EX(2)     EX(3)     EX(4)  CONDUCT.  DIELECT.

.000     .000      .000      .000      .000     .001     4.000

12    13    14    16    18    20    22    24    26    28    30

8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1 90

8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1 88

8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1 86

8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1 84

8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1 82

8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1 80

8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1 78

8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1 76

8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1 74

8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1 72

8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1 70

8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1 68

8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1 66

8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1 64

8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1 62

8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1 60

8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1 58

8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1 56

8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1 54

8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1 52

8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1 50

8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1 48

8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1 46

8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1 44

8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1 42

8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1 40

8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1 38

8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1 36

8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1 34

8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1 32

8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1 30

8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1 28

8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1 26

8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1 24

8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1 22

8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1 20

8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1 18

8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1 16

8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1 14

8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1 12

8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1 10

8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1  8

8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1  6

8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1  4

8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1  2

8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1   8.1  0

12    13    14    16    18    20    22    24    26    28    30

FREQUENCY IN MEGAHERTZ

ANTENNA EFFICIENCY

-1.9  -1.9  -1.9  -1.9  -1.9  -1.9  -1.9  -1.9  -1.9  -1.9  -1.9

12    13    14    16    18    20    22    24    26    28    30

FREQUENCY IN MEGAHERTZ

Figure 51a.  Constant gain receiver antenna with

antenna efficiency continued.

#### TABLE 1.   Files on Distribution Disks

| File # | File Name | Type | Description |
| --- | --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. 8. 9. 10. 11. 12. 13. 14. 15. 16. | ICEPAC.EXE IONINP.DAT BCOEF01.DAT BCPEF02.DAT BCOEF03.DAT BCOEF04.DAT BCOEFO5.DAT BCOEFO6.DAT BCOEFO7.DAT BCOEFO8.DAT BCOEFO9.DAT BCOEF10.DAT BCOEF11.DAT BCOEF12.DAT CGLALOB.DAT IONPC.LST | Binary ASCII Binary Binary Binary Binary Binary Binary Binary Binary Binary Binary Binary Binary Binary ASCII | ICEPAC program execution module Sample input data file Long term input data for January Long term input data for February Long term input data for March Long term input data for April Long term inptp data for May Long term input data for June Long term input data for July Long term input data for August Long term input data for September Long term input data for October Long term input data for November Long term input data for December Corrected geomagnetic file Output file formatted for output to printer |

TABLE 2.    Valid Name Identifier

| IDENTIFIER | DESCRIPTION OF INPUT PARAMETERS |
| --- | --- |
| METHOD MONTH SUNSPOT CIRCUIT SYSTEM TIME ANTENNA FREQUENCY LABEL INTEGRATE EXECUTE SAMPLE EFVAR EDP ANTOUT OUTGRAPH COMMENT QUIT FPROB TOPLINES BOTLINES | Program run option and beginning page number Year and month 12 month smoothed mean sunspot number and effective Q index Transmitter-receiver locations Power, noise, min. angle, req. reliablility, SNR, time delay and power increment for multipath Time of day loop (and indicator for LMT or UT) Transmitter or receiver, antenna type and parameters Operating frequencies Alphanumeric label for identification .GE. 0 will do a fast integration when no F1 is present Execute program with parameters currently set Optional geophysical samples (for a specified area) Optional E, F1 and F2 parameters (for a specified area) True heights and electron density (for a specified area) Write antenna patterns on a file Request output of several methods Comment line in user defined input Termination of program execution Critical frequency multipliers User specified heading lines (for method 23) User specified output lines (for method 23) |

TABLE 3.    Available Output Methods

| METHOD | DESCRIPTION OF METHOD |
| --- | --- |
| 1. 2. 3. 4. 5. 6. 7. 8. 9. 10. 11. 12. 13. 14. 15. 16. 17. 18. 19. 20. 21. 22. 23. 24. 25. 26. 27. 28. 29. | Ionospheric parameters Ionograms MUF - FOT lines (nomogram) MUF - FOT graph HPF - MUF - FOT graph MUF - FOT - Es graph FOT - MUF table (full ionosphere) MUF - FOT graph HPF - MUF - FOT graph MUF - FOT - ANG graph MUF - FOT - Es graph MUF by magnetic indices, K (not implemented) Transmitter antenna pattern Receiver antenna pattern Both transmitter and receiver antenna patterns System performance (S.P.) Condensed system performance, reliability Condensed system performance, service probability Propagation path geometry Complete system performance (C.S.P.) Forced long path model (C.S.P.) Forced short path model (C.S.P.) User selected output lines (set by TOPLINES and BOTLINES) MUF-REL table All modes table MUF - LUF - FOT table (nomogram) FOT - LUF graph ##### MUF - FOT - LUF graph MUF - LUF graph |

Table 4.  Required SNR's for Radiotelephone Service

| Radio Telephone Description | ##### GRADE OF SERVICE* |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Operator-to-Operator** |  |  | Good Commercial Quality |  |  |
|  | No Diversity | Dual Diversity | No Diversity | Dual Diversity |
| 6A3 Double Sideband - AM 3A3 Single Sideband - AM 3A3a (reduced carrier) 3A3j (suppressed carrier) 6A3 Independent Sideband - AM 6A3B (2-voice channels) 9A3B (3-voice channels) 12A3b (4-voice channels) | 51 49 48 50 50 51 | 48 46 45 47 47 48 | 75 73 72 74 74 75 | 70 68 67 69 69 70 |

* Required signal-to-Noise Ratio in occupied bandwidth relative to noise in a 1  Hz bandwidth (dB).

** For 90% intelligibility of related words.

TABLE 5.  REQUIRED SNR'S FOR RADIOTELETYPE SERVICE

| Type of Transmission | GRADE OF SERVICE* Character Error Rate** |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10-2 |  |  | 10-3 |  |  | 10-4 |  |  |
|  | No Diversity | Dual Diversity | No Diversity | Dual Diversity | No Diversity | Dual Diversity |
| 1.1F1 FSK, 60 WPM, 1500-Hz filter Start-Stop Synchronous ###### 3A7J SSB, Supressed carrier 16- teletype subchannels each subchannel ±42.5 Hz FSK, 110-Hz filter, 100 WPM, 5 unit Start-Stop Synchronous ###### 6A9b ISB, 1-voice channel and 16-teletype subchannels each subchannel ±42.5 Hz, FSK, 110-Hz filter, 100 WPM Start-Stop Synchronous ###### 12A9b ISB, 2-voice channels and 32-teletype subchannels, each subchannel ±42.5 Hz, FSK, 110-filter, 100 WPM Start-Stop Synchronous | 55 50 63 58 64 59 66 61 | 51 47 59 54 60 55 62 57 | 62 59 70 66 71 67 73 69 | 58 54 65 62 66 63 68 65 | 68 65 76 73 77 74 79 76 | 63 60 70 68 71 69 73 71 |

* Required Signal-to-Noise Ratio in occupied bandwidth relative to noise in a 1 HZ bandwidth (db)

** 5-unit code, no error control schemes

Power assumed equally divided between channels

TABLE 6.  Typical Values of Ground Electrical Characteristics

| GROUND TYPE | CONDUCTIVITY | DIELECTRIC CONSTANT |
| --- | --- | --- |
| Sea Water Good Ground Poor Ground and Sea Ice Polar Ice Cap Fresh Water | 5.0 mhos/m 0.01 0.001 0.0001 0.002 | 80 10 4 1 80 |

TABLE 7.  OUTPUT COMBINATIONS FOR THE OUTGRAPH COMMAND

| METHOD | OUTGRAPH COMMAND |
| --- | --- |
| PROGRAM TASK OPTION* | ADDITIONAL OUTPUT AVAILABLE FOR THE SPECIFIED TASK OPTION (SPECIFIED ON OUTGRAPH COMMAND) |
| 3 4 5 6 | 4 5 6 3 5 6 3 4 6 3 4 5 |
| 7 8 9 10 11 | 8 to 11 9 to 11 8, 10, 11 8, 9, 11 8 to 10 |
| 16 to 25 | 8 to 11, 26 to 29** |
| 26 27 28 29 | 8 to 11, 27 to 29 8 to 11, 26, 28, 29 8 to 11, 26 to 29 8 to 11, 26 to 28 |
| 30 |  |

*	Optimal program task options for additional output.

**	LUF values set by first frequency in the frequency complement which has a computed reliability that is greater than or equal to the required system reliability.  If none of the computed reliabilities are at least as large as that required, the frequency with the greatest reliability is chosen and a designator is printed to indicate the reliability.  (Note that this occurs only when the user specifies 26 to 29 on the OUTGRAPH line and 16 to 25 on the METHOD command.  The actual computed values for LUF are printed when the user specifies 26 to 29 on the outgraph line and 26 to 29 on the METHOD command.)

TABLE 8.  Header Line Options for the TOPLINES Command

LINTP                        DESCRIPTION OF HEADER LINE

1			Month, year, and sunspot number

2			Alphanumeric information on the label command

3			Transmitter and receiver information consisting of

coordinates, azimuth, and great circle distance

(in kilometers and nautical miles)

4			Minimum take-off angle (in degrees)

5			Transmitter antenna information consisting of the

operating frequency range, antenna type (in mnemonics),

height*, length*, off-azimuth angle, conductivity,

dielectric constant as well as other antenna

characteristics

6			Receiver antenna information consisting of the

operating frequency range, antenna type (in mnemonics),

height*, length*, off-azimuth angle, conductivity,

dielectric constant as well as other antenna

characteristics

7			Power, 3 MHz man-made noise, required reliability and

required SNR

8			Multipath power tolerance and delay time tolerance

*Height and length given in meters if positive and wavelength if negative

Table 9.  System Performance Output Line Options for the

BOTLINES Command

LINEBD	OUTPUT MNEMONIC				VARIABLE DESCRIPTION

1			MODE*		Number of hops for MUF and mode type

MODE**		Mode type at transmitter end and at

receiver end

2			ANGLE*		Radiation angle at transmitter, degrees

ANGLE**		Radiation angle at transmitter and

receiver end, degrees

3			DELAY		Time delay most reliable mode (MRM),

milliseconds

4			V HITE		Virtual height MRM, kilometers

5			F DAYS		Prob. operating frequency exceeds the

predicted MUF

6			LOSS			Median system loss, MRM, dB

7			DBU			Field strength, median, dBu

8			SDBW			Signal power, median, dBw

9			NDBW			Noise strength, median, dBw

10			SNR			Median SNR, dB

11			RPWRG		Required power gain for MRM, db

12			REL			Reliability

13			MPROB		Multipath probability (short paths only)

14			SPRB			Service probability

15			SIGLW		Signal (loss), lower decile, dB

16			SIGUP		Signal (loss), upper decile, dB

17			VHFDBU		VHF field strength, median, dBu

18			VHFLW		VHF field strength, lower decile, dB

19			VHFUP		VHF field strength, upper decile, dB

20			VHFMOD		VHF mode type

21			SNRLW		SNR, lower decile, dB

22			SNRUP		SNR, upper decile, dB

*  if short path

** if long path

TABLE 10.  SYSTEM PERFORMANCE OUTPUT LINE OPTIONS AS

PRESET BY THE METHOD NUMBER

| METHOD | LINES | DESCRIPTION |
| --- | --- | --- |
| 16 | 1 to 13 | Detailed system performance |
| 17 | 1, 2, 5, 7, 10, 12 | Condensed S.P. with reliability |
| 18 | 1, 2, 5, 7, 10, 14 | Condensed S.P. with service prob. |
| 19 | 1, 2, 3, 4, 5 | Propagation path geometry |
| 20 | 1 to 22 (all) | Complete S.P. output |
| 21 | 1 to 16 | ### Force long path submodule |
| 22 | 1 to 16 | Force short path submodule |

TABLE 11.  INPUT DATA REQUIRED FOR OUTPUT METHODS

┌────────┬──────────────────────────────┬───────────────────────────────────────┐

│ DATA # │         DESCRIPTION          │         OUTPUT/RUN TYPES              │

├────────┼──────────────────────────────┼────────┬─────────┬──────────┬─────────┤

│        │                              │        │         │          │         │

│        │                              │        │         │          │         │

│        │                              │        │         │          │         │

│        │                              │        │         │          │         │

│        │                              │        │         │          │         │

│        │                              │        │         │          │         │

│        │                              │        │         │          │         │

│        │                              │        │         │          │         │

│        │                              │        │         │          │         │

│        │                              │        │         │          │         │

├────────┼──────────────────────────────┼────────┼─────────┼──────────┼─────────┤

│        │                              │        │         │          │         │

│ 1.  A  │     YEAR OR SSN              │ R      │         │ R        │ R       │

│     B  │     MONTH                    │ R      │         │ R        │ R       │

│     C  │     TRANSMITTER SITE, ,    │        │ R       │ R        │         │

│     D  │     RECEIVER SITE, ,       │        │ R       │          │         │

├────────┼──────────────────────────────┼────────┼─────────┼──────────┼─────────┤

│        │                              │        │         │          │         │

│ 2.  A  │     METHOD                   │ R      │R        │ R        │ R       │

│     B  │     BEGINNING HOUR           │ R      │         │          │         │

│     C  │     END HOUR                 │ R      │         │ R        │R        │

│     D  │     HOUR INCREMENT           │ R      │         │ R        │ R       │

│     E  │     ANTENNA PATTERN	         │        │         │          │ R       │

│     F  │     GREAT CIRCLE PATH        │ R      │         │ R        │ R       │

│     G  │     HEADER LINES             │        │         │          │ ?       │

│     H  │     SYS. PER. LINES          │        │         │          │ ?       │

├────────┼──────────────────────────────┼────────┼─────────┼──────────┼─────────┤

│        │                              │        │         │          │         │

│ 3.     │     FREQUENCY COMP.          │        │         │          │ R       │

├────────┼──────────────────────────────┼────────┼─────────┼──────────┼─────────┤

│        │                              │        │         │          │         │

│ 4.  A  │     TRANSMITTER SITE         │ R      │ *       │ R        │ R       │

│     B  │     RECEIVER SITE            │ R      │**       │ R        │ R       │

│     C  │     MIN. TAKE-OFF ANGLE      │        │         │ R        │ R       │

│     D  │     POWER                    │        │         │          │ R       │

│     E  │     REQUIRED SNR             │        │         │          │ R       │

│     F  │     NOISE                    │        │         │          │ R       │

│     G  │     REQUIRED RELIABILITY     │        │         │          │ ?       │

│     H  │     MULTIPATH                │        │         │          │ ?       │

│     I  │     MULTIPATH POWER          │        │         │          │ ?       │

│     J  │     MULTIPATH TIME           │        │         │          │ ?       │

├────────┼──────────────────────────────┼────────┼─────────┼──────────┼─────────┤

│ 5.     │      ANTENNA TYPES           │        │ R       │          │ R       │

│        │                              │        │         │          │         │

│ 6.     │     TRANSMITTING ANTENNAS    │        │ R       │          │ R       │

│        │                              │        │         │          │         │

│ 7.     │     RECEIVING ANTENNAS       │        │ R       │          │ R       │

└────────┴──────────────────────────────┴────────┴─────────┴──────────┴─────────┘

where  R  denotes required

?  required only if that output is requested

*  	Transmitter site is an optional requirement for Methods 13 and 15 if the 	        transmitter antenna orientation is not off-azimuth from path.

**  	Receiver site is an optional requirement for Methods 14 and 15 if the                 receiver antenna orientation is not off-azimuth from path.

TABLE 12.  CALCULATION OF AN EFFECTIVE Kp VALUE

Qe						Kpeff

Qe   0						0.0

Qe  <  2.0					0.0

2.0  Qe <  3.0				1.5-(2.0*(Qe-3.0)**2)/(Qe=3.0)

3.0  Qe <  6.0					Qe - 1.5

6.0  Qe <  8.0				   1.7 + 2/3 Qe

8.0  Qe						7.0

Fig. 52  Effective Kp Index as a function of effective Q index.

1. METHOD Command Line (3.3.1)

| Columns | Name | Format | Description of Input Data |
| --- | --- | --- | --- |
| 1-10 |  | A10 | "METHOD" command |
| 11-15 | Method | 15 | Program task option, 1 to 30 |

2. MONTH Command Line (3.3.1)

| Columns | Name | Format | Description of Input Data |
| --- | --- | --- | --- |
| 1-10 |  | A10 | "MONTH" command |
| 11-15 | NYEAR | I5 | Year |
| 16-20 | MONTH | I5 | Month where January is represented by 1, February by 2, etc. |

3. SUNSPOT Command Line (3.3.2)

| Columns | Name | Format | Description of Input Data |
| --- | --- | --- | --- |
| 1-10 |  | A10 | "SUNSPOT" command |
| 11-15 | SSN | F5.1 | 12 month smooth mean sunspot # |
| 16-20 | EFFQ | F5.1 | Effective Q value |

4. CIRCUIT Command Line (3.3.3)

| Columns | Name | Format | Description of Input Data |
| --- | --- | --- | --- |
| 1-10 |  | A10 | "CIRCUIT" command |
| 11-15 | TLATD | F5.2 | Latitude of transmitter,degrees |
| 16 | ITLAT | A1 | N = north, S = south |
| 21-25 | TLONGD | F5.2 | Longitude of transmitter, degrees |
| 26 | ITLONG | A1 | E = east, W = west |
| 31-35 | RLATD | F5.2 | Latitude of receiver, degrees |
| 36 | IRLAT | A1 | N = north, S = south |
| 41-45 | RLONGD | F5.2 | Longitude of receiver, degrees |
| 46 | IRLONG | A1 | E = east, W = west |
| 51-55 | NPSL | I5 | 0 = shortpath, 1 = longpath |

5. SYSTEM Command Line (3.3.3)

| Columns | Name | Format | Description of Input Data |
| --- | --- | --- | --- |
| 1-10 |  | A10 | "SYSTEM" command |
| 11-15 | PWR | F5.2 | Transmitter power, kilowatts |
| 16-20 | XNOISE | F5.0 | Man-made noise in dB below a watt (-dBW);-1 is industrial (125), -2 is residential (136), -3 is rural (148), -4 is remote (164)* |
| 21-25 | AMIND | F5.2 | Minimum radiation angle, degrees |
| 26-30 | XLUFP | F5.0 | Required reliability, percent |
| 31-35 | RSN | F5.2 | Required SNR,dB |
| 36-40 | PMP | F5.2 | Multipath power tolerance, dB |
| 41-45 | DMPX | F5.2 | Multipath time delay increment, milliseconds |
|  |  |  | *The user may indicate other man-made noise values by specifying a positive value in this field. |

6. TIME Command Line (3.3.2)

| Columns | Name | Format | Description of Input Data |
| --- | --- | --- | --- |
| 1-10 |  | A10 | "TIME" command |
| 11-15 | IHRO | I5 | Start time, hours |
| 16-20 | IHRE | I5 | Stop time, hours |
| 21-25 | IHRS | I5 | Hour increment, hours |
| 26-30 | ITIM | I5 | If negative LMT at transmitter, otherwise UT |

- FREQUENCY Command Line (3.3.3)

| Columns | Name | Format | Description of Input Data |
| --- | --- | --- | --- |
| 1-10 |  | A10 | "FREQUENCY" command |
| 11-15 | FREL(1) | F5.2 | Array of up to 11 frequencies, MHz, |
| 16-20 | FREL(2) | F5.2 | if FREL(1) is zero, then the FOT replaces FREL(1) |
| 21-25 | FREL(3) | F5.2 |  |
| 26-30 | FREL(4) | F5.2 |  |
| 31-35 | FREL(5) | F5.2 |  |
| 36-40 | FREL(6) | F5.2 |  |
| 41-45 | FREL(7) | F5.2 |  |
| 46-50 | FREL(8) | F5.2 |  |
| 51-55 | FREL(9) | F5.2 |  |
| 56-60 | FREL(10) | F5.2 |  |
| 61-65 | FREL(11) | F5.2 |  |

- LABEL Command Line (3.3.3)

| Columns | Name | Format | Description of Input Data |
| --- | --- | --- | --- |
| 1-10 |  | A10 | "LABEL" command |
| 11-20 | ITRAN(1) | A10 | For output identification only |
| 21-30 | ITRAN(2) | A10 | For output identification only |
| 31-40 | IRCVR(1) | A10 | For output identification only |
| 41-50 | IRCVR(2) | A10 | For output identification only |

- INTEGRATE Command Line (3.3.4)

| Columns | Name | Format | Description of Input Data |
| --- | --- | --- | --- |
| 1-10 |  | A10 | "INTEGRATE" command |
| 11-15 | INTEG | I5 |  0 indicates fast integration; < 0 indicates Gaussian integration |
|  | -or- |  |  |
| 11-20 | ITEMP | A10 | "OFF" indicates return to fast integration |

- EXECUTE Command Line (3.3.1)

| Columns | 9 Name | Format | Description of Input Data |
| --- | --- | --- | --- |
| 1-10 |  | A10 | "EXECUTE" command |
| 11-15 | KRUN | I5 | 3 = no indices calculation; 2 = Es indices only; 1 = E, F1,F2 indices only; 0 = E, F1, F2, Es indices. (default is KRUN = 0 which indicates indices are calculated for each ionospheric layer) |

- EFVAR Command Line (3.3.4)

| Columns | Name | Format | Description of Input Data |
| --- | --- | --- | --- |
| 1-10 |  | A10 | "EFVAR" command |
| 11-15 | I | I5 | Sample area 1 to 5 |
| 16-20 | FI(1,I) | F5.2 | foE, MHz |
| 21-25 | YI(1,I) | F5.1 | ymE, km |
| 26-30 | HI(1,I) | F5.1 | hmE, km |
| 31-35 | FI(2,I) | F5.2 | foF1, MHz |
| 36-40 | YI(2,I) | F5.1 | ymF1, km |
| 41-45 | HI(2,I) | F5.1 | hmF1, km |
| 46-50 | FI(3,I) | F5.2 | foF2, MHz |
| 51-55 | YI(3,I) | F5.1 | ymF2, km |
| 56-60 | HI(3,I) | F5.1 | hmF2, km |

12. ESVAR Command Line (3.3.4)

| Columns | Name | Format | Description of Input Data |
| --- | --- | --- | --- |
| 1-10 |  | A10 | "ESVAR" command |
| 11-15 | I | I5 | Sample area 1 to 5 |
| 16-20 | FS(1,I) | F5.2 | foEs, lower decile, MHz = lowest value, e.g., 2 |
| 21-25 | FS(2,I) | F5.2 | foEs median, e.g., 4., MHz |
| 26-30 | FS(3,I) | F5.1 | foEs upper decile, MHz = highest value, e.g., 6. |
| 31-35 | HS(I) | F5.1 | Virtual height of reflection, km, e.g., 110 |

13. EDP Command Line (3.3.4)

| Columns | Name | Format | Description of Input Data |
| --- | --- | --- | --- |
| 1-10 |  | A10 | "EDP" command |
| 11-15 | JSAMP | I5 | Must = 1 (limitation on current implementation) |
| 16-25 | ITEMP | A10 | If "OFF" return to use calculated EDP |
|  |  |  | If not "OFF" followed by 8 lines; first four contain heights, second four contain square of plasma frequency |

13a. Data for external electron density profile (3.3.4)

| Columns | Name | Format | Description of Input Data |
| --- | --- | --- | --- |
| 1-5 | HTR(1) | F5.2 | True heights, kilometers |
| 6-10 | HTR(2) | F5.2 |  |
|  | . |  |  |
|  | . |  |  |
|  | . |  | Four lines containing 50 values |

13b. Data for external electron density profile (3.3.4)

| Columns | Name | Format | Description of Input Data |
| --- | --- | --- | --- |
| 1-5 | FNSQ(1) | F5.2 | Plasma frequency squared (MHz)2 |
| 6-10 | FNSQ(2) | F5.2 |  |
|  | . |  |  |
|  | . |  |  |
|  | . |  | Four lines containing 50 values |

14. ANTOUT Command Line (3.3.5)

| Columns | Name | Format | Description of Input Data |
| --- | --- | --- | --- |
| 1-10 |  | A10 | "ANTOUT" command |
| 11-13 | ITEMP | A3 | If = "OFF" do not output any antenna patterns, otherwise pattern will be output to file |
| 15-24 | AOUTPT | A20 | File name for antenna output |

15. COMMENT Command Line (3.3.5)

| Columns | Name | Format | Description of Input Data |
| --- | --- | --- | --- |
| 1-10 |  | A10 | "COMMENT" command |
| 11-80 |  |  | User inserts description of the input file or other remarks and comments; not internally read by program |

16. QUIT Command Line (3.3.1)

| Columns | Name | Format | Description of Input Data |
| --- | --- | --- | --- |
| 1-10 |  | A10 | "QUIT" command |

17. OUTGRAPH Command Line (3.3.5)

| Columns | Name | Format | Description of Input Data |
| --- | --- | --- | --- |
| 1-10 |  | A10 | "OUTGRAPH" command |
| 11-15 | KTOUT(1) | I5 | Indicates additional "methods" of desired graphical output (specify 1 to 12 methods) |
| 16-20 | KTOUT(2) | I5 |  |
| 21-25 | KTOUT(3) | I5 |  |
| 26-30 | KTOUT(4) | I5 | (See Table 7 for valid output combinations) |
| 31-35 | KTOUT(5) | I5 |  |
| 36-40 | KTOUT(6) | I5 |  |
| 41-45 | KTOUT(7) | I5 |  |
| 46-50 | KTOUT(8) | I5 |  |
| 51-55 | KTOUT(9) | I5 |  |
| 56-60 | KTOUT(10) | I5 |  |
| 61-65 | KTOUT(11) | I5 |  |
| 66-70 | KTOUT(12) | I5 |  |
|  | -OR- |  |  |
| 11-20 | ITEMP | A10 | If = "OFF", stop additional output |

18. FPROB Command Line (3.3.4)

| Columns | Name | Format | Description of Input Data |
| --- | --- | --- | --- |
| 1-10 |  | A10 | "FPROB" command |
| 11-15 | PSC(1) | F5.1 | Multiplier for foE > 0 |
| 16-20 | PSC(2) | F5.1 | Multiplier for foF1 |
| 21-25 | PSC(3) | F5.S | Multiplier for foF2 > 0 |
| 26-30 | PSC(4) | F5.1 | Multiplier for foEs |
|  | -OR- |  |  |
| 11-20 | ITEMP | A10 | "OFF" returns to default values of 1.0, 1.0, 1.0, 0.7. The values for foE and foF2 must be positive. |

19. TOPLINES Command Line (3.3.5)

| Columns | Name | Format | Description of Input Data |
| --- | --- | --- | --- |
| 1-10 |  | A10 | "TOPLINES" command |
| 11-15 | LINTP(1) | I5 | Header line number; 1 to 8 (refer to table 8 for description) |
| 16-20 | LINTP(2) | I5 | Program will print those header lines specified when method 23 is used |
| 21-25 | LINTP(3) | I5 |  |
| 26-30 | LINTP(4) | I5 |  |
| 31-35 | LINTP(5) | I5 |  |
| 36-40 | LINTP(6) | I5 |  |
| 41-45 | LINTP(7) | I5 |  |
| 46-50 | LINTP(8) | I5 |  |
| 51-55 | LINTP(9) | I5 |  |
| 56-60 | LINTP(10) | I5 |  |
| 61-65 | LINTP(11) | I5 |  |
| 66-70 | LINTP(12) | I5 |  |
| 71-75 | LINTP(13) | I5 |  |
| 76-80 | LINTP(14) | I5 |  |
| 11-20 | ITEMP | A10 | "OFF" indicates to discontinue printing the header line(s) specified on a previous "TOPLINES" command. |

20. BOTLINES Command Line (3.3.5)

| Columns | Name | Format | Description of Input Data |
| --- | --- | --- | --- |
| 1-10 |  | A10 | "BOTLINES" command |
| 11-15 | LINBD(1) | I5 | Line number, 1 to 22 (indicate parameter to output, see Table 9) when method 23 is used |
| 16-20 | LINBD(2) | I5 |  |
| 21-25 | LINBD(3) | I5 |  |
| 26-30 | LINBD(4) | I5 |  |
| 31-35 | LINBD(5) | I5 |  |
| 36-40 | LINBD(6) | I5 |  |
| 41-45 | LINBD(7) | I5 |  |
| 46-50 | LINBD(8) | I5 |  |
| 51-55 | LINBD(9) | I5 |  |
| 56-60 | LINBD(10) | I5 |  |
| 61-65 | LINBD(11) | I5 |  |
| 66-70 | LINBD(12) | I5 |  |
| 71-75 | LINBD(13) | I5 |  |
| 76-80 | LINBD(14) | I5 |  |
| 11-20 | ITEMP | A10 | "OFF" indicates to discontinue printing the parameter(s) specified on a previous "BOTLINES" command |

21. ANTENNA COMMAND, TERMINATED RHOMBIC

| Columns | Name | Format | Description of Input Data |
| --- | --- | --- | --- |
| 1-10 |  | A10 | "ANTENNA" 1 |
| 11-15 | IAT | I5 | 1 for transmitter, 2 for receiver |
| 16-20 | IANTR | I5 | Antenna type = 1 |
| 21-25 | AETA | F5.1 | Bearing of antenna, degrees, E of N |
| 26-30 | ASIG | F5.1 | Ground conductivity, mhos/m |
| 31-35 | AEPS | F5.1 | Relative dielectric constant |
| 36-40 | AND | F5.1 | Tilt angle, degrees (1/2 the large interior angle) |
| 41-45 | ANL | F5.1 | Antenna leg length, meters (wavelengths if negative) |
| 46-50 | ANH | F5.1 | Antenna height, meters (wavelengths if negative) |
| 51-55 |  |  | Not used for this antenna |
| 56-60 |  |  | Not used for this antenna |
| 61-65 |  |  | Not used for this antenna |
| 66-70 |  |  | Not used for this antenna |
| 71-75 | AFQB | F5.1 | Ending frequency when more than one antenna is used |
| 76-80 | IAIN | I5 | Antenna number indicator (each transmitter or receiver antenna can be defined by up to three different antennas over the frequency range; default is one, if IAIN is left blank. |

22. ANTENNA C0MMAND, VERTICAL MONOPOLE

| Columns | Name | Format | Description of Input Data |
| --- | --- | --- | --- |
| 1-10 |  | A10 | "ANTENNA" 2 |
| 11-15 | IAT | I5 | 1 for transmitter, 2 for receiver |
| 16-20 | IANTR | I5 | Antenna type = 2 |
| 21-25 |  |  | Not used for this antenna |
| 26-30 | ASIG | F5.1 | Ground conductivity, mhos/m |
| 31-35 | AEPS | F5.1 | Relative dielectric constant |
| 36-40 |  |  | Not used for this antenna |
| 41-45 | ANL | F5.1 | Antenna height, meters (wavelengths if negative) |
| 46-50 | ANH | F5.1 | Gain above a dipole, dB |
| 51-55 |  |  | Not used for this antenna |
| 56-60 |  |  | Not used for this antenna |
| 61-65 |  |  | Not used for this antenna |
| 66-70 |  |  | Not used for this antenna |
| 71-75 | AFQB | F5.1 | Ending frequency when more than one antenna is used |
| 76-80 | IAIN | I5 | Antenna number indicator (each transmitter or receiver antenna can be defined by up to three different antennas over the frequency range; default is one, if IAIN is left blank. |

23. ANTENNA COMMAND, HORIZONTAL DIPOLE

| Columns | Name | Format | Description of Input Data |
| --- | --- | --- | --- |
| 1-10 |  | A10 | "ANTENNA" 3 |
| 11-15 | IAT | I5 | 1 for transmitter, 2 for receiver |
| 16-20 | IANTR | I5 | Antenna type = 3 |
| 21-25 | AETA | F5.1 | Bearing, degrees E of N |
| 26-30 | ASIG | F5.1 | Ground conductivity, mhos/m |
| 31-35 | AEPS | F5.1 | Relative dielectric constant |
| 36-40 |  |  | Not used for this antenna |
| 41-45 | ANL | F5.1 | Antenna length, meters (wavelengths if negative) |
| 46-50 | ANH | F5.1 | Antenna height, meters (wavelengths if negative) |
| 51-55 | AEX(1) | F5.1 | Gain above 1/2 wavelength horizontal dipole, dB |
| 56-60 |  |  | Not used for this antenna |
| 61-65 |  |  | Not used for this antenna |
| 66-70 |  |  | Not used for this antenna |
| 71-75 | AFQB | F5.1 | Ending frequency when more than one antenna is used |
| 76-80 | IAIN | I5 | Antenna number indicator (each transmitter or receiver antenna can be defined by up to three different antennas over the frequency range; default is one, if IAIN is left blank. |

24. ANTENNA COMMAND, HORIZONTAL YAGI

| Columns | Name | Format | Description of Input Data |
| --- | --- | --- | --- |
| 1-10 |  | A10 | "ANTENNA" 4 |
| 11-15 | IAT | I5 | 1 for transmitter, 2 for receiver |
| 16-20 | IANTR | I5 | Antenna type = 4 |
| 21-25 | AETA | F5.1 | Bearing of antenna, degrees E of N |
| 26-30 | ASIG | F5.1 | Ground conductivity, mhos/m |
| 31-35 | AEPS | F5.1 | Relative dielectric constant |
| 36-40 |  |  | Not used for this antenna |
| 41-45 | ANL | F5.1 | Antenna length, meters (wavelengths if negative) |
| 46-50 | ANH | F5.1 | Antenna height, meters (wavelengths if negative) |
| 51-55 | AEX(1) | F5.1 | Gain above 1/2 wavelength horizontal dipole, dB |
| 56-60 |  |  | Not used for this antenna |
| 61-65 |  |  | Not used for this antenna |
| 66-70 |  |  | Not used for this antenna |
| 71-75 | AFQB | F5.1 | Ending frequency when more than one antenna is used |
| 76-80 | IAIN | I5 | Antenna number indicator (each transmitter or receiver antenna can be defined by up to three different antennas over the frequency range; default is one, if IAIN is left blank. |

25. ANTENNA COMMAND, VERTICAL LOG PERIODIC ARRAY OF MONOPOLES

| Columns | Name | Format | Description of Input Data |
| --- | --- | --- | --- |
| 1-10 |  | A10 | "ANTENNA" 5 |
| 11-15 | IAT | I5 | 1 for transmitter, 2 for receiver |
| 16-20 | IANTR | I5 | Antenna type = 5 |
| 21-25 |  |  | Not used for this antenna |
| 26-30 | ASIG | F5.1 | Ground conductivity, mhos/m |
| 31-35 | AEPS | F5.1 | Relative dielectric constant |
| 36-40 |  |  | Not used for this antenna |
| 41-45 | ANL | F5.1 | Antenna height, must be 1/4 wavelength (-.25) |
| 46-50 |  |  |  |
| 51-55 | AEX(1) | F5.1 | Gain above 1/4 wavelength vertical |
| 56-60 |  |  | Not used for this antenna |
| 61-65 |  |  | Not used for this antenna |
| 66-70 |  |  | Not used for this antenna |
| 71-75 | AFQB | F5.1 | Ending frequency when more than one antenna is used |
| 76-80 | IAIN | I5 | Antenna number indicator (each transmitter or receiver antenna can be defined by up to three different antennas over the frequency range; default is one, if IAIN is left blank. |

26. ANTENNA COMMAND, CURTAIN ANTENNA

| Columns | Name | Format | Description of Input Data |
| --- | --- | --- | --- |
| 1-10 |  | A10 | "ANTENNA" 6 |
| 11-15 | IAT | I5 | 1 for transmitter, 2 for receiver |
| 16-20 | IANTR | I5 | Antenna type = 6 |
| 21-25 | AETA | F5.1 | Antenna bearing degrees E of N |
| 26-30 | ASIG | F5.1 | Ground conductivity, mhos/m |
| 31-35 | AEPS | F5.1 | Relative dielectric constant |
| 36-40 | AND | F5.1 | Number of bays |
| 41-45 | ANL | F5.1 | Antenna element length, meters |
| 46-50 | ANH | F5.1 | Antenna height to first element, meters |
| 51-55 | AEX(1) | F5.1 | Number of elements per bay |
| 56-60 | AEX(2) | F5.2 | Distance between elements centers, meters |
| 61-65 | AEX(3) | F5.1 | Vertical spacing of elements, meters |
| 66-70 | AEX(4) | F5.1 | Distance from screen, meters |
| 71-75 | AFQB | F5.1 | Ending frequency when more than one antenna is used |
| 76-80 | IAIN | I5 | Antenna number indicator (each transmitter or receiver antenna can be defined by up to three different antennas over the frequency range; default is one, if IAIN is left blank. |

27. ANTENNA COMMAND, TERMINATED SLOPING VEE

| Columns | Name | Format | Description of Input Data |
| --- | --- | --- | --- |
| 1-10 |  | A10 | "ANTENNA" 7 |
| 11-15 | IAT | I5 | 1 for transmitter, 2 for receiver |
| 16-20 | IANTR | I5 | Antenna type = 7 |
| 21-25 | AETA | F5.1 | Bearing of antenna degrees E of N |
| 26-30 | ASIG | F5.1 | Ground conductivity, mhos/m |
| 31-35 | AEPS | F5.1 | Relative dielectric constant |
| 36-40 | AND | F5.1 | 1/2 apex angle in plane of wires |
| 41-45 | ANL | F5.1 | Antenna leg length, meters |
| 46-50 | ANH | F5.1 | Antenna height, meters |
| 51-55 | AEX(1) | F5.1 | Terminated height |
| 56-60 |  |  | Not used for this antenna |
| 61-65 |  |  | Not used for this antenna |
| 66-70 |  |  | Not used for this antenna |
| 71-75 | AFQB | F5.1 | Ending frequency when more than one antenna is used |
| 76-80 | IAIN | I5 | Antenna number indicator (each transmitter or receiver antenna can be defined by up to three different antennas over the frequency range; default is one, if IAIN is left blank. |

28. ANTENNA COMMAND, INVERTED L

| Columns | Name | Format | Description of Input Data |
| --- | --- | --- | --- |
| 1-10 |  | A10 | "ANTENNA" 8 |
| 11-15 | IAT | I5 | 1 for transmitter, 2 for receiver |
| 16-20 | IANTR | I5 | Antenna type = 8 |
| 21-25 | AETA | F5.1 | Antenna bearing degrees E of N |
| 26-30 | ASIG | F5.1 | Ground conductivity, mhos/m |
| 31-35 | AEPS | F5.1 | Relative dielectric constant |
| 36-40 |  |  | Not used for this antenna |
| 41-45 | ANL | F5.1 | Antenna length, meters |
| 46-50 | ANH | F5.1 | Antenna height, meters |
| 51-55 |  |  | Not used for this antenna |
| 56-60 |  |  | Not used for this antenna |
| 61-65 |  |  | Not used for this antenna |
| 66-70 |  |  | Not used for this antenna |
| 71-75 | AFQB | F5.1 | Ending frequency when more than one antenna is used |
| 76-80 | IAIN | I5 | Antenna number indicator (each transmitter or receiver antenna can be defined by up to three different antennas over the frequency range; default is one, if IAIN is left blank. |

29. ANTENNA COMMAND, TERMINATED SLOPING RHOMBIC

| Columns | Name | Format | Description of Input Data |
| --- | --- | --- | --- |
| 1-10 |  | A10 | "ANTENNA" 9 |
| 11-15 | IAT | I5 | 1 for transmitter, 2 for receiver |
| 16-20 | IANTR | I5 | Antenna type = 9 |
| 21-25 | AETA | F5.1 | Antenna bearing degrees E of N |
| 26-30 | ASIG | F5.1 | Ground conductivity, mhos/m |
| 31-35 | AEPS | F5.1 | Relative dielectric constant |
| 36-40 | AND | F5.1 | 1/2 large interior angle in plane of wire |
| 41-45 | ANL | F5.1 | Leg length, meters |
| 46-50 | ANH | F5.1 | Antenna height, meters |
| 51-55 | AEX(1) | F5.1 | Terminate height, meters |
| 56-60 |  |  | Not used for this antenna |
| 61-65 |  |  | Not used for this antenna |
| 66-70 |  |  | Not used for this antenna |
| 71-75 | AFQB | F5.1 | Ending frequency when more than one antenna is used |
| 76-80 | IAIN | I5 | Antenna number indicator (each transmitter or receiver antenna can be defined by up to three different antennas over the frequency range; default is one, if IAIN is left blank. |

30. ANTENNA COMMAND, INTERLACED RHOMBIC

| Columns | Name | Format | Description of Input Data |
| --- | --- | --- | --- |
| 1-10 |  | A10 | "ANTENNA" 11 |
| 11-15 | IAT | I5 | 1 for transmitter, 2 for receiver |
| 16-20 | IANTR | I5 | Antenna type = 11 |
| 21-25 | AETA | F5.1 | Antenna bearing degrees E of N |
| 26-30 | ASIG | F5.1 | Ground conductivity, mhos/m |
| 31-35 | AEPS | F5.1 | Relative dielectric constant |
| 36-40 | AND | F5.1 | 1/2 large interior angle in plane of wire, degrees |
| 41-45 | ANL | F5.1 | Leg length, meters |
| 46-50 | ANH | F5.1 | Lower antenna height, meters |
| 51-55 | AEX(1) | F5.1 | Vertical displacement, meters |
| 56-60 | AEX(2) | F5.1 | Horizontal feed point displacement, meters |
| 61-65 |  |  | Not used for this antenna |
| 66-70 |  |  | Not used for this antenna |
| 71-75 | AFQB | F5.1 | Ending frequency when more than one antenna is used |
| 76-80 | IAIN | I5 | Antenna number indicator (each transmitter or receiver antenna can be defined by up to three different antennas over the frequency range; default is one, if IAIN is left blank. |

31. ANTENNA COMMAND, CONSTANT GAIN

| Columns | Name | Format | Description of Input Data |
| --- | --- | --- | --- |
| 1-10 |  | A10 | "ANTENNA" 12 |
| 11-15 | IAT | I5 | 1 for transmitter, 2 for receiver |
| 16-20 | IANTR | I5 | Antenna type = 12 |
| 21-25 | AETA | F5.1 | Gain above isotropic, dB |
| 26-30 | ASIG | F5.1 | Ground conductivity, mhos/m |
| 31-35 | AEPS | F5.1 | Relative dielectric constant |
| 36-40 |  |  | Not used for this antenna |
| 41-45 |  |  | Not used for this antenna |
| 46-50 | ANH | F5.1 | Antenna efficiency, dB |
| 51-55 |  |  | Not used for this antenna |
| 56-60 |  |  | Not used for this antenna |
| 61-65 |  |  | Not used for this antenna |
| 66-70 |  |  | Not used for this antenna |
| 71-75 | AFQB | F5.1 | Ending frequency when more than one antenna is used |
| 76-80 | IAIN | I5 | Antenna number indicator (each transmitter or receiver antenna can be defined by up to three different antennas over the frequency range; default is one, if IAIN is left blank. |

32. ANTENNA COMMAND, ANTENNA READ FROM FILE

| Columns | Name | Format | Description of Input Data |
| --- | --- | --- | --- |
| 1-10 |  | A10 | "ANTENNA" 18 |
| 11-15 | IAT | I5 | 1 for transmitter, 2 for receiver |
| 16-20 | IANTR | I5 | Antenna type = 18 |
| 21-40 |  |  | FILE NAME of antenna input file |
| 76-80 | IAIN | I5 | Antenna type 18 If IAIN  0, then IAIN indicates the number of antennas to skip forward from position one on the antenna file before reading the desired pattern. |

9.  REFERENCES

Aker, H. C., and A. H. Lagrone (l962), Digital computation of the mutual impedance between thin dipoles, Proc. IRE Trans. Ant. & Prop., AP-10 (2) 	172-178.

Barghausen, A. F., J. W. Finney, L. L. Proctor, and L.D. Schultz (1969), 	Predicting long-term operational parameters of high-frequency sky-wave 	telecommunication systems, ESSA Technical Report ERL 110-ITS78, Washington, D. C., May.

Beckmann, B. (1958), Concerning the relationship of the field intensity to the 	limits of the transmission frequency range, NTZ 11, 623, 528.

Bibl, K., A. Paul, and K. Rawer (1961), Absorption in the D and E regions and its 	time variation, J. Atmospheric and Terrest. Phys. 23, 244-259, December.

Bremmer, H. (l949), Terrestrial Radio Waves (Elsevier Publishing Co., New York, New York).

Budden, K. G. (l966), Radio Waves in the Ionosphere (Cambridge University Press, 	Cambridge, England).

CCIR (1964), World distribution and characteristics of atmospheric radio noise, 	Rept. 322, Documents of the Xth Plenary Assembly, Geneva, 1963.

CCIR (l966), Atlas of ionospheric characteristics, Rept. 340-1, Oslo, ITU, Geneva, Switzerland.

CCIR (1970), CCIR interim method for estimating sky-wave field strength and 	transmission loss at frequencies between the approximate limits of 2 and 30 MHz, CCIR Report 252-2, ITU, Geneva, Switzerland.

Central Radio Propagation Laboratory (1948), Ionospheric radio propagation, NBS 	Circular 462.

Croft, T. A. (l967), HF radio focusing caused by the electron distribution between ionospheric layers, J. Geophys, Res., 72, No. 9, 2343-2355.

Davies, K. (l966), Ionospheric Radio Propagation, NBS Monograph 80 (U. S. Government Printing Office, Washington, D. C.  20402).

Davies, K. (1969), Ionospheric Radio Waves (Blaisdell Publishing Co., Waltham, 	Mass).

Davis, R. M., and N. L. Groome (1965), The effect of auroral zone absorption on high frequency system loss, NBS Report 8810, Boulder, Colorado.

Fejer, J. A. (1961), The absorption of short radio waves in the ionospheric D and E regions, J. A. and T. P. 23, 260-274.

Feldstein, Y. I., and G. V. Starkov (1967),  Dynamics of Auroral Belt and Polar Geomagnetic Disturbances, Planet Space Sci. 15, 209.

Finney, J. B. (1963), Ray tracing development for propagation studies, NBS Report 7697.

George, P. L. (1971), The calculation of ionospheric absorption in HF radio propagation prediction, WRE-Technical Note-A207(AP).  Department of Supply, Weapons Research Establishment, South Australia.

Harnischmacher, E. (l960), A calculation method of ionospheric propagation conditions for very high and antipode distance, in Electromagnetic Wave Propagation, (Academic Press).

Haselgrove, J. (1954), Ray theory and a new method for ray tracing, (Report of Conference on Physics of the Ionosphere, London Physical Society).

Haydon, G. W., M. Leftin, and R. K. Rosich (1976), Predicting the Performance of High Frequency Sky-wave Telecommunication Systems, OT Report 76-102, Boulder, Colorado 80303.

Haydon, G. W., and D. L. Lucas (1968), Predicting ionosphere electron density profiles, Radio Science 3, No. 13, pp. 111-119.

Haydon, G. W., D. L. Lucas, and R. A. Hanson (1969), Technical considerations in the selection of optimum frequencies for high frequency skywave communication services, ESSA Tech. Report ERL 113-ITS 81, U. S. Dept. of Commerce, Boulder, Colorado.  This report is a reprint of NBS Report 7249, originally issued November, l962.

Headrick, J. M., and M. I. Skolnik (1974), Over-the-Horizon Radar in the HF Band, Proc. IEEE 62, No. 6, 664-673.

Headrick, J. M., J. M. Thomason, D. L. Lucas, S. R. McCammon, R. A. Hanson and J  L  Lloyd (1971), virtual path tracing for HF radar including an ionospheric model, NRL Memo Report 2226, Naval Research Laboratory, Washington, D. C.

Jasik, H. (1961), Antenna Engineering Handbook (McGraw-Hill, New York).

Jones, W. B., and R. M. Gallet (l962), The representation of diurnal and geographic variations of ionospheric data by numerical methods, Radio Sci. (J. Res. N8S), No. 4, 419-438.

Jones, W. B., and F. G. Stewart (1970), A numerical method for global mapping of plasma frequency, Radio Sci. No. 6, 773-784.

Jordan, E. C., and K. G. Balmain (1968), Electromagnetic Waves and Radiating Systems, (Prentice-Hall, Englewood Cliffs, N. J).

Kelley, L. (1946), Calculation of skywave field intensities, maximum usable frequencies, U. S. Army Signal Corps Tech. Report No. 6.

King, R. W. P., and T. T. Wu (1965), The cylindrical antenna with arbitrary driving point, IEEE Trans. on Ant. & Prop., AP-13, 710-718.

Laitinen, P. 0. (1957), Linear communication antennas, Tech. Report No. 7, revised, October, U. S. Signal Radio Propagation Agency, Fort Monmouth, N.J.

Laitinen, P. 0., and G. W. Haydon (1950), Analysis and prediction of skywave field intensities in the high frequency band, U. S. Army Signal Radio Propagation Agency Tech. Report No. 9, Rev. (RPN 203).

Leftin, M. (1976), Numerical representation of monthly median critical frequencies of the regular E region (foE), OT Report 76-88, Boulder, Colorado 80303.

Leftin, M., S. M. Ostrow, and C. Preston (1968), Numerical maps of foEs for solar cycle minimum and maximum, ESSA Technical Report ERL 73-ITS 63, Boulder, Colorado 80303.

Lejay, P., and D. Lepechinsky (l950), Field intensity at the receiver as a function of distance, Nature 165, 306.

Lucas, D. L., and J. D. Harper (1966), A numerical representation of CCIR Report 233 high frequency (3-30 Mc/s) atmospheric radio noise data, NBS Tech. Note No. 318, U. S. Department of Commerce, Boulder, Colorado 80303.

Lucas, D. L., and G. W. Haydon (1961), MUF-FOT predictions by electronic computers, NBS Report 6789.  U. S. Department of Commerce, Boulder, Colorado  80303.

Lucas, D. L., and G. W. Haydon (l962), Predicting the performance of band 7 communications systems using electronic computer, NBS Report 7619.

Lucas, D. L., and G. W. Haydon (l966), Predicting statistical performance indexes for high frequency telecommunications systems, ESSA Tech. Report IER 1-ITSA 1, U  S  Department of Commerce, Boulder, Colorado  80303.

Lucas, D. L., J. L. Lloyd, J. M. Headrick, and J. F. Thomason (l972), Computer techniques for planning and management of OTH radars, NRL Memo Report 2500, Naval Research Laboratory, Washington, D. C.

Ma, M. T. (1974), Theory and Application of Antenna Arrays (John Wiley & Sons, New York).

Ma, M. T.. and L. C. Walters (1967), Computed radiation patterns of log-periodic antennas over lossy plane ground, ESSA Tech. Report IER 54-ITSA 52, U. S. Department of Commerce, Boulder, Colorado 80303.

Ma, M. T., and L. C. Walters (l969), Power gains for antennas over lossy plane ground, ESSA Tech. Report ERL 104-ITS 74. U. S. Department of Commerce, Boulder, Colorado 80303.  See also Ma (1974).

Martyn, D. F. (1959), The normal F region of the ionosphere, Proc. IRE 47, No. 2, 147-155.

Pedersen, P. 0. (l927), The Propagation of Radio Waves, Danmarks Naturvidenskabelige Samfund, Copenhagen.

Philips, M. L. (l962), Evaluation of effective Es reflectivity of obscuration because of Es ionization, External Tech. Memo No. 14, Electro-Physics Labs.

Piggott, W. R. (1953), The reflection and absorption of radio waves in the ionosphere, Proc. IEEE 100, Part III, No. 64, pp. 61-72.

Piggott, W. R., and K. Rawer (1972), U.R.S.I. Handbook of ionogram interpretation and reduction, World Data Center A for Solar-Terrestrial Physics, Report UAG-23. NOAA, Boulder, Colorado 80303.

Ramo, S.,and J. R. Whinnery (l960), Fields and waves in modern radio, (John Wiley and Sons, New York).

Ratcliffe, J. A. (l951), A quick method for analyzing ionospheric records, J. Geophys. Res. 56. 463-485.

Rawer, K. (ed.) (1976), Manual on Ionospheric Absorption Measurements, Report UAG-57, NOAA, Boulder, Colorado 80303.

Rice, P. L., A. G. Longley, K. A. Norton, and A. P. Barsis (l965), Transmission Loss Predictions for Tropospheric Communication Circuits, NBS Technical Note, No. 101, U. S. Department of Commerce, Boulder, Colorado 80303.

Rosich, R. K., and W. B. Jones (1973), The numerical representation of the critical frequency of the Fl region of the ionosphere, OT Report 73-22, Boulder, Colorado 80303.

Schelkunoff, S. A., and H. F. Friis (1952), Antennas - theory and practice, (John Wiley and Sons, New York).

Schultz, L. D., and R. M. Gallet (1970), A survey and analysis of normal ionospheric absorption measurements obtained from pulse reflections, ESSA Professional Paper 4.

Shimazaki, T. (1965), Worldwide daily variations in the height of the maximum electron density of the ionospheric F2 layer, J. Radio Res. Labs., Japan, 2, No. 7, 86-97.

Spaulding, A.D. and F.G. Stewart (1987), An Updated Noise Model for use in IONCAP, NTIA Report 87-212.  U.S. Department of Commerce, Boulder, Colorado  80303

Spogen, Leo R., J. L. Lloyd, and E. P. Moore, Bell Aerosystems Company, Arizona Operations, "HF and LF Propagation Models for Interference Prediction," 	Technical Report No. RADC-TR-67-396, August 1967.

Stanford Research Institute (1957), The application of digital computing techniques to the problem of interference on HF communication circuits and networks, Special Report 1, SRI Project 2124, Menlo Park, California.

Tascione, T. F. (1979),  AFGWC Aurora Program Documentation.

Whale, H. -. (1969), Effects of ionospheric scattering in very-long distance radio communications (Plenum Press, New York, New York).

Wheeler, J. L. (1966), Transmission loss for ionospheric propagation above the standard MUF, Radio Science, 1, No. 11, 1303-1308.